3GPP TSG-SA4 Conf Call	S4-AHI966

# 30 April, 2020, Electronic Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | <Spec#> | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GMSd based Network Assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501 defines two methods for providing network assistance, ANBR based and 5GMSd based. This pCR proposes the stage 3 for 5GMSd based network assistance. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.6.7.3	5GMSd AF-based Network Assistance

This procedure is used by the 5GMSd Client to request Network Assistance from the 5GMSd AF.

The 5GMSd client first creates a network assistance session resource and provides information such as segment duration (recommendation window), usable bitrate values (as bitrate array) and the flow identification. The information is used by the Network Assistance function to derive bitrate recommendations.

In case of DASH, the 5GMSd Client determines the values for the usable bitrates based on the DASH MPD by creating reasonable representation combinations with media components from different adaptation sets. The 5GMSd Client may provide the bitrates as bitrate array to the 5GMSd AF. When present, the 5GMSd AF selects bitrates from the bitrate array for recommendations.

After the Network Assistance session resource is created and populated, the 5GMSd client uses the Network Assistance session id as identification when requesting a bitrate recommendation.

In order to terminate a network assistance session, the 5GMSd Client deletes the Network Assistance session resource.

**** Next Change ****

## 10.x	5GMSd AF based Network Assistance API

### 10.x.1	Overview

### 10.x.2	Data model

#### 10.x.2.1 	Network Assistance session resource

#### A single Network Assistance session resource is defined in Table 10.x.2.2-1.

Table 10.x.2.1-1: Definition of Network Assistance session resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| duration | Int | 1..1 | Identifies the duration, for which the network assistance request is needed. |
| bitrateArray | [int] | 0..1 | Valid application flow bitrate values, e.g. total representation bitrates derived from the DASH MPD. |
| flowDescriptions | [FlowDescription] | 0..1 | Identification of application flows, e.g. 2-Tuple (IP addresses) or 5-Tuple (IP Addresses, protocol and ports). |

#### 10.x.2.2	Bitrate Recommendation resource

Table 10.x.2.2-1: Definition of bitrate recommendation resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| recommendation | Bitrate | 1..1 | Recommended bitrate to be used for the next duration. |

### 10.x.3	Resource structure

The network assistance API is accessible through this URL paths:

{apiRoot}/3gpp-m5d-network-assistance/v1/

The following operations and the corresponding HTTP methods are supported:

| Operation | {apiSpecificResourceUriPart} | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new NA Session resource | / | POST | This is used to create a Network Assistance Session resource. |
| Fetch a NA Session resource | {session-id} | GET | Fetch the properties of a NA Session resource |
| Update a NA Session resource | {session-id} | PUT, PATCH | Update properties of a NA session Resource |
| Delete a NA Session resource | {session-id} | DELETE | Delete a NA Session resource. |
| Get a bitrate recommendation | {session-id}/recommendation | GET | Read a bitrate recommendation |

### 10.x.4	Operations

**** Next Change ****