3GPP TSG-SA4 Meeting #110-e	S4-AHI995

# Electronic, Online, 19th–28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | – | rev | – | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of Ingest Protocols API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-06-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Complete the stage 3 design for Ingest Protocols. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing definition of procedures. - Modification of resource structure in response to feedback. - Addition of a simple HTTP pull-based media ingest protocol. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 design for Ingest Protocols will be incomplete in Release 16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4, 7.5.3, 7.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

================================START OF FIRST CHANGE==============================

### 4.3.4	Ingest Protocols procedures

#### 4.3.4.1	General

The set of content ingest protocols supported by the 5GMSd AS at interface M2d and/or the set of content ingest protocols supported by the 5GMSu AS at interface M4u are described by the Ingest Protocols resource at M1d, as specified in clause 7.5.3.1.

#### 4.3.4.2	Create Ingest Protocols

The Create operation is not permitted for the Ingest Protocols resource.

#### 4.3.4.3	Read Ingest Protocols

This procedure is used by the 5GMSd Application Provider to retrieve a list of media ingest protocols supported by the 5GMSd AS and/or the 5GMSu AS. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response that includes an Ingest Protocols resource in the response message body, as specified in clause 7.5.3.1.

#### 4.3.4.4	Update Ingest Protocols

The Update operation is not permitted for the Ingest Protocols resource.

#### 4.3.4.5	Delete Ingest Protocols

The Delete operation is not permitted for the Ingest Protocols resource.

=================================END OF FIRST CHANGE===============================

===============================START OF SECOND CHANGE=============================

### 7.5.3	Data model

#### 7.5.3.1	IngestProtocols resource

The data model for the IngestProtocols resource is specified in table 7.5.3.1-1 below:

Table 7.5.3.1-1: Definition of IngestProtocols resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| downlinkIngestProtocols | array(ProtocolDescriptor) | 0..1 | An array of ProtocolDescriptor objects, as specified in clause 7.5.3.2, each one uniquely identifying a media ingest protocol supported at interface M2d by the 5GMSd AS(s) associated with the responding 5GMSu AF. |
| uplinkIngestProtocols | array(ProtocolDescriptor) | 0..1 | An array of ProtocolDescriptor objects, as specified in clause 7.5.3.2, each one uniquely identifying a media ingest protocol supported at interface M4u by the 5GMSu AS(s) associated with the responding 5GMSu AF. |

#### 7.5.3.2	ProtocolDescriptor type

The data model for the ProtocolDescriptor type is specified in table 7.5.3.2-1 below:

Table 7.5.3.2-1: Definition of ProtocolDescriptor resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| termIdentifier | URI String | 1..1 | A fully-qualified term identifier from the controlled vocabulary urn:3gpp:5gms:ingest-protocol, as specified in clause 7.5.4. |
| descriptionLocator | URL String | 0..1 | The location of a description of the ingest protocol, for example the public web URL of its specification. |

================================END OF SECOND CHANGE==============================

================================START OF THIRD CHANGE==============================

### 7.5.4	Media ingest protocols

#### 7.5.4.1	Introduction

Every media ingest protocol shall be uniquely identified by a URI string term identifier allocated within the scope of the name space urn:3gpp:5gms:ingest-protocol.

#### 7.5.4.2	HTTP pull-based media ingest protocol

If IngestConfiguration[protocol] in the Content Hosting Configuration is set to urn:3gpp:5gms:ingest-protocol:http-pull, media resources shall be ingested by the 5GMSd AS using HTTP [9]. The IngestConfiguration[pull] shall be set to True, indicating that a Pull-based protocol is used. The IngestConfiguration[entryPoint] and IngestConfiguration[path] parameters shall point at the content location on the 5GMSd Application Provider’s origin server, as specified in table 7.6.3.1-1. The base URL provided in IngestConfiguration[entryPoint] may indicate the use of HTTPS [16].

When the 5GMSd AS receives a request for a media resource at interface M4d that cannot be satisfied from its content cache, the request shall be transformed into a corresponding HTTP GET request directed to the 5GMSd Application Provider’s origin server via interface M2d, using the abovementioned entryPoint and path parameters to construct the request URL.

#### 7.5.4.3	DASH push-based media ingest protocol

If IngestConfiguration[protocol] in the Content Hosting Configuration is set to urn:3gpp:5gms:ingest-protocol:dash-if-ingest, media resources shall be ingested by the 5GMSd AS as specified by [3]. The IngestConfiguration[pull] shall be set to False, indicating that a Push-based protocol is used. The IngestConfiguration[entryPoint] parameter shall be set to the URL that will be used to upload the DASH segments and MPD to the 5GMSd AS at interface M2d. This entry point URL shall not contain a path: the path for the URL shall instead be specified by the IngestConfiguration[Path] parameter.

=================================END OF THIRD CHANGE===============================