3GPP TSG-SA4 Meeting #110-e	S4-AHI996

# Electronic, Online, 19th–28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | – | rev | – | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of Content Preparation Template procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-06-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Complete the stage 3 design for Content Preparation Templates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing definition of procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 design for Content Preparation Templates will be incomplete in Release 16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

================================START OF FIRST CHANGE==============================

### 4.3.5	Content Preparation Template procedures

#### 4.3.5.1	General

The 5GMSd AS is able to process content ingested at interface M2d before serving it at interface M4d, as specified in clause 5.2.4.4. The content processing operations are specified in a Content Preparation Template resource, as specified in clause 5.2.2.3.

#### 4.3.5.2	Create Content Preparation Template

This procedure is used by the 5GMSd Application Provider to register a new Content Preparation Template with a Provisioning Session. The 5GMSd Application Provider shall use the HTTP POST method to upload a new Content Preparation Template resource. The MIME content type of the Content Preparation Template shall be supplied in the Content-Type HTTP request header.

Upon successful creation, the 5GMSd AF shall respond with a 201 (Created) response message and the URL of the newly created resource, including its resource identifier, shall be returned as part of the HTTP Location header field.

If the MIME content type indicated in Content-Type is not understood by the 5GMSd AF, the creation of the Content Preparation Template resource shall fail with HTTP error response status code 422 (Unprocessable entity).

If the 5GMSd AF is unable to provision the resources indicated in the supplied Content Preparation Template, the creation operation shall fail with an HTTP response status code of 503 (Service Unavailable).

#### 4.3.5.3	Read Content Preparation Template

This procedure is used by the 5GMSd Application Provider to download a copy of a Content Preparation Template resource from the 5GMSd AF. The 5GMSd Application Provider shall use the GET method for this purpose.

If the procedure is successful, the 5GMSd AF shall respond with 200 (OK) and shall provide the requested resource in the HTTP message response body. The Content-Type response header shall have the same value as that supplied when the Content Preparation Template was created.

#### 4.3.5.4	Update Content Preparation Template

The update procedure is used by the 5GMSd Application Provider to modify or replace an existing Content Preparation Template resource. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) and provide the content of the resource in the response, reflecting the successful update operation.

#### 4.3.5.5	Delete Content Preparation Template

This operation is used by the 5GMSd Application Provider to destroy a Content Preparation Template resource. The HTTP DELETE method shall be used for this purpose.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message.

If the Content Preparation Template is in use as part of a Content Hosting Configuration, the procedure shall fail with HTTP error response status code 409 (Conflict).

=================================END OF FIRST CHANGE===============================