Agenda Item:	4

Source:	Tencent, Sony Corporation

Title:	FS_EMSA streaming use cases and requirements V2

Document for:	Discussion and Agreement

# Summary

This contribution proposes the following use cases for FS_EMSA PD. These use-cases were submitted to the SA4#109 meeting in S4-200749 but were not discussed. They are presented using the approved template of S4-200942 and then at the ad-hoc telco as S4-AHI991.

# Background

The SI defines its objectives as:

- “determine the processes for discovering, configuring, running, and managing media processing workflows on the 5G edge and network

- study the mapping of the new processes into the 5GMSA architecture

- how to leverage architecture and functions defined by SA2 and SA6 for edge computing and applications

- Validate the architecture extensions against the identified key use cases and recommendations from XR5G and E-FLUS (e.g. split rendering, VR stitching)

- document architectural extensions for edge media processing to support other 5GMSA features such as:

  - Online gaming, Ad-insertion, hybrid DASH/HLS services based on CMAF”

# Proposed use-cases

## User-generated live streaming

Use Case Name

User-generated live streaming (1-5)

Description

- Live streaming of personal content: This use-case is similar to Facebook Live when the user starts a live streaming session. The content is uplink streamed to an Application server in CMAF format, and then the content is transcoded into a limited number of multiple representations. All representations are encoded using a default codec. - On-the-fly transcoding: The Application Service Provider provides the default transcoding/streaming profile at the beginning of the initial session. With more joins, the Application Server provides updates on the transcoding profile. When more people with different device capabilities are joining, the Application Service Provider may transcode the content to more coding formats, to be able to deliver the content to the new audience. The number of coding formats and the bitrates depends on the audiences’ diversity and available bitrates. - High mobility: The uplink user may be on the move and therefore the available Application Server may change during the uplink streaming session. The Application Service Provider or MNO may want to move the service from one AS to another AS during the session. - High-volume live streaming: The user is a popular influencer, the Application Service Provider expects high volume streaming to many different devices. The content preparing service may occur in multiple Application Servers, based on the location profiling of the influencer and the viewers. Lower latency might be expected in this use-case. - Ad-insertion: Ads may be inserted as pre and mid-roll ads. The ads are targeted to each viewer, i.e. each viewer may see different ad content. - Secure delivery and content protection: The user might be a company that streams its all-hands meeting for its employees. A higher level of security/DRM is needed for the content.

Categorization

Type: 2D Delivery: 5GMS uplink streaming, 5GMS downlink streaming, live media streaming Device: Phone, HMD, Glasses

Preconditions

On the sending UE, a 3GPP supported encoder is installed. On every receiving UE, a 5GMS downlink streaming client is provided, possibly with server-guided client-side ad insertion capability. In the 5GMS System, one or more Application Servers are installed, running various instances of decoding/encoding or transcoding. On the network, QoS may be offered to some services including guarantees on bit rates, latency, loss rate. In the 5GMS System, content protection and content-encryption are provided as services on Application Servers. In the network, the ad server services (including ad decision, ad preparation, and ad serving) are provided as first- or third-party services for inserting the ads on the service or the client-side. In the 5GMS System, the Application Server may have a location and device profile of viewers of high demand streamers.

Requirements in terms of Capabilities and QoS/QoE Considerations

- Capabilities - Real-time transcoding and packaging to different codecs and different bit-rates - On-the-fly transcoding and packaging depending on the receiving UEs supports for codec, resolution, and bandwidth - Content caching and CDN services - KPI - Subjective/objective quality of the encoded streams - Adaptation of encoders to the requested characteristics - Changing the number of streams based on the demand - Changing the workflow distribution among multiple edge server based on the number of viewers, their location and their device characteristics - Seamless integration of ads with the main content - Efficiency in terms of - Manifest generation - Target-based advertising - Caching the content based on the number of concurrent users and for the use in the feature. - Workflow management and dynamic distribution

Feasibility and Industry Practices

Today, multi-rate on-the-fly transcoding is achieved by running a decoder and a group of software encoders in the cloud. Alternatively, third-party cloud-based transcoding services exist. The number of streams, support of multiple codecs, the amount of delay, the number of representations in the encoding ladder varies depending on the service. Codec support evolves with the availability of new codecs. Services like Facebook Live encode the content-based on-demand and on-the-fly. Services like Youtube Live provide channels for professional content providers. They can insert advertisement clips, but the ad-insertion occurs using two video elements. Services like TikTok allow the user to add a lip-sync to a song, to add his/her reactions on a small window over the video, and to make short movies using filter and speed change of the video. The user also can capture a video next to a pre-recorded video.

Nominal Cost Analysis

The cost of service increases linearly with the number of ingest streams. The cost of service increases less than linearly with the number of download streaming clients as the encoding and caching requirement will be common with the large number of viewers.

Benefits and Impact

Multi-rate encoding is not always possible on the sending device or is very battery consuming. Multi-rate encoding for various numbers of devices on one edge might not be feasible depending on the availability of resources on that edge network. A sending device may move from one edge network to another edge network due to its mobility and running the workflow in the old edge network might not be bandwidth efficient. Distributed encoding according to the spread of consumers and the capabilities of their receiving devices might be the most optimal deployment scenario. The workflow might need to be redistributed by the increase of viewers.

Potential Technical Requirements

Allocation of encoding resources on different edges and network processing elements, depending on the distributions of the clients and diversity of them.

Potential Standardization Status and Needs

- Discovery of dynamic processing capabilities of various edge networks: - Available hardware resources at the current time - Built-in (hardware-assisted and/or software optimized) encoders/transcoders - Built-in packager, manifest translator and manipulator functions - Supported encryption and content protection schemes - Available third-party function libraries - First- and third-party workflow templates - Moving media workflow from one AF/AS pair to another AF/AS pair: Move a media workflow such as multi-rate transcoding from one AF/AS pair to another AF/AS pair during the media session. - Just-in-time multi-rate transcoding between multiple AF/AS pairs: Fire up different transcoding workflows during the media sessions that are split between different AF/AS pairs.

## User-generated On-demand video streaming

Use Case Name

User-generated on-demand video streaming (7-9)

Description

- In this case, the user captures and uploads the content in real-time. However, the user doesn’t publish the content immediately for live consumption by other users. Rather, the content is published later after applying some intelligent content editing/publishing/dubbing (similar to Instagram stories.) - The user may move during the uplink streaming session. - Each time that the content is played by an audience member, the reactions of other audience members past and present (smilies, comments, likes, audio dubs, images, avatars, and animations) are superimposed on the video content, with or without the owner’s approval per reaction.

Categorization

Type: 2D Delivery: 5GMS uplink streaming, 5GMS downlink streaming, on-demand media streaming Device: Phone, HMD, Glasses

Preconditions

On the sending UE, a 3GPP supported encoder is installed. On every receiving UE, 5GMS downlink streaming client is provided, possibly with the server-guided client-side ad-insertion capability. In the 5GMS System, one or more Application Servers are installed, running various instances of decoding/encoding or transcoding. On the network, QoS may be offered to some services including guarantees on bitrates, latency, loss rate. In the 5GMS System, content protection content encryption is provided as services on Application Servers. In the network, ad server services (including ad decision, ad preparation, and ad serving) are provided as first- or third-party services for inserting the ads on the service or the client-side. In the 5GMS System, the Application Server may have a location and device profile of viewers of high demand streamers. Network storage is available in the 5GMS System to store the encoded content for time-shifted downlink streaming.

Requirements in terms of Capabilities and QoS/QoE Considerations

- Capabilities - Near real-time transcoding and packaging to different codecs and different bit-rates - On-the-fly transcoding and packaging depending on the receiving UEs’ support for codec, resolution, and bandwidth - Content caching and CDN services - KPI - Subjective/objective quality of the encoded streams - Adaptation of encoders to the requested characteristics - Changing the number of streams based on the demand - Changing the workflow distribution among multiple edge server based on the number of viewers, their location and their device characteristics - Seamless integration of ads with the main content - Efficiency in terms of - Manifest generation - Target-based advertising - Caching the content based on the number of concurrent users and for the use in the feature. - Workflow management and dynamic distribution

Feasibility and Industry Practices

See the corresponding section in the “User-generated live streaming” Use Case.

Nominal Cost Analysis

See the corresponding section in the “User-generated live streaming” Use Case.

Benefits and Impact

See the corresponding section in the “User-generated live streaming” Use Case.

Potential Technical Requirements

See the corresponding section in the “User-generated live streaming” Use Case.

Potential Standardization Status and Needs

- Discovery of dynamic processing capabilities of various edge networks: - Available hardware resources at the current time - Built-in (hardware-assisted and/or software optimized) encoders/transcoders - Built-in packager, manifest translator and manipulator functions - Supported encryption and content protection schemes - Available third-party function libraries - First- and third-party workflow templates - Moving media workflow from one AF/AS pair to another AF/AS pair: Move a media workflow such as multi-rate transcoding from one AF/AS pair to another AF/AS pair during the media session. - Just-in-time multi-rate transcoding between multiple AF/AS pairs: Fire up different transcoding workflows during the media sessions that are split between different AF/AS pairs. - Parallel processing of content segments in the same AF/AS pair, or multiple AF/AS pairs: For stored content, the content might be parallel processed, i.e. transcoded in parallel streams to increase the speed for content-preparation.

## Remote live streaming of local programs

Use Case Name

Remote live streaming of local programs with or without ads (10, 11)

Description

A service provider offers local programs to the remote user as a subscription service. Since the program is not popular with many users when the user is in a remote area, the content is delivered to a remote edge server close to the user to be transcoded there. The programs might have pre-roll and mid-roll ads based on the location of the user.

Categorization

Type: 2D Delivery: 5GMS downlink streaming, live streaming Device: Phone, HMD, Glasses

Preconditions

On every receiving UE, a 5GMS downlink streaming client is provided, possibly with the server-guided client-side ad-insertion capability. In the 5GMS System, one or more Application Servers are installed, running various instances of decoding/encoding or transcoding. On the network, QoS may be offered to some services including guarantees on bit rates, latency, loss rate. In the 5GMS System, content protection and content-encryption are provided as services on Application Servers. In the network, ad server services (including ad decision, ad preparation, and ad serving) are provided as first- or third-party services for inserting the ads on the service or the client-side.

Requirements in terms of Capabilities and QoS/QoE Considerations

- Capabilities - Real-time transcoding and packaging to different codecs and different bit-rates - On-the-fly transcoding and packaging depending on the receiving UEs’ support for codec, resolution, and bandwidth - Content caching and CDN services - KPI - Subjective/objective quality of the encoded streams - Adaptation of encoders to the requested characteristics - Changing the number of streams based on the demand - Changing the workflow distribution among multiple edge server based on the number of viewers, their location and their device characteristics - Seamless integration of ads with the main content - Efficiency in terms of - Manifest generation - Target-based advertising - Caching the content based on the number of concurrent users and for the use in the feature. - Workflow management and dynamic distribution

Feasibility and Industry Practices

See the corresponding section in the “User-generated live streaming” Use Case.

Nominal Cost Analysis

See the corresponding section in the “User-generated live streaming” Use Case.

Benefits and Impact

See the corresponding section in the “User-generated live streaming” Use Case.

Potential Technical Requirements

Allocation of transcoding resources on different edges and network processing elements, depending on the distributions of the clients and diversity of them.

Potential Standardization Status and Needs

- CDN support - Discovery of dynamic processing capabilities of various edge networks: - Available hardware resources at the current time - Built-in (hardware-assisted and/or software optimized) encoders/transcoders - Built-in packager, manifest translator and manipulator functions - Supported encryption and content protection schemes - Available third-party function libraries - First- and third-party workflow templates - Moving media workflow from one AF/AS pair to another AF/AS pair: Move a media workflow such as multi-rate transcoding from one AF/AS pair to another AF/AS pair during the media session. - Just-in-time multi-rate transcoding between multiple AF/AS pairs: Fire up different transcoding workflows during the media sessions that are split between different AF/AS pairs.

## Remote On-demand streaming of local programs

Use Case Name

Remote On-demand streaming of local programs (12, 13)

Description

A service provider offers local programs to the remote user as a subscription video-on-demand service. Since the program is not popular with many users when the user is in a remote area, the content may need to deliver to the remote edge servers on the user and be transcoded there. The programs might have targeted ads.

Categorization

Type: 2D Delivery: 5GMS downlink streaming, on-demand streaming Device: Phone, HMD, Glasses

Preconditions

On every receiving UE, a 5GMS downlink streaming client is provided, possibly with the server-guided client-side ad-insertion capability. In the 5GMS System, one or more application servers are installed, that run various instances of decoding/encoding or transcoding. On the network, QoS may be offered to some services including guarantees on bit rates, latency, loss rate. In the 5GMS System, content protection and content-encryption are provided as services on Application Servers. In the network, ad server services (including ad decision, ad preparation, and ad serving) are provided as first- or third-party services, for inserting the ads on the service or the client-side. In the 5GMS System, network storage is available for storing the encoded content for time-shifted streaming.

Requirements in terms of Capabilities and QoS/QoE Considerations

- Capabilities - Real-time transcoding and packaging to different codecs and different bit-rates - On-the-fly transcoding and packaging depending on the receiving UEs’ support for codec, resolution, and bandwidth - Content caching and CDN services - KPI - Subjective/objective quality of the encoded streams - Adaptation of encoders to the requested characteristics - Changing the number of streams based on the demand - Changing the workflow distribution among multiple edge server based on the number of viewers, their location and their device characteristics - Seamless integration of ads with the main content - Efficiency in terms of - Manifest generation - Target-based advertising - Caching the content based on the number of concurrent users and for the use in the feature. - Workflow management and dynamic distribution

Feasibility and Industry Practices

See the corresponding section in the “User-generated live streaming” Use Case.

Nominal Cost Analysis

See the corresponding section in the “User-generated live streaming” Use Case.

Benefits and Impact

See the corresponding section in the “User-generated live streaming” Use Case.

Potential Technical Requirements

Allocation of transcoding resources on different edges and network processing elements, depending on the distributions of the clients and diversity of them.

Potential Standardization Status and Needs

- CDN support - Discovery of dynamic processing capabilities of various edge networks: - Available hardware resources at the current time - Built-in (hardware-assisted and/or software optimized) encoders/transcoders - Built-in packager, manifest translator and manipulator functions - Supported encryption and content protection schemes - Available third-party function libraries - First- and third-party workflow templates - Moving media workflow from one AF/AS pair to another AF/AS pair: Move a media workflow such as multi-rate transcoding from one AF/AS pair to another AF/AS pair during the media session. - Just-in-time multi-rate transcoding between multiple AF/AS pairs: Fire up different transcoding workflows during the media sessions that are split between different AF/AS pairs. - Parallel processing of content segment in the same AF/AS pair, or multiple AF/AS pairs: For stored content, the content might be parallel processed, i.e. transcoded in parallel streams to increase the speed for content-preparation.

## Private multi-player face streaming during game

Use Case Name

Private multi-player face streaming during a game (6)

Description

A few friends set up a multi-player game session among themselves. While players are using a third-party platform to play the game, they would like to also see each other’s faces while playing the game as one or more picture-in-picture items. Each player can see one or multiple thumbnail streaming videos of their friends during the game session.

Categorization

Type: 2D Delivery: 5GMS uplink streaming, 5GMS downlink streaming, live streaming, low –latency streaming Device: Phone, HMD, Glasses

Preconditions

On every sending UE, a 3GPP-supported encoder is installed. On every receiving UE, a 5GMS downlink streaming client is provided, possibly with the server-guided client-side ad-insertion capability. In the 5GMS System, one or more Application Servers are installed, running various instances of decoding/encoding or transcoding. On the network, QoS may be offered to some services including guarantees on bit rates, latency, loss rate. In the 5GMS System, content protection and content-encryption are provided as services on Application Servers. On the network, each user uplink streams and downlink streams video at the same time with low delay and in synchronisation with the video game.

Requirements in terms of Capabilities and QoS/QoE Considerations

- Capabilities - Real-time transcoding and packaging to different codecs and different bit-rates - On-the-fly transcoding and packaging depending on the receiving UEs’ support for codec, resolution, and bandwidth - Content caching and CDN services - KPI - Subjective/objective quality of the encoded streams - Adaptation of encoders to the requested characteristics - Changing the number of streams based on the demand - Changing the workflow distribution among multiple edge server based on the number of viewers, their location and their device characteristics - Seamless integration of ads with the main content - Efficiency in terms of - Manifest generation - Target-based advertising - Caching the content based on the number of concurrent users and for the use in the feature. - Workflow management and dynamic distribution

Feasibility and Industry Practices

See the corresponding section in the “User-generated live streaming” Use Case.

Nominal Cost Analysis

See the corresponding section in the “User-generated live streaming” Use Case.

Benefits and Impact

See the corresponding section in the “User-generated live streaming” Use Case.

Potential Technical Requirements

Allocation of encoding resources on different edges and network processing elements, depending on the distributions of the clients and diversity of them.

Potential Standardization Status and Needs

- Discovery of dynamic processing capabilities of various edge networks: - Available hardware resources at the current time - Built-in (hardware-assisted and/or software optimized) encoders/transcoders - Built-in packager, manifest translator and manipulator functions - Supported encryption and content protection schemes - Available third-party function libraries - First and third-party workflow templates - Moving media workflow from one AF/AS pair to another AF/AS pair: Move a media workflow such as multi-rate transcoding from one AF/AS pair to another AF/AS pair during the media session. - Just-in-time multi-rate transcoding between multiple AF/AS pairs: Fire up different transcoding workflows during the media sessions that are split between different AF/AS pairs. - Split the workflow between the sending/receiving UEs and AF/AS pair: Static and dynamic splits of the workflow between the sending/receiving UEs and an AF/AS pair. - Split the workflow between multiple AF/AS pairs: Static and dynamic workflow splits between multiple AF/AS pairs based on the proximity to each participant in the media session. - Change of the media workflow by using intermediate formats for optimizing split rendering in the sending/receiving UEs: Modification of workflow when split rendering between the sending/receiving UEs and the AF/AS pair to optimize the network bandwidth

## Common edge required features across various use-cases

The following table captures the various features needed for each use case:

| Features | User-generated live streaming | User-generated On-demand streaming | Remote live streaming of local channels | Remote On-demand of local channels | Multiplayer Game face streaming |
| --- | --- | --- | --- | --- | --- |
| Edge capability discovery | X | X | X | X | X |
| Edge switching for uplink | X | X |  |  |  |
| Edge switching for downlink | X | X | X | X |  |
| CDN | X | X | X | X |  |
| Workflow splitting | X | X | X | X | X |
| Dynamic workflow reconfiguration | X | X | X | X | X |
| Split rendering |  |  |  |  | X |
| Low delay delivery (2-6 sec) | X |  | X |  |  |
| Ultralow delay delivery (sub-sec) |  |  |  |  | X |
| Content decoding | X | X | X | X | X |
| Content encoding | X | X | X | X | X |
| Content storage | X | X | X | X |  |
| Content insertion | X | X | X | X |  |
| Content encryption | X | X | X | X |  |

## Description of features

- Edge capability discovery: Discovery capabilities of one edge or network element resources:

  - Available hardware resources at the current time

  - Built-in (hardware-assisted and/or software optimized) encoders/transcoders

  - Built-in packager, manifest translator and manipulator functions

  - Supported encryption and content protection schemes

  - Available third-party function libraries

  - First- and third-party workflow templates

- Edge switching for uplink: Switch the edge receiving the uplink from the user to another edge due to mobility of the user or for the purpose of load-balancing or achieving better performance, e.g. lower latency, higher bandwidth.

- Edge switching for downlink: Switch the edge providing the downlink to one or more viewers to another edge due to mobility of the user or for the purpose of load-balancing, achieving better performance, e.g. lower latency, higher bandwidth.

- CDN: CDN-like multi-stage network content caching.

- Dynamic workflow reconfiguration: change of media workflow during media session without any interruption in service, e.g. firing up more transcoders, on-the-fly transcoding, use of the intermediate format and adding transcoders at the downlink edge, firing up parallel encoders/transcoder, chaining of workflow split due to change of the number of users/topology of the workflow.

- Workflow splitting: Dividing a workflow to sub-workflow, each running in a different edge or network element.

- Split-rendering: Dividing a workflow (and change it properly) between the edge and UE, e.g. for reducing the workflow latency or reducing the battery consumption of device.

- Low delay delivery (2-6 sec): Latency in the range of broadcast latency.

- Ultralow delay delivery (sub-sec): Latency for pseudo-conversational applications, e.g. face synchronization with the game.

- Content decoding: To re-encode uplinked material into required downlink formats.

- Content encoding: One or multiple instances of encoding for the purposes of distribution over downlink streaming.

- Content storage: High volume and/or long-term storage as distinct from content caching in a CDN node.

- Content insertion: For ad-insertion, but also for other personalisation features.

- Content encryption: To support content protection requirements.

# Proposal

We propose the above updated use cases and requirements to be included in FS_ESMA permanent document.