3GPP TSG SA4 Joint MTSI MBS Conf Call	Tdoc S4-AHM360

July, 24th, 2017

Agenda item: 	4

Source: 	Ericsson LM

Title: 	Use-Cases and a reference architecture for FLUS

Document for	Agreement

# Introduction

This document contains a set of use-cases, realization considerations, requirements and a reference architecture for the Live Uplink Streaming Framework. The intention is to capture these in the permanent document.

# Use-Case Descriptions

## Use-Case: Live uplink video stream from drones or moving vehicles

### Description

The media producer for an event is using drone-mounted-360 cameras or other moving vehicles like F1 cars, sailing boats or bicycles to capture scenes from more innovative angles. The drone is flown using line of sight, i.e. the drone pilot has direct visual contact to the drone. Other vehicles may have the driver / pilot on-board.

The live video is streaming to the live ingest server and then used together with other camera feeds in a live TV broadcast.

Use-Case example: An event-organizer plans to use multiple camera-mounted-drones to capture live video from an event. All live video streams should be routed to an editing facility, where a program direct decides on the sequencing of live video into a single linear program. The media source of each drone is configured with their own target bitrate (quality) and target delay. Each media source is configured with a unique media sink so that the program director can identify each media source.

### Realization Considerations

- The media producer company may have a B2B relation with an operator

- Events are scheduled a long time in advance and the network capabilities in that area can be assessed (and improved when needed) in advance.

- The live video of a media source may be delayed by a configurable delay, e.g. to prioritize quality over latency.

- When the media producer collaborates with the MNO, the MNO may provide guaranteed QoS to provide a suitable bitrate.

- Only authorized media sources can send content into a media sink. User-Plane video data from not-authorized sources are discarded.

- The drone may not have a UI. It should be considered, whether drone configuration happens via 3GPP access.

### Potential Requirements

- The solution should support high bitrate uplink to upstream high quality video. Media Sources can be configured with a target bitrate (target quality)

- The solution should support configurable uplink streaming delays in order to compensate for bitrate variations.

- The solution should be able to leverage QoS bearers for high bitrate uplink streaming.

- The solution should allow for short interruptions, e.g. caused by handovers, (i.e. when the radio connection is switched over to another radio base station)

- The solution should support longer interruptions and link bitrate changes, e.g. caused by Inter-RAT handovers (e.g. from NR to LTE).

- The solution should support 360 video, UHD and other high bitrate video formats.

- The supported bitrate and the service delay requirements are FFS

- The ingest server may be operated by a 3rd party provider, which uses either subscription based QoS or 5G SBA for QoS and data plane.

- Media sources like drone based media sources may have a configuration interface to configure the Uplink Streaming service parameters.

## Use-Case: Breaking-News reporter

### Description

A News Corporation uses 5G and mobile equipment to speed up and simplify their breaking-news operations. Either, professional cameras are equipped with 5G uplink streaming modems, or regular smartphones (with external microphones) are used for video capturing. The universally available 3GPP coverage is used to stream the live video (with configurable, low delay) from the breaking news scene into the broadcast operation studio.

A news corporation negotiates a service level agreement with an operator so that a set of reporters can do sequential or simultaneous live reports. The general frame agreement between the news corporation and the MNO foresees, that each reporter can determine its own maximal video quality (measured in bit per sec). Each reporter should set its own quality, but some reporters are allowed to provider higher quality (i.e. use higher bitrates) than others.

### Realization Considerations

- The news corporation has a B2B agreement with an MNO to provide uplink QoS and special data plane tariffs.

- The MNO may have similar B2B agreements with several news corporations (or other companies).

- The company may need authorization on a per-company level and on a pre-reported level.

- Potentially, several reporters may upstream video simultaneously. The ingest needs to clearly separate users per domain (here separate between reporters of a single news corporation).

- Only authorized video sources can send data to the ingests. The user plane may require encryption.

- News events are generally unscheduled and at unknown locations. So, it is not possible to tune the network (cell coverage / capacity) for events.

- News events may not be stationary, e.g. from a moving vehicle (e.g. helicopter or car).

- The live news feed may be stored in the media source device for later upload and usage.

- The target video quality (i.e. bitrate) and target live video delay is configurable and may depend on the used access network and the news corporation requirements.

### Potential Requirements

- The solution should accept user plane traffic only from authorized media sources. User plane traffic may require encryption.

- The solution should support authorization facility on domain level (per company) and on user level (individual users).

- The solution should support different authorization levels. Each authorization level is allowed to configure / use different features.

- The solution should adapt to the currently available radio access technology.

- The solution should consider handovers within the same radio access technologies (Intra-RAT) and also to other radio access technologies (Inter-RAT).

- The ingest server and / or the media preparation may be operated by a 3rd party provider (like the news corporation).

- The solution should support domain authorization and user-level authorization of ingest service features.

## Use-Case: User-Generated Content

### Description

A user X has invested some good money to acquire a high-end live sports camera. The user is interested to live broadcast his sports event on YouTube (TM) or Facebook (TM). The user sets up an ingest account.

The user is interested to get a high probability for a sustainable bitrate for the short live broadcast. The user uses the self-service portal of an operator to schedule an uplink QoS session.

Example: A user A is interested to offer a live video feed through its social media account. The user has already a social media account and schedules a live event for the account. The user gets the ingest URL and parameters in response from the live event creation.

In order to leverage a sustainable quality service from an MNO, the user login to the self-service uplink streaming portal (provided with the MNO subscription) and schedules a live uplink streaming event. The user configures the ingest URL for the social media platform, so that the MNO ingest server can either forward the stream or can apply QoS settings to that stream.

### Realization Considerations

- Social Media Platforms like YouTube (TM) or Facebook (TM) determine the distribution video profiles. Only a single (high) media quality (according to the external provicer specifications) needs to be supported.

- The MNO offers an ingest server and a self-service portal for the ingest server, which allows the user to configure a stream-forwarder, so that uplink QoS can be controlled and monitored by the ingest server.

### Potential Requirements

- The ingest solution should support live stream configuration and forwarding to 3rd party ingest solutions.

# System Design and Reference Architecture

The reference architecture for the live uplink streaming work is depicted below. The distribution system is depicted for completeness.

The main functions are the configuration console, the media source and the ingest server. In case of a drone use-case, the configuration console is expected to be deployed on a device separate from the media source.

The live uplink stream is sent via the ingest link from the media source (the mobile device) to the ingest server (media sink). The term ‘contribution’ is often used in professional TV productions for the live ingest of media. The main characteristics of the live uplink streaming system is that the video flow is unidirectional, from the media source via some intermediate nodes to an ingest server, which may either store or replicate the content, typically to many receivers.

Figure 1: Reference Architecture for live uplink video streaming (presence of a distribution system is optional)

Media Source: The media source is the source of the live uplink video stream and streams media data at the configured target bitrate, with the configured target delay profile (aligned with the ingest server) and the configured bitrate adaptation scheme. The media source triggers the establishment of the uplink user plane for the live uplink video. The media source gets ingest parameters and authorization information from the configuration console.

Configuration Console: The function is used to configure the media source and the Ingest Server. When the configuration console is deployed on a different end-point than the media source, the configuration console is used to setup an ingest service on the ingest server and then authorizes the media source to ingest media data.

Ingest server: The function is the end-point for the live uplink streaming media data. The ingest server may compensate for network latency due to retransmissions and rate adaptation. The media data may either be stored for later processing or may be forwarded for live replication, optionally for transcoding, and for distribution. The ingest server decouples ingest from the distribution system.

The link between Media Source and Ingest Server may be QoS provisioned.

There are different distribution realizations. Adaptive bitrate streaming (like DASH) is today commonly used. Other distribution systems (IP and traditional broadcast systems) are certainly possible.

# Proposal

It is proposed to agree the use-cases, realization considerations and requirements in section 2 to be included into the permanent document for FLUS.

It is proposed to agree section 3 as base for the reference architecture and to be included into the permanent document for FLUS.