3GPP TSG-SA4 MTSI SWG on E-FLUS	S4-AHM447

# 20th December 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (dCR) E-FLUS network assistance solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony Mobile Communications, Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Tdoc S4-181435, which proposed the inclusion of the Network Assistance feature for FLUS in TR 26.939, was agreed at SA4#101. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The solution approach for Network Assistance for FLUS is proposed. Compared to the draft CR dicussed at SA4 #101 (S4-181251), the following additional aspects are covered: - Architectural aspects, based on the approved CR in S4-181458; - Outline of the protocols and messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of a specification for Network Assistance for FLUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

===== 1ST CHANGE =====

### 4.2.3	Uplink streaming for PSS-based distribution

Figure 4.2-4a shows the most important interfaces for Evolved Packet System (EPS) and 5G System (5GS) for Live Uplink Streaming Services.

Figure 4.2-4a: Architecture for Live Uplink Streaming

An uplink streaming service requires at least a FLUS source and a FLUS sink. A FLUS sink is located at the SGi reference point in case of EPS [13] and at the N6 reference point in case of 5GS [15]. When QoS support is needed, the FLUS sink may request QoS support using Rx [13] or T8 [14] reference points in case of EPS, and using Nnef or Npcf [15] reference points in case of 5GS.

When the FLUS instantiation uses the Uplink Network Assistance (UNA) function, the Network Assistance Server (NAssS) is implemented as an Application Function (AF) within the EPS. The FLUS Source UE uses the UNA protocols and messages as specified in clause 5.3.4 to discover and communicate with the NAssS.

The architecture of uplink streaming for subsequent PSS distribution is depicted in Figure 4.2-4b.

Figure 4.2-4b: Uplink streaming for PSS

The PSS Content Source is located on the UE side and contains the FLUS source.

===== END OF 1ST CHANGE =====

===== 2ND CHANGE =====

## 5.3	Generic FLUS system

### 5.3.1	System configuration

In this version of the specification, this function is left for implementation, but expected to be specified in a future version.

### 5.3.2	Session management

#### 5.3.2.1	FLUS sink capability discovery

This procedure allows a FLUS source to discover capabilities of the FLUS sink.

Figure 4.4-5: Get current FLUS sink capabilities

1. The FLUS source sends the FLUS sink capability request to a known URL of the FLUS sink.

2. The FLUS sink provides a list of FLUS sink capabilities in response.

#### 5.3.2.2	FLUS session establishment

It is assumed that the FLUS source has selected a FLUS sink and acquired the necessary information to establish an F-C connection to that FLUS sink (i.e. the HTTPS URL).

The procedure allows a FLUS source to create a new FLUS session. Session configuration properties and in particular FLUS media instantiation selection is added in subsequent procedures.

Figure 4.4-2: FLUS session creation

1. The FLUS session is created. The FLUS source provides a valid access token.

2. On successful creation, the FLUS sink responds with the resource id of the session. FLUS session properties are fetched and modified with subsequent transactions.

### 5.3.2.3	Get FLUS session properties

The procedure allows a FLUS source to fetch the current configuration of the FLUS session.

Figure 4.4-3: Get current FLUS session properties

1. The FLUS source sends along with the session property request, the access token and the resource id of the session.

2. The FLUS sink provides the FLUS session properties in response.

#### 5.3.2.4	FLUS session update

The procedure allows a FLUS source to update the current configuration of the FLUS session.

Figure 4.4-4: FLUS session update

The FLUS source may first fetch the current FLUS session configuration using the Get FLUS session properties procedure.

1. The FLUS source modifies the properties of the session resource. The procedure may allow modification of individual properties or all properties.

2. The FLUS source updates the resource identified by the id of the session.

#### 5.3.2.5	FLUS session termination

The FLUS source may explicitly terminate a FLUS session and all its provisioned and active media sessions. Alternatively, the FLUS session is automatically terminated, when the last media session of the FLUS session is terminated. The procedure allows a FLUS source to terminate a FLUS session. All media streams will be terminated automatically with the termination of the FLUS session.

Figure 4.4-6: FLUS session termination

1. The FLUS source sends the terminate FLUS session command. The access token and the resource id of the FLUS session is provided as input.

2. The FLUS sink terminates the FLUS session, including all active media streams and acknowledges the reception of the request.

#### 5.3.2.6	List of FLUS sink capabilities

The FLUS sink capabilities should include the features that are listed under 4.4.4.

#### 5.3.2.7	List of FLUS session properties

All FLUS session properties, except for the resource id, are always carried in an HTTP message body. The access-token is always carried as part of HTTP headers. Except for the FLUS session creation request (where the id is not present), the resource id shall be present in the URL of all requests that relate to a specific FLUS session.

In the table below, the following assertions are made:

-	Table header: C stands for Create FLUS session procedure, G is for Get FLUS session properties procedure, U is for Update FLUS session properties procedure and T is for Terminate FLUS session procedure. “I”, and “O” respectively denote “request” (going Into the FLUS sink), and response (going Out of the FLUS sink).

-	Optional (“O”) means that the property may or may not be sent/received during a REST transaction. It does not necessarily mean that the property is optional. It is possible, for example, that a session is not yet active because the FLUS source has not set the property in any previous update transaction using the PUT or PATCH HTTP method, as opposed to representing a hint on the importance of the property for the FLUS sink.

-	A property marked as optional (O) in a request message may be present in the request. When not present in the request body, the property, if present in the FLUS sink, will not be updated.

-	A property marked as optional (O) in a response message is only present in the response when a value is assigned or changed by the FLUS sink.

-	A property marked as mandatory (M) in a response message is always present in the response. The FLUS sink provides defaults, which may be modified subsequently by the content provider.

-	A blank cell in the table means “forbidden” (the property cannot be added to the request or returned by the FLUS sink, depending on the transaction direction)..

Table 5.3.2.7-1: List of FLUS session properties

| Property Name | Property Description | C I | C O | G I | G O | U I | U O | T I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| id | Identifier of the FLUS session resource. Note, the id is only provided within an HTTP body during the Create FLUS session response. Otherwise, the id is present in the message URL to identify the resource in the FLUS sink. Type; Unit; Default. Integer; None; N/A. |  | M |  |  |  |  |  |
| F-U Instantiation | Identifier of the FLUS media instantiation that is used by this FLUS session. Vendor specific enumeration values shall start with ‘vnd-‘ followed by a unique vendor name and optionally followed by additional characters. The F-U instantiation shall be provided as a globally unique URN. Type; Unit; Default. URI; None; All. |  |  |  | M | O |  |  |
| Entrypoint URL | Entry point URL information (e.g., SIP URL) for establishing the F-U connection to start the media streaming. Details on the Entrypoint URL is F-U instantiation specific. |  |  |  |  |  |  |  |
| Processing Description | The property stores a media processing description document that defines the post processing pipeline that the FLUS sink shall apply to received media components. The pipeline description also may also set the distribution target (incl FLUS sink storage) for the media. The format and semantics of this document are out of scope of the FLUS specification. |  |  |  | O | O |  |  |

### 5.3.3	Data transport

In this version of the specification, this function is left for implementation, but expected to be specified in a future version.

### 5.3.4	Uplink Network Assistance

#### 5.3.4.1	General

The Uplink Network Assistance (UNA) feature is facilitated by the adaptation of MPEG-SAND, in particular by the definition of a new SAND mode for UNA. Even if the intended scope of MPEG-SAND is DASH-based content delivery, the feature of Network Assistance, whether for downlink or uplink, is valid for any content container format. Hence SAND is adapted as specified in the present clause in order to facilitate the UNA feature. Server and Network Assisted DASH (SAND) functionality is specified in ISO/IEC 23009-5 [z].

The present clause describes the following:

-	The SAND mode that facilitates UNA, specified in clause 5.3.4.2;

-	NAssS discovery procedures, specified in clause 5.3.4.3;

-	SAND messages and protocols to use for UNA, specified in clause 5.3.4.4;

-	Use of SAND functionality for enabling UNA, and normative behaviors around the related message handling for NAssS and FLUS Source client, specified in clause TBD;

-	XML schema for UNA messages, specified in clause TBD.

#### 5.3.4.2	Network Assistance mode for FLUS

Uplink Network Assistance is facilitated by the corresponding mode of SAND that is specified in the present clause.

The concept of the DANE, as defined within the SAND framework, is adopted but the corresponding server function for FLUS is given the more generic name of Uplink Network Assistance Server (NAssS), since FLUS media is not constrained to be carried in a DASH container.

The UNA function enables a FLUS Source to improve the QoE of content up-streaming sessions, and is provided by the NAssS.  The NAssS is out-of-band in the SAND context, i.e. it is not in the media up-stream path. The UNA communication is independent from the FLUS media (F-U) and FLUS Control (F-C) communication paths, hence the UNA communication occurs in a separate path to the transfer of the FLUS media stream. The FLUS sink does not need to be aware of the UNA function.

Uplink Network Assistance may be made available to certain clients only, for example subject to subscription options. Client authentication may also be applied before granting access to Network Assistance. Clients are able to discover the availability and information about the NAssS, and to establish an Uplink Network Assistance session with the NAssS.

Uplink Network Assistance is based on the model of the FLUS Source requesting network assistance and the NAssS responding to the request.  The Uplink Network Assistance functionality may be granted to a client supporting the provision of FLUS media content with either only the first or with both of the two functions below, in both cases based on the FLUS Source having made a request to the NAssS for Uplink Network Assistance:

-	The NAssS indicates to the FLUS Source the highest suitable media rate for the next period of up-streamed content, based on the versions that are able to be provided;

-	The NAssS indicates to the FLUS Source a temporary delivery boost facility for occasions when the content upstream output buffer on the client risks suffering from over-run.

Once an Uplink Network Assistance session is active, the client may issue an Uplink Network Assistance call prior to sending the next portion of upstream content to the FLUS Sink. The Uplink Network Assistance call consists of a single logical signalling exchange. This exchange with the NAssS activates either the first of the above functions or a sequence of both functions; the second only if the FLUS Source was granted access to the delivery boost function. If the client does not need a delivery boost, then the NAssS omits the second function in the response to the FLUS Source.

#### 5.3.4.3	NAssS discovery

The NAssS is out-of-band with respect to the media delivery path. NAssS discovery shall be performed using the DNS protocol as described in TS 23.003 [zzz]. Toward this purpose, the UE needs a Fully Qualified Domain Name (FQDN) for the NAssS (UNA DANE). The procedures for addressing and identification for Bootstrapping MBMS Service Announcement as described in clause 15.5 of TS 23.003 [zzz] shall be used for NAssS discovery. Accordingly, the Fully Qualified Domain Name (FQDN) for the Uplink Network Assistance Server (NAssS) (“UNA DANE”) shall be "unadane.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org".

The URL to establish the out-of-band connection with the Uplink NAssS (“UNA DANE”), if provided by the network, shall be:

http://unadane.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org/

The Uplink NAssS (“UNA DANE”) FQDN is composed of six labels. The last three labels shall be "pub.3gppnetwork.org". The second and third labels together shall uniquely identify the PLMN, as described in clause 15.5 of TS 23.003 [57]. The first label shall be "unadane".

When receiving a DNS query on the Uplink NAssS FQDN, the DNS server shall respond with the information, including IP address, of the Uplink NAssS that is available to the FLUS Source for Uplink Network Assistance functionality.

#### 5.3.4.4	FLUS Network Assistance protocol and messages

##### 5.3.4.4.1	General

Uplink Network Assistance is facilitated by the corresponding mode of SAND that is specified in the present clause.

If UNA is supported, HTTP shall be supported as the minimum transport protocol for carrying UNA messages. This does not preclude that other additional transport protocols could also be implemented. The mandatory use of HTTP as a minimum transport protocol for SAND messages shall be in accordance with table 5.3.4.4.1-1 (bold font represents mandatory). SAND Metrics messages are no used in UNA.

Table 5.3.4.4.1-1: Mandatory usages of HTTP for carrying UNA messages

| Status messages | HTTP POST |
| --- | --- |
| PER messages | HTTP GET |

The NAssS is always out-of-band with respect to the FLUS media path, hence for UNA status messages, HTTP POST shall be used for communicating with the NAssS.

FLUS sources that support the UNA feature, as well as NAssSs shall further support the WebSocket protocol specified in IETF RFC 6455 [n], provided that HTTP over TLS (HTTPS) is supported by the respective FLUS client or NAssS. If HTTP over TLS (HTTPS) is not supported by a FLUS source, then that FLUS source should support the WebSocket protocol for the ‘Uplink Network Assistance’ mode.  Similarly, if HTTP over TLS (HTTPS) is not supported by a NAssS, then that NAssS should support the WebSocket protocol for the ‘Uplink Network Assistance’ mode.

##### 5.3.4.4.2	SAND messages usage and extensions for UNA

The NAssS is out-of-band with respect to the media path. The FLUS source shall send the UNA messages as the body of HTTP requests directly to the NAssS, using the HTTP POST method to send a UNA message to the NAssS.

The UNA transactions between the FLUS source and the NAssS at the logical level consist of the UNA session initiation and termination transactions, the Network Assistance request and the Network Assistance response.

A combination of existing SAND messages as defined in the SAND specification, with usage as defined in clause 5.3.4.4.3, and SAND extension messages as defined in clause 5.3.4.4.4, are used.

##### 5.3.4.4.3	Use of existing SAND messages

###### 5.3.4.4.3.1	Shared resource allocation

The SAND status message SharedResourceAllocation is used in the UNA request from the FLUS source to the NAssS, in order to provide information about the available media bitrates for the content to be delivered on interface F-U.

The parameters operationPoints and bandwidth in the SAND message SharedResourceAllocation shall represent each of the available media bitrates, indicated as the sum of all media components in each case.

###### 5.3.4.4.3.2	Buffer level

In the case that the FLUS source is requesting a network delivery boost during the following media stream chunk, the SAND metrics message BufferLevel is used to inform the NAssS of the current buffer level for the content item being delievered on interface F-U.

###### 5.3.4.4.3.3	Shared resource assignment

The SAND PER message SharedResourceAssignment is used in the UNA response from the NAssS to the FLUS source, in order to provide the recommended choice of bitrate version for the next chunk of the content item being accessed.

##### 5.3.4.4.4	SAND message extensions

###### 5.3.4.4.4.1	UNA session initiation

The FLUS source initiates a UNA session by sending the NetworkAssistanceInitiation SAND extension message to the NAssS within a SAND message envelope.

The NAssS responds to the session initiation request with the NetworkAssistanceInitiationResponse SAND extension message to the FLUS source.

###### 5.3.4.4.4.2	UNA session termination

The FLUS source terminates a UNA session by sending the NetworkAssistanceTermination SAND extension message to the NAssS within a SAND message envelope.

The NAssS responds to the session termination request with the NetworkAssistanceTerminationResponse SAND extension message to the FLUS source.

###### 5.3.4.4.4.3	Segment duration

The SegmentDuration SAND extension status message is used to provide the nominal duration for the upcoming media stream chunk.

###### 5.3.4.4.4.4	Delivery boost request

The FLUS source may include a request for uplink delivery boost in the UNA request, when there is a risk that buffer overflow is imminent. The request is valid for the time period of the upcoming media stream chunk, as determined by the FLUS source and indicated in the SegmentDuration message. The FLUS source may request uplink delivery boost for the delivery of the first media stream chunk, in order to accelerate the start of delivery. If buffer over-run remains to be an imminent risk then the FLUS source may repeat the uplink delivery boost request for the following media stream chunk. The client may request uplink delivery boosts until it attains a satisfactory media buffer fullness level. The decision to grant or decline any boost request lies entirely with the network policy in place at the NAssS.

###### 5.3.4.4.4.5	Delivery boost response

The delivery boost response is actuated by including the DeliveryBoostResponse message in the UNA response. It is classed as a SAND PED message extension. It indicates whether the NAssS grants or declines the corresponding uplink delivery boost request.

##### 5.3.4.4.5	UNA transactions

###### 5.3.4.4.5.1	General

The UNA transactions consist of the UNA request and the UNA response compound messages. Each of these compound messages consists of several SAND messages and/or several SAND extension messages as defined in the present specification, contained in a single SAND envelope message.

###### 5.3.4.4.5.2	UNA request

The UNA request transaction is realised by using a single SAND message envelope containing the following SAND messages:

-	SegmentDuration SAND extension message;

-	SharedResourceAllocation SAND status message;

-	Optionally the DeliveryBoostRequest SAND extension message, if the FLUS source is requesting uplink delivery boost during the upcoming media media stream chunk;

-	BufferLevel SAND metrics message, which shall be included if the DeliveryBoostRequest message is included in the UNA request transaction.

The SAND message envelope is used to carry the UNA request compound message, with the following constraints:

-	The senderId element shall be used by the UE as a reference for the UNA transaction;

-	The generationTime element is not required for UNA, hence it shall be omitted;

-	The messageId element is not used and shall be omitted in all UNA messages.

###### 5.3.4.4.5.3	UNA response

The UNA response transaction is realised by using a single SAND message envelope containing the following SAND messages:

-	SharedResourceAssignment SAND PER message;

-	Optionally the DeliveryBoostResponse SAND extension message.

-	The NAssS provides the recommended bitrate from those listed in the UNA request using the Bandwidth element in the SharedResourceAssignment message.

The validityTime element is used to express the end time of validity of the recommendation. If it is omitted then the client shall assume the validity is for the whole media media stream chunk duration, i.e. for the duration indicated in the UNA request.

The DeliveryBoostResponse message indicates whether the requested uplink boost, if requested for the upcoming media stream chunk, is granted or declined. If no uplink boost request was made then the UNA response transaction shall not contain a DeliveryBoostResponse message.

===== END OF 2ND CHANGE =====