3GPP TSG-SA4 MTSI SWG E-FLUS	S4-AHM452

# 17th January 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.939 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (pCR) E-FLUS network assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony Mobile Communications, Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 2018-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Perceived overlap of Network Assistance feature with ANBR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Elaboration of the rationale for the Netwrok Assistance approach, as previously presented in a discussion contribution, here proposed as TR text. Changes are drafted against the latest PD (S4-181444). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of background information on Network Assistance for FLUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

===== 1ST CHANGE =====

## 11.1	Introduction

The feature of Network Assistance has been defined so far for 3GPP content downlink services, in particular 3GP-DASH in TS 26.247 [17], including the extension of the 3GP-DASH file format specification in TS 26.244 [18] to adopt the MPEG-SAND messaging construct for its realization. While SAND was developed within the scope of DASH-based content delivery, the general concept of “network-assisted” content delivery is valid for any content container format. Hence this messaging construct is adapted to realise the equivalent functionality for uplink content provision in FLUS.

MPEG-SAND provides a generic message container format and a set of messages. The adaptation and extension of MPEG-SAND to realise downlink Network Assistance for PSS uses largely content container format-independent messages. The only parameter defined within those messages that originates from the principles of DASH is the segmentDuration message element. This element is re-purposed to mean any arbitrary regular time period of a content stream, whether in a segmented format or not.

The applicability and usage of Network Assistance for the various FLUS content formats is specified in TS 26.238 [2].

The key functional entity of SAND, namely the DANE (DASH-Aware Network Element), is inappropriately named for the purposes of FLUS Network Assistance. Hence a more appropriate name for the corresponding network element is “Network Assistance Server” (NAssS). This network element is generic, in that in principle it could be used for 3GPP services other than FLUS.

The SAND PER and status messages that are appropriate for usage to realise Uplink Network Assistance are specified in TS 26.238 [2].

As with downlink Network Assistance for PSS, DANE discovery for FLUS Network Assistance will be realised best using the DNS-based discovery method rather than methods that use the sand:Channel MPD element, which is in any case a DASH-specific tool.

The MTSI specification [16] already includes a facility to make bitrate recommendations for both uplink and downlink media streams, using the Access Network Bitrate Recommendation (ANBR) feature. However, there are two considerations around ANBR that make it unsuitable for the realisation of the UNA feature. The first is that ANBR does not include the boost request/response mechanism, which is the second important aspect of UNA and Network Assistance in general, in addition to the bitrate recommendation function. The second issue is that, as stated in clause 10.7.1 of the MTSI specification [16], ANBR is performed by the “mapping” of the “conceptual” ANBR and ANBRQ messages to MAC layer messages that vary according to which RAN technology is being used. Such mappings are not specified in MTSI, for very good reason – they rely on UE platform-specific implementation, which is out of scope of 3GPP specifications, and hence cannot be relied upon for the implementation of a feature such as UNA. UNA needs to be specified completely at the application layer as concerns the UE, leaving the implementation of cross-layer signalling or mapping at the discretion of the network infrastructure provider, who provides the NAssS function. Thus UNA is much more easily managed on the UE within the application layer, avoiding breaking the well-established practice of not relying on proprietary cross-layer functionality in the UE.

===== END OF 1ST CHANGE =====