Source:	Qualcomm Incorporated

Title:	On VRStream LiQuImAS loudspeaker test setup

## Document for:	Agreement

## Agenda Item:	Unique

# Introduction

At SA4#97, the SA4 plenary agreed in [1] on the following aspects pertaining to Audio Quality Characterization tests for VRStream:

The following characterization test shall be conducted and results provided and documented: Codec Quality Characterization (Tests 1 and 2) - ITU-R BS.1534-3 - Evaluates Basic Audio Quality - Test 1 - Using Reference Renderer for Reference and Degraded conditions - Over loudspeakers - Test 2 - Optional - Using Common Informative Binaural Renderer for Reference and Degraded conditions - Over headphones - All test materials for all proponents are according to corresponding LiQuImAS test plan. Reference Binaural Renderer Quality Characterization (Test 3) - 3GPP TS.26.260 (LiQuImAS) - Rendering Test (TBD in LiQuImAS) - Over headphones with head tracking - Characterize media profile performance with Reference Renderer and optionally but strongly recommended media profile performance with Common Informative Binaural Renderer. - Evaluates the following attributes: - Spatial Quality - Artefacts - Timbre Quality - Overall Quality - All test materials for all proponents are according to corresponding LiQuImAS test plan.

The agreement in [1] leaves an open question on the loudspeaker test setup for Test 1. The present document makes a proposal on loudspeaker test setups to be used as part of a LiQuImAS subjective test plan targeting the evaluation of potential VRStream audio codecs.

# Discussion

Selection of loudspeaker test setups is required to conduct Test 1 of [1]. The non-uniform loudspeakers density of standardized loudspeaker configurations (especially in the hemisphere below the listener’s ears) limits the ability to represent fully immersive sound fields. To minimize this limitation, the source generally proposes to utilize a 22.2 loudspeaker configuration (System H in [3]) for Test 1.

Compared to other possible immersive configurations such as 5.1+2 or 7.1+4, the 22.2 configuration features:

- loudspeakers below ear level

- a higher density of elevated loudspeakers

- a higher density of loudspeakers at ear level where the auditory spatial resolution is known to be most critical.

Thus, among standardized loudspeaker configurations, the 22.2 loudspeaker configuration provides the best listening conditions for the codec characterization test.

# Proposal

The source proposes to agree on the following aspects:

- The loudspeakers and listening room acoustics shall comply to the recommendations in ITU-R BS.1116-3 [2].

- All loudspeaker configurations to be used for Test 1 of [1] shall be according to ITU-R BS.2051-1 [3].

- Loudspeaker configurations for Channel-based audio:

  - All Channel-based only test materials, created for different immersive loudspeaker setups, shall be reproduced in their original loudspeaker configuration.

  - All mixed Objects and Channel-based test materials, created for different immersive loudspeaker setups, shall be reproduced in their original loudspeaker configuration.

  - All mixed Scene and Channel-based test materials, created for different immersive loudspeaker setups, shall be reproduced in their original loudspeaker configuration.

- Loudspeaker configuration for Object-based audio:

  - All Object-based only test materials shall be reproduced in a 22.2 loudspeaker configuration (System H in [3]).

  - All mixed Objects and Scene-based test materials shall be reproduced in a 22.2 loudspeaker configuration (System H in [3]).

- Loudspeaker configuration for Scene-based audio:

  - All Scene-based test material shall be reproduced in a 22.2 loudspeaker configuration (System H in [3]).

# References

[1] S4-180318, Proposal for Submission Information for VRStream audio – Qualcomm Incorporated, Source: Fraunhofer IIS, ORANGE – Fukuoka, Japan, February 2018.

[2] ITU-R BS.1116-3: “Methods for the subjective assessment of small impairments in audio systems including multichannel sound systems,” International Telecommunication Union, Geneva, 2015.

[3] ITU-R BS.2051-1: “Advanced sound system for programme production,” International Telecommunication Union, Geneva, 2017.