3GPP TSG-SA4 Video SWG Telco on VRStream	AHVIC-113

09 January 2018

Agenda item:	4.2

Source: 	Fraunhofer HHI

Title: 	Addition of local playback case to the client reference architecture in TS 26.118

Document for	Agreement

## Introduction

During the ad-hoc meeting in Santa Clara, use cases were considered in which the VR content present on the local storage of the device is played back. As a result of the discussion, having a separate figure on Client Reference Architecture demonstrating Local playback functionality was agreed.

Furthermore, at the ad-hoc meeting, the definition of “3GPP VR Track” was discussed. It was decided to agree on a definition via offline discussions and provide a definition under Clause 4.3.

This document provides the necessary additions to Clause 4.3 to include the local playback of VR content. Further changes were made to Clause 4.4 to align the terminology with Clause 4.3.

________________________________________________________________________________________________

## 4.3	Client Reference Architecture

This section provides an overview of client reference architecture for VR streaming applications and describes their components and interfaces.

Editor’s Note: The text of this section needs to be checked carefully for consistency and definitions such that it aligns with clauses 4.1 – 4.2.

Editor’s Note: This section needs to be checked by audio experts if the audio system aligns.

Figure 4.6 and Figure 4.7 show a high-level structure of the client reference architecture for VR DASH streaming and VR local playback, respectively, which consist of five functional components:

- VR Application:  The VR application controls the rendering depending on the user’s current field of view or display capabilities. The application may communicate with other functional components, e.g., the access engine or the file decoder. The access engine or file decoder may pass some abstracted control information to the VR application and the application makes the decision on which viewport to request or extract.

- Access Engine: The access engine connects through a 3GPP bearer and provides a conforming VR presentation to the receiver. The access engine fetches the Media Presentation Description (MPD), constructs and issues requests and receives DASH resources. In the case of local playback, 3GPP VR Track is fetched from the local storage. The access engine may interface with the VR application function to dynamically change the delivery session. The access engine dynamically provides a conforming 3GPP VR track to the file decoder.

- File Decoder: The file decoder processes the VR Track to generate signals that can be processed by the renderer. The file decoder typically includes at least of two sub-modules; the file parser and the media decoder. The file parser processes the file format data structures, extracts elementary streams, and parses the metadata, if present. The processing may be supported by dynamic field of view information provided by the VR application. The media decoder decodes media streams of the selected tracks and provides decoded signals. The file decoder outputs the decoded signals and metadata which is used for rendering.

- VR Renderer: The VR Renderer uses the decoded signals and rendering metadata and provides a field of view information taking into account the current field of view and possible other information. With the viewing orientation, the user field of view is determined by horizontal/vertical field of view of the screen of a head-mounted display or any other display device used to render the appropriate part of decoded video or audio signals. For video, textures from decoded signals are projected to the sphere with rendering metadata received from the file decoder. Each sample of the decoded signal is remapped to a position on the sphere using the rendering metadata. Likewise, decoded audio signals are rendered in the reference system domain. The appropriate part of video and audio signals for a current field of view is generated by synchronizing and spatially aligning the rendered video and audio.

- Sensor: The sensor extracts the current field of view tracking the user’s movement. The current field of view may for example be determined by head tracking or possibly also eye tracking functionalities. The current field of view may also be used to control the access engine to select or file decoder to select an appropriate viewport.

The primary part of this specification is the file decoder which enables to generated decoded signals and rendering metadata based on a conforming 3GPP VR Track. A 3GPP VR Track is a well-defined conformance point for a VR File decoder that enables to represent the contained media in the VR reference system (spatially and temporarily).

Figure 4.6 – Client Reference Architecture for VR DASH Streaming Applications

Figure 4.7 – Client Reference Architecture for VR Local Playback

NOTE: the dashed arrows indicate optional interfaces between components in the Figure 4.X. Viewport information should optionally be input to the access engine and file decoder.

Editor’s Note: RTP streaming case will be considered in further study

## 4.4	Rendering Schemes and Media Profiles

Editor’s Note: The text of this section needs to be checked carefully for consistency and definitions such that it aligns with clauses 4.1 – 4.3.

Editor’s Note: This section needs to be checked by audio experts if the audio system aligns.

This specification provides several interoperability points that may be referred external specifications. Two key elements are

- Media profiles providing DASH, file format and elementary stream constraints for a single media type

- Scheme types for post-decoder processing of decoder output signals together with rendering metadata.

Both features provide interoperability requirements for receivers. Figure 4-Y and Figure X provide an overview on this.

Figure 4-Y – Interoperability aspects for 3GPP VR Profiles for VR DASH Streaming Applications

Figure X – Interoperability aspects for 3GPP VR Profiles for VR Local Playback

Editor’s Note: Remove the text in the arrows (especially metadata) and make it more like APIs

Editor’s Note: Diagram should be updated with interoperability aspects also on API level

Editor’s Note: an editable version of Figure X in visio needs to be made available

A media profile for timed media is defined as requirements and constraints for a set of one or more 3GPP VR tracks of a single media type. The conformance of a set of one or more 3GPP VR tracks to a media profile is specified as a combination of:

- Specification of which sample entry type(s) are allowed, and which constraints and extensions are required in addition to those imposed by the sample entry type(s).

- Constraints on the samples of the tracks, typically expressed as constraints on the elementary stream contained within the samples of the tracks.

The elementary stream constraints of a media profile may be indicated by a requirement to comply with a certain profile and level of the media coding specification, possibly including additional constraints and extensions, such as a requirement of the presence of certain information for rendering and presentation.

Each video media profile specified in this specification includes a file decoding process such that all file decoders that conform to the video or image media profile will produce numerically identical cropped decoded pictures when invoking the file decoding process associated with that video or image media profile for a set of 3GPP VR tracks conforming to the video media profile or a set of 3GPP VR tracks conforming to the image media profile, respectively. A bitstream that conforms to the elementary stream constraints specified for the video media profile is reconstructed as an intermediate product of the file decoding process. Output of the file decoding process consists of all of the following:

- a list of decoded pictures with associated presentation times;

- for projected omnidirectional video VR rendering metadata

A file decoder conforms to the file decoding process requirements of this document when it complies with both of the following:

- The file decoder includes a conforming decoder that produces numerically identical cropped decoded pictures to those produced by the file decoding process specified for the video or image media profile in clause 5 (with the correct output order or output timing, as specified in the video or image coding specification of the video or image media profile, respectively).

- The file decoder outputs rendering metadata that is equivalent to that produced by the file decoding process specified for the video or image media profile in clause 5 (with the correct association of the rendering metadata to particular cropped decoded pictures, as specified in this document).

A player claiming conformance to a video or image media profile shall include a file decoder complying with the file decoding process of that video media profile as specified above. While the player operation, with the exception of the file decoding process, is not specified normatively in this document, specifications of a media profile may include an informative clause on expectations of a player operation, for example including recommendations for rendering.

Each audio profile specified in clause 6 includes a file decoding process such that all file decoders that conform to the audio profile will produce an output according to the specification of the file decoding process. For audio, the conformance of the output signal is defined by the file decoding process. The output of the file decoding process provides a signal that can be represented in the reference system.

In addition to the interoperability on track level, also a DASH level interoperability for each media profile is defined. This interoperability includes the signalling and content generation, such that by dynamic switching based on network constraints or sensor input a conforming 3GPP VR Track for this media profile may be obtained.

_______________________________________________________________________________________________

## Proposal

Integrate the proposed changes in Clauses 4.3 and 4.4 into TS 26.118.