Source:	Fraunhofer IIS

Title:	Detailed experiment description of P.SUPPL800 tests for IVAS characterization testing

Document for:	Discussion and Agreement

Agenda Item:	2.4

Introduction

During the last Audio SWG Ad-hoc meeting on March 11th, there was agreement on accepting the proposal "Alternative 1 variant 1" of [1] for the allocation of 23 P.SUPPL800 experiments and Table 7 of the test plan for characterization IVAS-8b [2] was updated accordingly.

In this contribution, to complement the proposal of [1], the detailed description of each experiment in Annex F of [2] is provided. This additional information aims to provide a detailed overview and better understanding of the proposed experiments.

Proposal

It is proposed that Annex F of [2] is modified according to the following:

----------------------------------

P.SUPPL800 Experiments

[

Experiment P800-1: Stereo

Tables  F.1.1 to  F.1.5 show conditions to be used for this experiment, list of preliminaries ,full list of conditions, and definition of Speech categories, and Mixed content and Music categories, respectively.

Table  F.1.1: Conditions for Experiment P800-1

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool [18] and impulse responses provided by MC. Cat. 5-6: Pre-produced content |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to SWB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |
| Languages | TBD |

Table  F.1.2: Preliminaries for Experiment P800-1

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table  F.1.3: Test conditions for Experiment P800-1,
stereo speech and mixed music under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.1.4: Clean and noisy speech categories and scene definitions

| Category | Room | Reverb | Microphone Setup | Background | SNR [dB] | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | Low level idle noise | 45 | 1 | Max available up to SWB | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | echoic | A-B (150cm) | Low level idle noise | 45 | -1 | max available up to SWB | 5-11 1-6 3-7 5-8 9-7 10-9 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | echoic | Binaural | office | 15 | 1 | max available up to SWB | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | car | car | A-B Cardioid pair20 cm | car | 15 | -1 | Max available up to SWB | Driver-Passenger BackRight-Driver Driver-BackCenter BackLeft-Driver BackRight-BackLeft BackCenter-BackRight | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.1.5: Mixed content and music categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | music |

Experiment P800-2: Stereo

Tables  F.2.1 to  F.2.5 show conditions to be used for this experiment, list of preliminaries ,full list of conditions, and definition of Speech categories, and Mixed content and Music categories, respectively.

Table   F2.1 : Conditions for Experiment P800-2

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX on |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 5% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool [18] and impulse responses provided by MC. Cat. 5-6: Pre-produced content |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to SWB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |
| Languages | TBD |

Table  F.2.2 : Preliminaries for Experiment P800-2

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile | DTX |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | 5% | on |
| 2 | c26 |  | IVAS FL | 96 | 5% | on |
| 3 | c06 |  | ESDRU α = xx | - | - |  |
| 4 | c20 |  | IVAS FL | 16.4 | 5% | on |
| 5 | c09 |  | ESDRU α = xx | - | - |  |
| 6 | c22 |  | IVAS FL | 32 | 5% | on |
| 7 | c03 |  | MNRU Q=xx dB | - | - |  |
| 8 | c01 |  | Reference | - | - |  |
| 9 | c27 |  | IVAS FL | 128 | 5% | on |
| 10 | c07 |  | ESDRU α = xx | - | - |  |
| 11 | c05 |  | MNRU Q=xx dB | - | - |  |
| 12 | c24 |  | IVAS FL | 64 | 5% | on |

Table  F.2.3: Test conditions for Experiment P800-2,
stereo speech and mixed music under clean impaired channel conditions

| Label | Condition | Bitrate [kbps] | DTX | FER |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=xx dB | - | - |  |
| c03 | MNRU Q=xx dB | - | - |  |
| c04 | MNRU Q=xx dB | - | - |  |
| c05 | MNRU Q=xx dB | - | - |  |
| c06 | ESDRU α=xx | - | - |  |
| c07 | ESDRU α=xx | - | - |  |
| c08 | ESDRU α=xx |  |  |  |
| C09 | ESDRU α=xx | - | - |  |
| c10 | IVAS FL enc / FX dec | 13.2 | on | 5% |
| c11 | IVAS FX enc / FL dec | 16.4 | on | 5% |
| c12 | IVAS FL enc / FX dec | 24.4 | on | 5% |
| c13 | IVAS FX enc / FL dec | 32.0 | on | 5% |
| c14 | IVAS FL enc / FX dec | 48.0 | on | 5% |
| c15 | IVAS FX enc / FL dec | 64.0 | on | 5% |
| c16 | IVAS FL enc / FX dec | 80.0 | on | 5% |
| c17 | IVAS FX enc / FL dec | 96.0 | on | 5% |
| c18 | IVAS FL enc / FX dec | 128.0 | on | 5% |
| c19 | IVAS FL | 13.2 | on | 5% |
| c20 | IVAS FL | 16.4 | on | 5% |
| c21 | IVAS FL | 24.4 | on | 5% |
| c22 | IVAS FL | 32.0 | on | 5% |
| c23 | IVAS FL | 48.0 | on | 5% |
| c24 | IVAS FL | 64.0 | on | 5% |
| c25 | IVAS FL | 80.0 | on | 5% |
| c26 | IVAS FL | 96.0 | on | 5% |
| c27 | IVAS FL | 128.0 | on | 5% |
| c28 | IVAS FX | 13.2 | on | 5% |
| c29 | IVAS FX | 16.4 | on | 5% |
| c30 | IVAS FX | 24.4 | on | 5% |
| c31 | IVAS FX | 32.0 | on | 5% |
| c32 | IVAS FX | 48.0 | on | 5% |
| c33 | IVAS FX | 64.0 | on | 5% |
| c34 | IVAS FX | 80.0 | on | 5% |
| c35 | IVAS FX | 96.0 | on | 5% |
| c36 | IVAS FX | 128.0 | on | 5% |

Table  F.2.4: Clean and noisy speech categories and scene definitions

| Category | Room | Reverb | Microphone Setup | Background | SNR [dB] | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | Low level idle noise | 45 | 1 | Max available up to SWB | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | echoic | A-B (150cm) | Low level idle noise | 45 | -1 | max available up to SWB | 5-11 1-6 3-7 5-8 9-7 10-9 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | echoic | Binaural | office | 15 | 1 | max available up to SWB | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | car | car | A-B Cardioid pair20 cm | car | 15 | -1 | Max available up to SWB | Driver-Passenger BackRight-Driver Driver-BackCenter BackLeft-Driver BackRight-BackLeft BackCenter-BackRight | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.2.5: Mixed content and music categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | music |

Experiment P800-3: FOA

Tables F.3.1 to F.3.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Generic audio categories, respectively.

Table F.3.1: Conditions for Experiment P800-3

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences convolved with (FOA) Spatial Room Impulse Responses respective to various talker positions relative to a capture point and spatial (FOA) ambient noise mixing. Cat. 5-6: Pre-produced content |
| Binaural renderer | FOA to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.54.5 |
| Listening Environment | No room noise |

Table F.3.2: Preliminaries for Experiment P800-2

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.3.3: Test conditions for Experiment P800-3,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.3.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_FOA | room_1_cleanbg_FOA | 45 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_FOA | room_[1/4]_cleanbg_FOA | 45 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.3.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | Generic audio |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-4: HOA2

Tables F.4.1 to F.4.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Generic audio categories, respectively.

Table F.4.1: Conditions for Experiment P800-4

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX on |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 5% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences convolved with (HOA2) Spatial Room Impulse Responses respective to various talker positions relative to a capture point and spatial (HOA2) ambient noise mixing. Cat. 5-6: Pre-produced content |
| Binaural renderer | HOA2 to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.54.5 |
| Listening Environment | No room noise |

Table F.4.2: Preliminaries for Experiment P800-4

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile | DTX |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | 5% | on |
| 2 | c26 |  | IVAS FL | 96 | 5% | on |
| 3 | c06 |  | ESDRU α = xx | - | - |  |
| 4 | c20 |  | IVAS FL | 16.4 | 5% | on |
| 5 | c09 |  | ESDRU α = xx | - | - |  |
| 6 | c22 |  | IVAS FL | 32 | 5% | on |
| 7 | c03 |  | MNRU Q=xx dB | - | - |  |
| 8 | c01 |  | Reference | - | - |  |
| 9 | c27 |  | IVAS FL | 128 | 5% | on |
| 10 | c07 |  | ESDRU α = xx | - | - |  |
| 11 | c05 |  | MNRU Q=xx dB | - | - |  |
| 12 | c24 |  | IVAS FL | 64 | 5% | on |

Table F.4.3: Test conditions for Experiment P800-4,
under impaired channel conditions

| Label | Condition | Bitrate [kbps] | DTX | FER |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=xx dB | - | - |  |
| c03 | MNRU Q=xx dB | - | - |  |
| c04 | MNRU Q=xx dB | - | - |  |
| c05 | MNRU Q=xx dB | - | - |  |
| c06 | ESDRU α=xx | - | - |  |
| c07 | ESDRU α=xx | - | - |  |
| c08 | ESDRU α=xx |  |  |  |
| C09 | ESDRU α=xx | - | - |  |
| c10 | IVAS FL enc / FX dec | 13.2 | on | 5% |
| c11 | IVAS FX enc / FL dec | 16.4 | on | 5% |
| c12 | IVAS FL enc / FX dec | 24.4 | on | 5% |
| c13 | IVAS FX enc / FL dec | 32.0 | on | 5% |
| c14 | IVAS FL enc / FX dec | 48.0 | on | 5% |
| c15 | IVAS FX enc / FL dec | 64.0 | on | 5% |
| c16 | IVAS FL enc / FX dec | 80.0 | on | 5% |
| c17 | IVAS FX enc / FL dec | 96.0 | on | 5% |
| c18 | IVAS FL enc / FX dec | 128.0 | on | 5% |
| c19 | IVAS FL | 13.2 | on | 5% |
| c20 | IVAS FL | 16.4 | on | 5% |
| c21 | IVAS FL | 24.4 | on | 5% |
| c22 | IVAS FL | 32.0 | on | 5% |
| c23 | IVAS FL | 48.0 | on | 5% |
| c24 | IVAS FL | 64.0 | on | 5% |
| c25 | IVAS FL | 80.0 | on | 5% |
| c26 | IVAS FL | 96.0 | on | 5% |
| c27 | IVAS FL | 128.0 | on | 5% |
| c28 | IVAS FX | 13.2 | on | 5% |
| c29 | IVAS FX | 16.4 | on | 5% |
| c30 | IVAS FX | 24.4 | on | 5% |
| c31 | IVAS FX | 32.0 | on | 5% |
| c32 | IVAS FX | 48.0 | on | 5% |
| c33 | IVAS FX | 64.0 | on | 5% |
| c34 | IVAS FX | 80.0 | on | 5% |
| c35 | IVAS FX | 96.0 | on | 5% |
| c36 | IVAS FX | 128.0 | on | 5% |

Table  4.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_HOA2 | room_1_cleanbg_HOA2 | 45 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_HOA2 | room_[1/4]_cleanbg_HOA2 | 45 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_HOA2 | [park_1_bg_HOA2 / nature_1_bg_HOA2 / event_1_bg_HOA2 / street_[1/2]_bg_HOA2] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_HOA2 | [cafeteria_1_bg_HOA2 / mall_1_bg_HOA2/ office[1/2]_bg_HOA2] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  4.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | Generic audio |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Backround name ‘clean_bg_[X]_HOA2’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-5: HOA3

Tables F.5.1 to F.5.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Generic audio categories, respectively.

Table F.5.1: Conditions for Experiment P800-5

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences convolved with (HOA3) Spatial Room Impulse Responses respective to various talker positions relative to a capture point and spatial (HOA3) ambient noise mixing. Cat. 5-6: Pre-produced content |
| Binaural renderer | HOA3 to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.54.5 |
| Listening Environment | No room noise |

Table F.5.2: Preliminaries for Experiment P800-5

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.5.3: Test conditions for Experiment P800-5,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.5.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_HOA3 | room_1_cleanbg_HOA3 | 45 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_HOA3 | room_[1/4]_cleanbg_HOA3 | 45 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_HOA3 | [park_1_bg_HOA3 / nature_1_bg_HOA3 / event_1_bg_HOA3 / street_[1/2]_bg_HOA3] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_HOA3 | [cafeteria_1_bg_HOA3 / mall_1_bg_HOA3/ office[1/2]_bg_HOA3] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.5.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | Generic audio |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Backround name ‘clean_bg_[X]_HOA3’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-6: MC 5-1, 7-1

Tables F.6.1 to F.6.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Music categories, respectively.

Table F.6.1: Conditions for Experiment P800-6

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences convolved with (FOA) Spatial Room Impulse Responses respective to various talker positions relative to a capture point and spatial (FOA) ambient noise mixing, converted to 5.1 using IVAS Pre-renderer. Cat. 5-6: Pre-produced native 5.1 content |
| Binaural renderer | MC to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Generic audio content as described in clause 4.4.4 |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.6.2: Preliminaries for Experiment P800-6

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.6.3: Test conditions for Experiment P800-6,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.6.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_FOA | room_1_cleanbg_FOA | 45 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_FOA | room_[1/4]_cleanbg_FOA | 45 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.6.5: Mixed content and Music categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | music |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Background name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-7: MC 5-1-4, 7-1-4

Tables F.7.1 to F.7.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Music categories, respectively.

Table F.7.1: Conditions for Experiment P800-7

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences convolved with (FOA) Spatial Room Impulse Responses respective to various talker positions relative to a capture point and spatial (FOA) ambient noise mixing, converted to 5.1 using IVAS Pre-renderer. Cat. 5-6: Pre-produced native 5.1 content |
| Binaural renderer | MC to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Generic audio content as described in clause 4.4.4 |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.7.2: Preliminaries for Experiment P800-7

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.7.3: Test conditions for Experiment P800-7,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.7.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_FOA | room_1_cleanbg_FOA | 45 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_FOA | room_[1/4]_cleanbg_FOA | 45 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.7.5: Mixed content and Music categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | music |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Background name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-8: MC - Mixed CICP

Tables F.8.1 to F.8.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Music categories, respectively.

Table F.8.1: Conditions for Experiment P800-8

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 5% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences convolved with (FOA) Spatial Room Impulse Responses respective to various talker positions relative to a capture point and spatial (FOA) ambient noise mixing, converted to 5.1 using IVAS Pre-renderer. Cat. 5-6: Pre-produced native 5.1 content |
| Binaural renderer | MC to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Generic audio content as described in clause 4.4.4 |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.8.2: Preliminaries for Experiment P800-8

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | 5% |
| 2 | c26 |  | IVAS FL | 96 | 5% |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | 5% |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | 5% |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | 5% |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | 5% |

Table F.8.3: Test conditions for Experiment P800-8,
under impaired channel conditions

| Label | Condition | Bitrate [kbps] | FER |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 | 5% |
| c11 | IVAS FX enc / FL dec | 16.4 | 5% |
| c12 | IVAS FL enc / FX dec | 24.4 | 5% |
| c13 | IVAS FX enc / FL dec | 32.0 | 5% |
| c14 | IVAS FL enc / FX dec | 48.0 | 5% |
| c15 | IVAS FX enc / FL dec | 64.0 | 5% |
| c16 | IVAS FL enc / FX dec | 80.0 | 5% |
| c17 | IVAS FX enc / FL dec | 96.0 | 5% |
| c18 | IVAS FL enc / FX dec | 128.0 | 5% |
| c19 | IVAS FL | 13.2 | 5% |
| c20 | IVAS FL | 16.4 | 5% |
| c21 | IVAS FL | 24.4 | 5% |
| c22 | IVAS FL | 32.0 | 5% |
| c23 | IVAS FL | 48.0 | 5% |
| c24 | IVAS FL | 64.0 | 5% |
| c25 | IVAS FL | 80.0 | 5% |
| c26 | IVAS FL | 96.0 | 5% |
| c27 | IVAS FL | 128.0 | 5% |
| c28 | IVAS FX | 13.2 | 5% |
| c29 | IVAS FX | 16.4 | 5% |
| c30 | IVAS FX | 24.4 | 5% |
| c31 | IVAS FX | 32.0 | 5% |
| c32 | IVAS FX | 48.0 | 5% |
| c33 | IVAS FX | 64.0 | 5% |
| c34 | IVAS FX | 80.0 | 5% |
| c35 | IVAS FX | 96.0 | 5% |
| c36 | IVAS FX | 128.0 | 5% |

Table  F.8.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_FOA | room_1_cleanbg_FOA | 45 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_FOA | room_[1/4]_cleanbg_FOA | 45 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.8.5: Mixed content and Music categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | music |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Background name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-9: 1-2 Objects

Tables F.9.1 to F.9.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Speech with effects and Music categories, respectively.

Table F.9.1: Conditions for Experiment P800-9

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: | Cat. 1-3: Defined scenes, 1 ISM Cat. 3-6: Defined scenes, 2 ISMs |
| Binaural renderer | ISM to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.9.2: Preliminaries for Experiment P800-9

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.9.3: Test conditions for Experiment P800-9,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Scene definitions categories 1-2

A leading and trailing silence is present for each sample, in accordance with IVAS-7a. The metadata corresponds to the whole duration of the samples. This means that for moving objects, only a part of the trajectory corresponds to active speech. The following scenes are used:

Talker sitting at a table (elevation 0°), at different azimuths.

Standing talker (elevation 35°), at different azimuths.

Smaller talker (child) walking around a table in the positive sense (counterclockwise), elevation 0°. Azimuth varies continuously for the sentence pair.

Adult talker walking around a table in the negative sense (clockwise), elevation 35°. Azimuth varies continuously for the sentence pair.

Elevation displacement: Elevation varies continuously for the sentence pair . Azimuth is constant for a sentence pair, but different for each sentence pair.

Azimuth and elevation displacement: Azimuth and elevation vary continuously.

Each of the sentences uttered by a certain talker is encoded using different scene. To balance the test, in addition to listeners of each panel listening to all talkers (Categories), all scenes are also covered in each panel. Allocation of scenes to each panel is given in the Table E 4.4

Scene definitions categories 3-4

The listening database consists of artificially created spatial audio samples from monophonic clean speech recordings where always 1 female and 1 male talker are combined in conversation-like scenarios following the Scene descriptions below.

A leading and trailing silence is present for each artificially created spatial audio sample, in accordance with IVAS-7a. The metadata corresponds to the whole duration of the sample. This means that for moving objects, only a part of the trajectory corresponds to active speech.

In one half of the samples, the 2nd talker’s utterance follows the 1st talker’s utterance simulating natural conversation. The gap between the utterances is set to 1 s. In the other half of the samples, the situation is similar, but the utterances partially overlap. The targeted overlap is also 1 s. Non-overlapping sentence pairs are used for Scenes a., c., and e. as described below. Overlapping sentence pairs are used for Scenes b., d., and f. The following scenes are used:

- Two talkers sitting at a table (elevation 0°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair.

Two standing talkers (elevation 35°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair.

One talker sitting at a table (elevation 0°), second talker standing beside the table (elevation 45°). Non-overlapping utterances.

One talker sitting at a table (elevation 0°), second talker walking around the table (elevation 45°). The azimuth of the 2nd talker varies continually, positive sense is counter clockwise.

Two talkers walking side-by-side around the table (elevation 45°). The azimuth is the same for both talkers and varies continually.

Two talkers walking around the table in opposite directions (elevation 30°), starting at the same position. Azimuths of both talkers vary continually.

The following table lists the test Categories corresponding to different talker pairs. Each of the sentence pairs uttered by a certain talker pair is associated to different scenes. To balance the test, listeners of each panel listen to all talker pairs, and all scenes are covered in each panel.

Table F.9.4: Allocation of scenes for each talker pair (category cat 1 – cat 4) and listening panel (P1-P6)

| Category | Talker initial elevation | Elevation change(1 | Talker initial azimuth | Azimuth change(2 | Panel |
| --- | --- | --- | --- | --- | --- |
| cat 1: M1 | 0° 35° 0° 35° -90° 35° | static static static static 0.3°/ frame -0.2°/ frame | 0° 180° 120° 180° 120° 0° | static static 1°/ frame -1°/ frame static 0.5°/ frame | P1 P2 P3 P4 P5 P6 |
| cat 2: F1 | 35° 0° 35° -90° 35° 0° | static static static 0.3°/ frame -0.2°/ frame static | 120° 60° 120° 60° 300° 300° | static 1°/ frame -1°/ frame static 0.5°/ frame static | P1 P2 P3 P4 P5 P6 |
| cat 3? |  |  |  |  |  |

| Category | Overtalk [s] (1 | 1st talker elevation | 2nd talker elevation | 1st talker initial azimuth | 1st talker azimuth change(2 | 2nd talker initial azimuth | 2nd talker azimuth change(2 | Panel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 3: M2 + F2 | 1 -1 1 -1 1 -1 | 35° 0° 0° 45° 30° 0° | 35° 45° 45° 45° 30° 0° | 20° 30° 250° 290° 180° 10° | static static static -1°/ frame 1°/ frame static | 170° 230° 340° 290° 180° 110° | static static -1°/ frame -1°/ frame -1°/ frame static | P1 P2 P3 P4 P5 P6 |
| cat 4: M3 + F3 | -1 1 -1 1 -1 1 | 0° 0° 45° 30° 0° 35° | 45° 45° 45° 30° 0° 35° | 40° 300° 180° 240° 20° 30° | static static 1°/ frame 1°/ frame static static | 290° 290° 180° 240° 170° 230° | static -1°/ frame 1°/ frame 1°/ frame static static | P1 P2 P3 P4 P5 P6 |
| cat 5: ? |  |  |  |  |  |  |  |  |

Notes:

(1 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(2 The positive sense for azimuth is counterclockwise

Experiment P800-10: 3-4 Objects

Tables F.10.1 to F.10.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Speech with effects and Music categories, respectively.

Table F.10.1: Conditions for Experiment P800-10

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: | Cat. 1-3: Pre-produced content, 3 ISMs Cat. 4-6: Pre-produced content, 4 ISMs |
| Binaural renderer | ISM to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.10.2: Preliminaries for Experiment P800-10

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.10.3: Test conditions for Experiment P800-10,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.10.4: Categories for Speech with effects, and Speech with music or Music

| Category | Type |
| --- | --- |
| cat 1 | speech + effects 3 objects |
| cat 2 | speech + music 3 objects |
| cat 3 | music 3 objects |
| cat 4 | speech + effects 4 objects |
| cat 5 | speech + music 4 objects |
| cat 6 | music 4 objects |

Experiment P800-11: 1-4 Objects

Tables F.11.1 to F.11.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Speech with effects and Music categories, respectively.

Table F.11.1: Conditions for Experiment P800-11

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX on |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 5% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: | Cat. 1-2: Defined scenes, 1 ISM Cat. 3-4: Defined scenes, 2 ISMs Cat. 5-6: Pre-produced content, 3-4 ISMs |
| Binaural renderer | ISM to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.11.2: Preliminaries for Experiment P800-11

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile | DTX |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | 5% | on |
| 2 | c26 |  | IVAS FL | 96 | 5% | on |
| 3 | c06 |  | ESDRU α = xx | - | - |  |
| 4 | c20 |  | IVAS FL | 16.4 | 5% | on |
| 5 | c09 |  | ESDRU α = xx | - | - |  |
| 6 | c22 |  | IVAS FL | 32 | 5% | on |
| 7 | c03 |  | MNRU Q=xx dB | - | - |  |
| 8 | c01 |  | Reference | - | - |  |
| 9 | c27 |  | IVAS FL | 128 | 5% | on |
| 10 | c07 |  | ESDRU α = xx | - | - |  |
| 11 | c05 |  | MNRU Q=xx dB | - | - |  |
| 12 | c24 |  | IVAS FL | 64 | 5% | on |

Table F.11.3: Test conditions for Experiment P800-11,
 under impaired channel conditions

| Label | Condition | Bitrate [kbps] | DTX | FER |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=xx dB | - | - |  |
| c03 | MNRU Q=xx dB | - | - |  |
| c04 | MNRU Q=xx dB | - | - |  |
| c05 | MNRU Q=xx dB | - | - |  |
| c06 | ESDRU α=xx | - | - |  |
| c07 | ESDRU α=xx | - | - |  |
| c08 | ESDRU α=xx |  |  |  |
| C09 | ESDRU α=xx | - | - |  |
| c10 | IVAS FL enc / FX dec | 13.2 | on | 5% |
| c11 | IVAS FX enc / FL dec | 16.4 | on | 5% |
| c12 | IVAS FL enc / FX dec | 24.4 | on | 5% |
| c13 | IVAS FX enc / FL dec | 32.0 | on | 5% |
| c14 | IVAS FL enc / FX dec | 48.0 | on | 5% |
| c15 | IVAS FX enc / FL dec | 64.0 | on | 5% |
| c16 | IVAS FL enc / FX dec | 80.0 | on | 5% |
| c17 | IVAS FX enc / FL dec | 96.0 | on | 5% |
| c18 | IVAS FL enc / FX dec | 128.0 | on | 5% |
| c19 | IVAS FL | 13.2 | on | 5% |
| c20 | IVAS FL | 16.4 | on | 5% |
| c21 | IVAS FL | 24.4 | on | 5% |
| c22 | IVAS FL | 32.0 | on | 5% |
| c23 | IVAS FL | 48.0 | on | 5% |
| c24 | IVAS FL | 64.0 | on | 5% |
| c25 | IVAS FL | 80.0 | on | 5% |
| c26 | IVAS FL | 96.0 | on | 5% |
| c27 | IVAS FL | 128.0 | on | 5% |
| c28 | IVAS FX | 13.2 | on | 5% |
| c29 | IVAS FX | 16.4 | on | 5% |
| c30 | IVAS FX | 24.4 | on | 5% |
| c31 | IVAS FX | 32.0 | on | 5% |
| c32 | IVAS FX | 48.0 | on | 5% |
| c33 | IVAS FX | 64.0 | on | 5% |
| c34 | IVAS FX | 80.0 | on | 5% |
| c35 | IVAS FX | 96.0 | on | 5% |
| c36 | IVAS FX | 128.0 | on | 5% |

Scene definitions categories 1-2

A leading and trailing silence is present for each sample, in accordance with IVAS-7a. The metadata corresponds to the whole duration of the samples. This means that for moving objects, only a part of the trajectory corresponds to active speech. The following scenes are used:

Talker sitting at a table (elevation 0°), at different azimuths.

Standing talker (elevation 35°), at different azimuths.

Smaller talker (child) walking around a table in the positive sense (counterclockwise), elevation 0°. Azimuth varies continuously for the sentence pair.

Adult talker walking around a table in the negative sense (clockwise), elevation 35°. Azimuth varies continuously for the sentence pair.

Elevation displacement: Elevation varies continuously for the sentence pair . Azimuth is constant for a sentence pair, but different for each sentence pair.

Azimuth and elevation displacement: Azimuth and elevation vary continuously.

Each of the sentences uttered by a certain talker is encoded using different scene. To balance the test, in addition to listeners of each panel listening to all talkers (Categories), all scenes are also covered in each panel. Allocation of scenes to each panel is given in the Table E 4.4

Scene definitions categories 3-4

The listening database consists of artificially created spatial audio samples from monophonic clean speech recordings where always 1 female and 1 male talker are combined in conversation-like scenarios following the Scene descriptions below.

A leading and trailing silence is present for each artificially created spatial audio sample, in accordance with IVAS-7a. The metadata corresponds to the whole duration of the sample. This means that for moving objects, only a part of the trajectory corresponds to active speech.

In one half of the samples, the 2nd talker’s utterance follows the 1st talker’s utterance simulating natural conversation. The gap between the utterances is set to 1 s. In the other half of the samples, the situation is similar, but the utterances partially overlap. The targeted overlap is also 1 s. Non-overlapping sentence pairs are used for Scenes a., c., and e. as described below. Overlapping sentence pairs are used for Scenes b., d., and f. The following scenes are used:

- Two talkers sitting at a table (elevation 0°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair.

Two standing talkers (elevation 35°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair.

One talker sitting at a table (elevation 0°), second talker standing beside the table (elevation 45°). Non-overlapping utterances.

One talker sitting at a table (elevation 0°), second talker walking around the table (elevation 45°). The azimuth of the 2nd talker varies continually, positive sense is counter clockwise.

Two talkers walking side-by-side around the table (elevation 45°). The azimuth is the same for both talkers and varies continually.

Two talkers walking around the table in opposite directions (elevation 30°), starting at the same position. Azimuths of both talkers vary continually.

The following table lists the test Categories corresponding to different talker pairs. Each of the sentence pairs uttered by a certain talker pair is associated to different scenes. To balance the test, listeners of each panel listen to all talker pairs, and all scenes are covered in each panel.

Table F.11.4: Allocation of scenes for each talker pair (category cat 1 – cat 4) and listening panel (P1-P6)

| Category | Talker initial elevation | Elevation change(1 | Talker initial azimuth | Azimuth change(2 | Panel |
| --- | --- | --- | --- | --- | --- |
| cat 1: M1 | 0° 35° 0° 35° -90° 35° | static static static static 0.3°/ frame -0.2°/ frame | 0° 180° 120° 180° 120° 0° | static static 1°/ frame -1°/ frame static 0.5°/ frame | P1 P2 P3 P4 P5 P6 |
| cat 2: F1 | 35° 0° 35° -90° 35° 0° | static static static 0.3°/ frame -0.2°/ frame static | 120° 60° 120° 60° 300° 300° | static 1°/ frame -1°/ frame static 0.5°/ frame static | P1 P2 P3 P4 P5 P6 |

| Category | Overtalk [s] (1 | 1st talker elevation | 2nd talker elevation | 1st talker initial azimuth | 1st talker azimuth change(2 | 2nd talker initial azimuth | 2nd talker azimuth change(2 | Panel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 3: M2 + F2 | 1 -1 1 -1 1 -1 | 35° 0° 0° 45° 30° 0° | 35° 45° 45° 45° 30° 0° | 20° 30° 250° 290° 180° 10° | static static static -1°/ frame 1°/ frame static | 170° 230° 340° 290° 180° 110° | static static -1°/ frame -1°/ frame -1°/ frame static | P1 P2 P3 P4 P5 P6 |
| cat 4: M3 + F3 | -1 1 -1 1 -1 1 | 0° 0° 45° 30° 0° 35° | 45° 45° 45° 30° 0° 35° | 40° 300° 180° 240° 20° 30° | static static 1°/ frame 1°/ frame static static | 290° 290° 180° 240° 170° 230° | static -1°/ frame 1°/ frame 1°/ frame static static | P1 P2 P3 P4 P5 P6 |

Table  F.11.5: Categories for Speech with effects, and Speech with music or Music

| Category | Type |
| --- | --- |
| cat 5 | Speech + effects (3-4 objects) |
| cat 6 | Speech + music or music only (3-4 objects) |

Notes:

(1 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(2 The positive sense for azimuth is counterclockwise

Experiment P800-12: MASA 1 TC

Tables F.12.1 to F.12.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Generic audio categories, respectively.

Table F.12.1: Conditions for Experiment P800-12

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Model-based generation according to convolution of raw mono clean speech sentences with FOA Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA background. MASA format generation from FOA according to MASA analysis [19]. |
| Binaural renderer | IVAS MASA internal binaural rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.12.12: Preliminaries for Experiment P800-12

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.12.3: Test conditions for Experiment P800-5,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.12.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_MASA | room_1_cleanbg_MASA | 45 | 1 | Max | tbd | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_MASA | room_4_cleanbg_MASA | 45 | -1 | Max | tbd | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[1/2]_MASA | [park_1_bg_MASA / nature_1_bg_MASA / event_1_bg_MASA / street_[1/2]_bg_MASA] | 15 | -1 | Max | tbd | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_MASA | [cafeteria_1_bg_MASA / mall_1_bg_MASA/ office[1/2]_bg_MASA] | 15 | -1 | Max | tbd | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.12.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | Generic audio |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-13: MASA 2 TC

Tables F.13.1 to F.13.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Generic audio categories, respectively.

Table F.13.1: Conditions for Experiment P800-13

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Model-based generation according to convolution of raw mono clean speech sentences with FOA Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA background. MASA format generation from FOA according to MASA analysis [19]. |
| Binaural renderer | IVAS MASA internal binaural rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.13.2: Preliminaries for Experiment P800-13

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.13.3: Test conditions for Experiment P800-13,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.13.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_MASA | room_1_cleanbg_MASA | 45 | 1 | Max | tbd | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_MASA | room_4_cleanbg_MASA | 45 | -1 | Max | tbd | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[1/2]_MASA | [park_1_bg_MASA / nature_1_bg_MASA / event_1_bg_MASA / street_[1/2]_bg_MASA] | 15 | -1 | Max | tbd | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_MASA | [cafeteria_1_bg_MASA / mall_1_bg_MASA/ office[1/2]_bg_MASA] | 15 | -1 | Max | tbd | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.13.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | Generic audio |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-14: MASA 1-2 TC

Tables F.14.1 to F.14.5 show conditions to be used for this experiment, list of preliminaries, full list of conditions, and definition of Speech categories, and Mixed content and Generic audio categories, respectively.

Table F.14.1: Conditions for Experiment P800-14

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX on |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 5% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Model-based generation according to convolution of raw mono clean speech sentences with FOA Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA background. MASA format generation from FOA according to MASA analysis [19]. |
| Binaural renderer | IVAS MASA internal binaural rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table F.14.2: Preliminaries for Experiment P800-14

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile | DTX |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | 5% | on |
| 2 | c26 |  | IVAS FL | 96 | 5% | on |
| 3 | c06 |  | ESDRU α = xx | - | - |  |
| 4 | c20 |  | IVAS FL | 16.4 | 5% | on |
| 5 | c09 |  | ESDRU α = xx | - | - |  |
| 6 | c22 |  | IVAS FL | 32 | 5% | on |
| 7 | c03 |  | MNRU Q=xx dB | - | - |  |
| 8 | c01 |  | Reference | - | - |  |
| 9 | c27 |  | IVAS FL | 128 | 5% | on |
| 10 | c07 |  | ESDRU α = xx | - | - |  |
| 11 | c05 |  | MNRU Q=xx dB | - | - |  |
| 12 | c24 |  | IVAS FL | 64 | 5% | on |

Table F.14.3: Test conditions for Experiment P800-14,
under impaired channel conditions

| Label | Condition | Bitrate [kbps] | DTX | FER |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=xx dB | - | - |  |
| c03 | MNRU Q=xx dB | - | - |  |
| c04 | MNRU Q=xx dB | - | - |  |
| c05 | MNRU Q=xx dB | - | - |  |
| c06 | ESDRU α=xx | - | - |  |
| c07 | ESDRU α=xx | - | - |  |
| c08 | ESDRU α=xx |  |  |  |
| C09 | ESDRU α=xx | - | - |  |
| c10 | IVAS FL enc / FX dec | 13.2 | on | 5% |
| c11 | IVAS FX enc / FL dec | 16.4 | on | 5% |
| c12 | IVAS FL enc / FX dec | 24.4 | on | 5% |
| c13 | IVAS FX enc / FL dec | 32.0 | on | 5% |
| c14 | IVAS FL enc / FX dec | 48.0 | on | 5% |
| c15 | IVAS FX enc / FL dec | 64.0 | on | 5% |
| c16 | IVAS FL enc / FX dec | 80.0 | on | 5% |
| c17 | IVAS FX enc / FL dec | 96.0 | on | 5% |
| c18 | IVAS FL enc / FX dec | 128.0 | on | 5% |
| c19 | IVAS FL | 13.2 | on | 5% |
| c20 | IVAS FL | 16.4 | on | 5% |
| c21 | IVAS FL | 24.4 | on | 5% |
| c22 | IVAS FL | 32.0 | on | 5% |
| c23 | IVAS FL | 48.0 | on | 5% |
| c24 | IVAS FL | 64.0 | on | 5% |
| c25 | IVAS FL | 80.0 | on | 5% |
| c26 | IVAS FL | 96.0 | on | 5% |
| c27 | IVAS FL | 128.0 | on | 5% |
| c28 | IVAS FX | 13.2 | on | 5% |
| c29 | IVAS FX | 16.4 | on | 5% |
| c30 | IVAS FX | 24.4 | on | 5% |
| c31 | IVAS FX | 32.0 | on | 5% |
| c32 | IVAS FX | 48.0 | on | 5% |
| c33 | IVAS FX | 64.0 | on | 5% |
| c34 | IVAS FX | 80.0 | on | 5% |
| c35 | IVAS FX | 96.0 | on | 5% |
| c36 | IVAS FX | 128.0 | on | 5% |

Table  14.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_MASA | room_1_cleanbg_MASA | 45 | 1 | Max | tbd | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_MASA | room_4_cleanbg_MASA | 45 | -1 | Max | tbd | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[1/2]_MASA | [park_1_bg_MASA / nature_1_bg_MASA / event_1_bg_MASA / street_[1/2]_bg_MASA] | 15 | -1 | Max | tbd | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_MASA | [cafeteria_1_bg_MASA / mall_1_bg_MASA/ office[1/2]_bg_MASA] | 15 | -1 | Max | tbd | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.14.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | Generic audio |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-15: OSBA (1-2 objects)

Tables F151 toF15.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively. Tables  F15.4 to F15.5 show definition of categories and scenes.

Table F.15.1: Conditions for Experiment P800-15

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | 15 dB for cat 1,2,3,4, tbd for cat 5,6 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-3: Defined scenes, 1 ISM + FOA [or/and HOA3 if available] background Cat. 3-6: Defined scenes, 2 ISMs + FOA [or/and HOA3 if available] background |
| Binaural renderer | OSBA to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.54.5 |
| Listening Environment | No room noise |

Table F.15.2: Preliminaries for Experiment P800-15

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.15.3: Test conditions for Experiment P800-15,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.15.4: Clean and noisy speech categories and scene definitions for SBA

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 |  |  |  |  |  |  |  |
| cat 4 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 5 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 6 |  |  |  |  |  |  |  |

Note: ISM positions for categories cat 1-4 are as defined respectively in Table F.9.4.

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-16: OSBA (3-4 objects)

Tables F.16.1 to F.16.4 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively. Tables  F.16.4 to F.16.5 show definition of categories and scenes.

Table F.16.1: Conditions for Experiment P800-16

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | 15 dB for cat 1,2,3,4, tbd for cat 5,6 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-3: Pre-produced content, 3 ISMs + FOA [or/and HOA3 if available] background Cat. 4-6: Pre-produced content, 4 ISMs + FOA [or/and HOA3 if available] background |
| Binaural renderer | OSBA to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.54.5 |
| Listening Environment | No room noise |

Table F.16.2: Preliminaries for Experiment P800-16

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.16.3: Test conditions for Experiment P800-16,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.16.4: Clean and noisy speech categories and scene definitions for SBA

| Category | Type |
| --- | --- |
| cat 1-3 | 3-object + General audio background |
| cat 4-6 | 4-objects + General audio background |

Experiment P800-17: OSBA (1-4 objects)

Table F17.1: Overview of test conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 15 | IVAS OSBA operated at all bitrates 13.2- 512 kbps with DTX off at 0% FER |
|  | 3 | IVAS OSBA operated at 32, 64, 256 kbps with 5% FER |
|  | 4 | IVAS OSBA fixed point / floating point interoperability conditions at 32, 64, 128 and 256 kbps |
| Codec references |  |  |
| Codec references | 6 | IVAS operation in two separate instances (SBA + ISM) |
| Other references |  |  |
| Direct | 1 | Fixed point IVAS_rend. Nominal input level |
| P.50 MNRU (applied to SBA transport streams) | 4 | Q = 34, 30, 26, 22 dB (all: nominal input level) |
| ESDRU [5] | 4 | α = 0.8, 0.6, 0.4, 0.2 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 1 | Model-based generation according to processing scripts. |
| Binaural rendering | 1 | IVAS codec internal binaural renderer and for references IVAS external renderer (IVAS_rend) |
| Audio sampling frequency / bandwidth | 1 | 48 kHz / maximum available audio bandwidth (WB, SWB, FB) |
| Content types / categories | 6 | Scenes as described in Table F.17.2 |
| Number of talkers | 6 | ISM signals are generated so that different objects are uttered by different talkers (one each of 3 male and 3 female talkers). All talkers are tried to be used equal amount. |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([8]) |
| Background signal (SBA) input loudness | 1 | -36 LKFS ([8]) |
| ISM speech input loudness | 1 | -26 LKFS ([8]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with instructions according to [12] |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

Table F.17.2: Sample Categories

| Category | Number of objects | Speech Level [dB] | Background signal type** | Background Level | Overtalk [s] | Talker positions |
| --- | --- | --- | --- | --- | --- | --- |
| cat 1 | 1 | -26 | Indoors 1 | [-36] | No overtalk | 2 fixed, 4 with movement |
| cat 2 | 2 | -26 | Indoors 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 3 | 3 | -26 | Outdoors 1 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 4 | 4 | -26 | Outdoors 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 5 | 2 | -26 | Background with music 1 | [-36] | No overtalk | 2 fixed, 4 with movement* |
| cat 6 | 3 | -26 | Background with music 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |

*for 2 samples one ISM is moving, for the last 2 samples two or more objects are moving. For practice sample one ISM is moving.

** Background type signal is HOA3

Table F.9.3: Test conditions for P.SUPPL800 OSBA, speech and background environments.

| Label | Condition | Bitrate [kbps] | FER/float | Notes |
| --- | --- | --- | --- | --- |
| c01 | Reference IVAS_rend | - | - |  |
| c02 | MNRU Q=34 dB | - | - |  |
| c03 | MNRU Q=30 dB | - | - |  |
| c04 | MNRU Q=26 dB | - | - |  |
| c05 | MNRU Q=22 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.6 | - | - |  |
| c08 | ESDRU α=0.4 | - | - |  |
| c09 | ESDRU α=0.2 | - | - |  |
| c10 | OSBA | 32 | Float |  |
| c11 | OSBA | 64 | Float |  |
| c12 | OSBA | 128 | Float |  |
| c13 | OSBA | 256 | Float |  |
| c14 | OSBA | 32 | 5% FER |  |
| c15 | OSBA | 64 | 5% FER |  |
| c16 | OSBA | 256 | 5% FER |  |
| c17 | OSBA | 13.2 | No errors |  |
| c18 | OSBA | 16.4 | No errors |  |
| c19 | OSBA | 24.4 | No errors |  |
| c20 | OSBA | 32 | No errors |  |
| c21 | OSBA | 80 | No errors |  |
| c22 | OSBA | 160 | No errors |  |
| c23 | OSBA | 384 | No errors |  |
| c24 | OSBA | 512 | No errors |  |
| c25 | OSBA | 48 | No errors | C31 |
| c26 | OSBA | 64 | No errors | C32 |
| c27 | OSBA | 96 | No errors | C33 |
| c28 | OSBA | 128 | No errors | C34 |
| c29 | OSBA | 192 | No errors | C35 |
| c30 | OSBA | 256 | No errors | C36 |
| c31 | ISM + SBA (HOA3) | 24.4 + 24.4 | No errors |  |
| c32 | ISM + SBA (HOA3) | 32 + 32 | No errors |  |
| c33 | ISM + SBA (HOA3) | 48 + 48 | No errors |  |
| c34 | ISM + SBA (HOA3) | 64 + 64 | No errors |  |
| c35 | ISM + SBA (HOA3) | 96 + 96 | No errors |  |
| c36 | ISM + SBA (HOA3) | 128 + 128 | No errors |  |

Experiment P800-18: OMASA (1-2 objects)

Tables F.18.1 to F.18.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively. Tables  F.18.4 to F.18.5 show definition of categories and scenes.

Table F.18.1: Conditions for Experiment P800-18

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | 15 dB for cat 1,2,3,4, tbd for cat 5,6 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-3: Defined scenes, 1 ISM + MASA background Cat. 3-6: Defined scenes, 2 ISMs + MASA background |
| Binaural renderer | OMASA to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.54.5 |
| Listening Environment | No room noise |

Table F.18.2: Preliminaries for Experiment P800-18

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.18.3: Test conditions for Experiment P800-18,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.18.4: Clean and noisy speech categories and scene definitions for SBA

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | out_[X]_MASA | [park_1_bg_MASA / nature_1_bg_MASA / event_1_bg_MASA / street_[1/2]_bg_MASA] | 15 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_[X]_MASA | [cafeteria_1_bg_MASA / mall_1_bg_MASA/ office[1/2]_bg_MASA] | 15 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 |  |  |  |  |  |  |  |
| cat 4 | out_[X]_MASA | [park_1_bg_MASA / nature_1_bg_MASA / event_1_bg_MASA / street_[1/2]_bg_MASA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 5 | room_[X]_MASA | [cafeteria_1_bg_MASA / mall_1_bg_MASA/ office[1/2]_bg_MASA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 6 |  |  |  |  |  |  |  |

Note: ISM positions for categories cat 1-4 are as defined respectively in Table F.9.4.

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8b.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8b.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-19: OMASA (3-4 objects)

Tables F.19.1 to F.19.4 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively. Tables  F.19.4 to F.19.5 show definition of categories and scenes.

Table F.19.1: Conditions for Experiment P800-19

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | 15 dB for cat 1,2,3,4, tbd for cat 5,6 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-3: Pre-produced content, 3 ISMs + MASA background Cat. 4-6: Pre-produced content, 4 ISMs + MASA background |
| Binaural renderer | OMASA to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.3 |
| Listening System | Headphones, in accordance with clause 4.54.5 |
| Listening Environment | No room noise |

Table F.19.2: Preliminaries for Experiment P800-19

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c26 |  | IVAS FL | 96 | No errors |
| 3 | c06 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c09 |  | ESDRU α = xx | - | - |
| 6 | c22 |  | IVAS FL | 32 | No errors |
| 7 | c03 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c27 |  | IVAS FL | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table F.19.3: Test conditions for Experiment P800-19,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table  F.19.4: Clean and noisy speech categories and scene definitions for SBA

| Category | Type |
| --- | --- |
| cat 1-3 | 3-object + General audio background |
| cat 4-6 | 4-objects + General audio background |

Experiment P800-20: OMASA

Table F.20.1: Overview of test conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 15 | IVAS oMASA operated at all bitrates 13.2- 512 kbps with DTX off at 0% FER |
|  | 3 | IVAS oMASA operated at 32, 64, 128 kbps with 5% FER |
|  | 4 | IVAS oMASA fixed point / floating point interoperability conditions at 32, 64, 128 and 256 kbps |
| Codec references |  |  |
| Codec references | 6 | IVAS operation in two separate instances (MASA + ISM) |
| Other references |  |  |
| Direct | 1 | Fixed point IVAS_rend. Nominal input level |
| P.50 MNRU (applied to MASA transport streams) | 4 | Q = 34, 30, 26, 22 dB (all: nominal input level) |
| ESDRU [5] | 4 | α = 0.8, 0.6, 0.4, 0.2 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 1 | Model-based generation according to processing scripts. |
| Binaural rendering | 1 | IVAS codec internal binaural renderer and for references IVAS external renderer (IVAS_rend) |
| Audio sampling frequency / bandwidth | 1 | 48 kHz / maximum available audio bandwidth (WB, SWB, FB) |
| Content types / categories | 6 | Scenes as described in Table2 |
| Number of talkers | 6 | ISM signals are generated so that different objects are uttered by different talkers (one each of 3 male and 3 female talkers). All talkers are tried to be used equal amount. |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([8]) |
| Background signal (MASA) input loudness | 1 | -36 LKFS ([8]) |
| ISM speech input loudness | 1 | -26 LKFS ([8]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with instructions according to [12] |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

Table F.20.2: Sample Categories

| Category | Number of objects | Speech Level [dB] | Background signal type | Background Level | Overtalk [s] | Talker positions |
| --- | --- | --- | --- | --- | --- | --- |
| cat 1 | 1 | -26 | Indoors 1 | [-36] | No overtalk | 2 fixed, 4 with movement |
| cat 2 | 2 | -26 | Indoors 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 3 | 3 | -26 | Outdoors 1 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 4 | 4 | -26 | Outdoors 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 5 | 2 | -26 | Background with music 1 | [-36] | No overtalk | 2 fixed, 4 with movement* |
| cat 6 | 3 | -26 | Background with music 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |

*for 2 samples one ISM is moving, for the last 2 samples two or more objects are moving. For practice sample one ISM is moving.

Table F.20.3: Test conditions for P.SUPPL800 oMASA, speech and background environments.

| Label | Condition | Bitrate [kbps] | FER/float | Notes |
| --- | --- | --- | --- | --- |
| c01 | Reference IVAS_rend | - | - |  |
| c02 | MNRU Q=34 dB | - | - |  |
| c03 | MNRU Q=30 dB | - | - |  |
| c04 | MNRU Q=26 dB | - | - |  |
| c05 | MNRU Q=22 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.6 | - | - |  |
| c08 | ESDRU α=0.4 | - | - |  |
| c09 | ESDRU α=0.2 | - | - |  |
| c10 | OMASA 2TC | 32 | Float |  |
| c11 | OMASA 2TC | 64 | Float |  |
| c12 | OMASA 2TC | 128 | Float |  |
| c13 | OMASA 2TC | 256 | Float |  |
| c14 | OMASA 2TC | 32 | 5% FER |  |
| c15 | OMASA 2TC | 64 | 5% FER |  |
| c16 | OMASA 2TC | 128 | 5% FER |  |
| c17 | OMASA 2TC | 13,2 | No errors |  |
| c18 | OMASA 2TC | 16,4 | No errors |  |
| c19 | OMASA 2TC | 24,4 | No errors |  |
| c20 | OMASA 2TC | 32 | No errors |  |
| c21 | OMASA 2TC | 80 | No errors |  |
| c22 | OMASA 2TC | 160 | No errors |  |
| c23 | OMASA 2TC | 384 | No errors |  |
| c24 | OMASA 2TC | 512 | No errors |  |
| c25 | OMASA 2TC | 48 | No errors | C31 |
| c26 | OMASA 2TC | 64 | No errors | C32 |
| c27 | OMASA 2TC | 96 | No errors | C33 |
| c28 | OMASA 2TC | 128 | No errors | C34 |
| c29 | OMASA 2TC | 192 | No errors | C35 |
| c30 | OMASA 2TC | 256 | No errors | C36 |
| c31 | ISM + MASA | 24,4 + 24,4 | No errors |  |
| c32 | ISM + MASA | 32 + 32 | No errors |  |
| c33 | ISM + MASA | 48 + 48 | No errors |  |
| c34 | ISM + MASA | 64 + 64 | No errors |  |
| c35 | ISM + MASA | 96 + 96 | No errors |  |
| c36 | ISM + MASA | 128 + 128 | No errors |  |

]

|  | α=0.8 |  |  |  |
| --- | --- | --- | --- | --- |
|  | α=0.6 |  |  |  |
|  | α=0.4 |  |  |  |

|  | α=0.8 |  |  |  |
| --- | --- | --- | --- | --- |
|  | α=0.6 |  |  |  |
|  | α=0.4 |  |  |  |

|  | α=0.8 |  |  |  |
| --- | --- | --- | --- | --- |
|  | α=0.6 |  |  |  |
|  | α=0.4 |  |  |  |
|  | α=0.2 |  |  |  |

|  | α=0.8 |  |  |  |
| --- | --- | --- | --- | --- |
|  | α=0.6 |  |  |  |
|  | α=0.4 |  |  |  |
|  | α=0.2 |  |  |  |

Conclusion

In conclusion, it is proposed to incorporate the revised Annex F into the IVAS-8b PDoc [2]. This updated annex is to serve as a basis for further work towards finalizing the test plan.

References

S4aA240018 - Proposal for IVAS Characterization PSUPPL800 Tests Allocation, Fraunhofer IIS

S4aA240021 - IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v.0.2.2