| Joint TSG-S4#9 – SMG11#14 meeting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  | S4-(00)0091R |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| January 24-28 2000, Puerto Vallarta, Mexico |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 26.102 |  |  |  |  | CR |  |  |  | 001 Rev1 |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.0.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submision to TSG |  |  |  |  | SA4#9 |  |  |  | for approval |  |  |  |  |  | X |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  |  | ME |  |  |  |  |  |  |  |  |  | UTRAN |  |  |  |  |  |  |  | Core Network |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 28-01-2000 |  |  |  |  |
| Subject: |  |  | Introduction of determination of QoS parameters used at RAB assignment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | QoS for Speech & Multimedia Codecs / Adaptive Multi-Rate Speech Codec |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | The determination of the QoS parameters, generated by the bearer capability information element received at call set up, that are used at RAB assignment is clarified. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | 2, 5, 6.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

Technical Specification

TS 26.102 V3.0.0 (1999-12)

3rd Generation Partnership Project (3GPP)

TSG-SA Codec Working Group

Mandatory speech codec;

AMR speech codec; Interface to Iu and Uu

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

TSG-SA4-W1 (26102-050.doc)

Keywords

Adaptive Multi-Rate, Mandatory speech codec

3GPP

Postal address

Office address

Internet

secretariat@3gpp.org

Individual copies of this  deliverable
can be downloaded from

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 1999, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA, TTC).

All rights reserved.

# 2	Normative References

This TS incorporates by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this TS only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	3G TS 25.415 : “Iu Interface CN-UTRAN User plane Protocols”

[2]	3G TS 26.101 : “AMR Speech Codec, Frame structure".

[3]	3G TS 23.107:  “QoS Concept and Architecture”.

[Editor's note : To add references to the 3G TS 28.062 (TFO), GSM EFR, IS-641and PDC 6,70]

# 5	RAB aspects

During the RAB Assignment procedure initiated by the CN to establish the RAB for AMR, the RAB parameters are defined. The AMR RAB is established with one or more RAB co-ordinated sub flows with predefined sizes and QoS parameters. In this way, each Transport Format Combination between sub flows corresponds to one AMR frame type. On the Iu interface, these RAB parameters define the corresponding parameters regarding the transport of AMR frames.

Some of the QoS parameters in the RAB assignment procedure are determined from the Bearer Capability Information Element used at call set up. These QoS parameters as defined in [3], can be set as follows:

| RAB service attribute |  |  | ## RAB service attribute value |  |  | ## Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Traffic Class |  |  | Conversational |  |  |  |
| Maximum bit rate |  |  | 12.2 / 10.2 / 7.95 / 7.4 / 6.7 / 5.9 / 5.15 / 4.75 kbit/s |  |  | This value depends on the highest mode rate in the RFCS |
| Guaranteed bit rate |  |  | 12.2 / 10.2 / 7.95 / 7.4 / 6.7 / 5.9 / 5.15 / 4.75 kbit/s |  |  | One of the values is chosen, depending on the lowest rate controllable SDU format (note 2) |
| Delivery Order |  |  | Yes |  |  | (note 1) |
| Maximum SDU size |  |  | 244 / 204 / 159 / 148 / 134 / 118 / 103 / 95 bits |  |  | Maximum size of payload field in IU UP, according to the highest mode rate in the RFCS |
| Traffic Handling Priority |  |  | Not applicable |  |  | Parameter not applicable for the conversational traffic class. (note 1) |
| Source statistics descriptor |  |  | Speech |  |  | (note 1) |
| SDU Parameters |  |  | RAB subflow 1 (Class A bits) | RAB subflow 2 (Class B bits) | RAB subflow 3 (Class C bits) | The number of SDU, their number of RAB subflow and their relative subflow size is subject to operator tuning (note 3) |
|  | SDU error ratio |  | 7 * 10-3 | - | - | (note 3) |
|  | Residual bit error ratio |  | 10-6 | 10-3 | 5 * 10-3 | (note 3 – applicable for every subflow) |
|  | Delivery of erroneous SDUs |  | yes | - | - | Class A bits are delivered with error indication; Class B and C bits are delivered without any error indication. |
|  | SDU format information 1-9 |  |  |  |  | (note 4) |
|  |  | Subflow SDU size 1-9 | (note 5) | (note 5) | (note 5) |  |
|  | Subflow SDU size parameters 10 |  |  |  |  | (note 6) |
|  |  | Subflow SDU size 10 | 0 | 0 | 0 |  |

Table 5-1: Example of mapping of BC IE into QoS parameters for UMTS AMR.

Note 1:	these parameters apply to all UMTS speech codec types.
Note 2:	the guaranteed bit rate depends on the periodicity and the lowest rate controllable SDU size.
Note 3:	these parameters are subject to operator tuning.
Note 4:	SDU format information has to be specified for each AMR core frame type (i.e. with speech
                    bits and comfort noise bits) included in the RFCS as defined in [2].
Note 5:	The subflow SDU size corresponding to an AMR core frame type indicates the number of bits
                    in the class A, class B and class C fields.
 	
Note 6:	Indication of SDU size = 0 is needed to inform RNC about possible change of the inter PDU
		transmission interval for some frames (SID frame in this case).

The conversational traffic class shall be used for the speech service, which is identified by the ITC parameter of the bearer capability information element in the SETUP message. This shall apply for all UMTS speech codec types.
The parameters traffic class, transfer delay, traffic handling priority and source statistics descriptor shall be the same for all speech codec types applicable for UMTS.

6	Iu Interface User Plane (RAN)

The data structure exchanged on the Iu interface are symmetrical, i.e. the structure of the uplink data frames is identical to that of the downlink data frames. This facilitates Tandem Free Operation and Transcoder Free Operation.

## 6.1	Frame structure on the Iu UP transport protocol

### 6.1.1	Initialisation

At the initialisation of the SMpSDU mode of operation, several parameters are set by the CN. The initialisation procedure is described in TS 25.415 [1].

- RFCS

In the case of AMR, the RFCS corresponds to the Active Codec Set (ACS) authorised in the communication. Annex A of [1] gives an illustration of the usage of RFCI for AMR speech RAB.

- Delivery of erroneous SDUs

This parameter shall be set to YES. Erroneous speech frames may be used to assist the error concealment procedures.

- [Editor’s note : this might need to be specified in another specifications]PDU type

The PDU type 0 shall be used for the transport of AMR data.

[Editor’s note : this might need to be specified in another specifications]

### 6.1.2	Time Alignment Procedure

[ffs as in [1]]

## 6.2	Mapping of the bits

The mapping of the bits between the generic AMR frames and the PDU is the same for both uplink and downlink frames.

The following table gives the correspondence of the bit fields between the generic AMR frames at the TC interface and the PDU exchanged with the Iu transport layer.

| PDU field | Corresponding AMR generic frame field | Comment |
| --- | --- | --- |
| PDU Type | N/A | Type 0 |
| Frame Number | N/A |  |
| FQC | Frame Quality Indicator |  |
| RFCI | AMR Frame Type |  |
| Payload CRC | N/A |  |
| Header CRC | N/A |  |
| Payload Fields (N Sub Flows) | Class A or SID payload Class B Class C |  |
| SDU #1 | Most important speech bits come first | Mandatory |
| SDU #2 | Next bits follow | Optional |
| … | … | Optional |
| SDU #N | Least important speech bits | Optional |

Table 0-1 : Mapping of generic AMR frames onto Iu PDUs

The number of RAB sub flows, their corresponding sizes, and their attributes such as "Delivery of erroneous SDUs" shall be defined at the RAB establishment and signalled in the RANAP RAB establishment request, as proposed in chapter 5. The number of RAB sub flows are corresponding to the desired bit protection classes. The total number of bits in all sub flows for one RFC shall correspond to the total number given in TS 26.101 for the corresponding Codec Mode respectively Frame Type.

Guidance for setting the number of bits in each RAB Sub Flow according to their relative subjective importance is given in TS 26.101.

The following two tables are examples of mapping of RAB sub flows.

Table 6-2 gives three examples of sub flow mapping. 
The RFCI definition is given in order of increasing SDU sizes.

- Example 1 describes Codec Type UMTS_AMR, with all eight codec modes foreseen in the Active Codec Set (ACS) and provision for Source Controlled Rate operation (SCR). In this example, Blind Transport Format Detection is supported and the sub flow mapping follows the 26.101 class division guidance.

- Example 2 describes Codec Type GSM_EFR, with one codec mode, including SCR.

- Example 3 describes Codec Type GSM_AMR, including AMR SCR

| UMTS_AMR | GSM_EFR | GSM_AMR | RAB sub-flows |  |  | Total size of bits/RAB sub-flows combination (Mandatory) | Source rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RFCI Example 1 | RFCI Example 2 | RFCI Example 3 | RAB sub- Flow 1 (Optional) | RAB sub- Flow 2 (Optional) | RAB sub- Flow 3 (Optional) |  |  |
| 2 |  | 2 | 42 | 53 | 0 | 95 | AMR 4.75kbps |
| 3 |  |  | 49 | 54 | 0 | 103 | AMR 5.15kbps |
| 4 |  | 3 | 55 | 63 | 0 | 118 | AMR 5.9kbps |
| 5 |  |  | 58 | 76 | 0 | 134 | AMR 6.7kbps |
| 6 |  | 4 | 61 | 87 | 0 | 148 | AMR 7.4kbps |
| 7 |  |  | 75 | 84 | 0 | 159 | AMR 7.95kbps |
| 8 |  | 5 | 65 | 99 | 40 | 204 | AMR 10.2kbps |
| 9 | 2 |  | 81 | 103 | 60 | 244 | AMR 12.2kbps |
| 1 |  | 1 | 39 | 0 | 0 | 39 | AMR SID |
|  | 1 |  | 47 | 0 | 0 | 47 | GSM EFR SID |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | NO DATA |

Table 6-2 : Example for AMR with SCR and three sub flows, according to subjective class division indication of TS 26.101.

Table 6-3 gives one examples of sub flow mapping that supports Equal Error Protection.
The RFCI definition is given in order of increasing SDU sizes.

- Example 4 describes Codec Type PDC_EFR and  the corresponding Source Controlled Rate operation (SCR).

| PDC_EFR | RAB sub-flow | Total size of bits/RAB sub-flows combination (Mandatory) | Source rate |
| --- | --- | --- | --- |
| RFCI Example 4 | RAB sub- Flow 1 (Mandatory) |  |  |
|  | 95 | 95 | AMR 4.75kbps |
|  | 103 | 103 | AMR 5.15kbps |
|  | 118 | 118 | AMR 5.9kbps |
| 2 | 134 | 134 | AMR 6.7kbps |
|  | 148 | 148 | AMR 7.4kbps |
|  | 159 | 159 | AMR 7.95kbps |
|  | 204 | 204 | AMR 10.2kbps |
|  | 244 | 244 | AMR 12.2kbps |
|  | 39 | 39 | AMR SID |
|  | 47 | 47 | GSM EFR SID |
|  | 42 | 42 | IS-641 SID |
| 1 | 41 | 41 | PDC 6,7 SID |
| 0 | 0 | 0 | NO DATA |

Table 6-3 : Example of SDU sizes for PDC_EFR with SCR and Equal Error Protection

## 6.3	Frame handlers

Iu PDU Frame handling functions are described in TS 25.415. This sections describes the mandatory frame handling functions at the AMR Generic frame interface.

### 6.3.1	Handling of frames from TC to Iu interface (downlink)

The frames from the TC in AMR generic frame format are mapped onto the Iu PDU as follows.

#### 6.3.1.1	Frame Quality Indicator

The Frame Quality Indicator from the TC, respectively from the distant TFO partner, is directly mapped to the Frame Quality Classification of the IU frame according to Table 6-6.

| FQI AMR | FQC PDU | FQC value |
| --- | --- | --- |
| GOOD | GOOD | 0 |
| BAD | BAD | 1 |

Table 6-4 : FQI AMR to FQC Iu PDU mapping

#### 6.3.1.2	Frame Type

The received Frame Type Index is mapped onto the RFCI thanks to the assigned RFCS table: The correspondence between Codec Mode, Frame Type Index and RFCI is defined at RAB assignment.

#### 6.3.1.3	Codec Mode Indication

The Codec Mode Indication is not used because it is redundant to the Frame Type.

#### 6.3.1.4	Codec Mode Request

Codec Mode Request (CMR) in downlink direction is forwarded to the rate control procedure if it changes.

#### 6.3.1.5	Optional internal 8 bits CRC

The internal AMR CRC is not used on the Iu interface.

#### 6.3.1.6	Mapping of Speech or Comfort Noise parameter bits

Let define the N payload fields of the N Sub flows for RFCI j as follow :

Ui(k)	shall be the bits in Sub Flow i, for  k =1 to Mi
	Mi  	shall be the size of Sub Flow i, for i = 1 to  N
	S(k) 	shall be the bits of the speech or comfort noise parameters of the corresponding Frame Type j 		
			in decreasing subjective importance.

Then the following mapping in pseudo code applies:

U1(k) 			= 	S(k)					with k = 1, 	… M1

U2(k) 			= 	S(k + M1)				with k = 1, 	… M2

U3(k) 			= 	S(k + M2)				with k = 1, 	… M3

…

UN(k) 			= 	S(k + MN-1)			with k = 1 	… MN

### 6.3.2	Handling of frames from Iu interface to TC (uplink)

The uplink Iu frames are mapped onto generic AMR frames as follow :

#### 6.3.2.1	Frame Quality Indicator

At reception of Iu PDU the Iu frame handler function set the Frame Quality Classification according to the received FQC, Header-CRC check, and Payload-CRC check (see 25.415). AMR Frame Type and Frame Quality Indicator are determined according to the following table:

| FQC | Resulting FQI | resulting Frame Type |
| --- | --- | --- |
| GOOD | GOOD | from RFCI |
| BAD | BAD | NO_DATA |
| BAD Radio | BAD | from RFCI |
| Reserved | Reserved | Reserved |

Table 6-5 : FQC Iu PDU type 0 to AMR FQI and AMR Frame Type mapping

#### 6.3.2.2	Frame Type

The received RFCI is mapped onto the Frame Type thanks to the RFCS table. I.e. the Type_Index is set according to the AMR mode.

#### 6.3.2.3	Codec Mode Indication

The Codec Mode Indication is not used because it is redundant to the Frame Type.

#### 6.3.2.4	Codec Mode Request

The received Downlink Rate Control command is mapped onto the Codec Mode Request. In case a new DRC is received it is mapped into the corresponding CMR AMR generic frame format. It is remembered by the TC until the next DRC is received. In each new frame that is sent to the AMR Codec, the stored CMR is resent, in order to control the Codec Mode for the downlink direction.

#### 6.3.2.5	Optional internal 8 bits CRC

The internal AMR CRC is not used on the Iu interface.

#### 6.3.2.6	Speech and Comfort noise parameter bits

The speech and Comfort noise parameter bits are mapped from the sub flows to the payload of the generic AMR frames with the reverse function of subclause 6.3.1.6.