TSG SA4#100 meeting	Tdoc S4-181020

15-19 October 2018, Kochi, India	Agenda item: 6.2

3GPP TSG-CT WG3 Meeting #98	C3-185174

# West Palm Beach, Florida (US), 20-24 August 2018

Title:	LS on RoHC support

Response to:	LS (C3-185132/S4-180990) on RoHC and FEC support from SA4

Release:	Rel-15

Work Item:	TEI-15, GCSE_LTE-CT

Source:	CT3

To:	SA4

Cc:	SA6

Contact Person:

#### Name:	Wenliang Xu

Tel. Number:	+86 137 6429 8380

E-mail Address:	wenliang (dot) xu (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

CT3 thanks SA4 for the LS on RoHC and FEC support (C3-185132/S4-180990), which provides a set of requirements for RoHC and FEC support within the attached CR to TS 26.346 (S4-180872).

For RoHC, in particular, the CR specifies in subclause 8B.4.1 that:

If RoHC is enabled through the MB2 for the GC delivery session, then the following conditions shall apply: - Only the U-mode shall be used throughout the lifetime of the session. Other modes shall not be used. - The RTP/UDP/IP profile with identifier 0x0101 shall be used as specified in [138] - The LARGE_CIDS flag shall be set to false - There shall only be one context per ROHC channel and MAX_CID shall be set to 0 - ROHC segmentation shall not be used and the MRRU value shall be set to 0 - A single ROHC channel per UDP flow shall be used

Consequently, if several UDP/RTP flows are delivered within a MBMS bearer, each will use a different ROHC channel with the CID 0.

Given that ROHC packets do not provide information about the ROHC channel they belong to, it is not clear for CT3 how UEs could demux the flows as each received ROHC packet is associated to the CID 0.

Question 1): can SA4 clarify how the UE can sort out multiple ROHC channels delivered over an MBMS bearer?

Use of different contexts is specified precisely to handle multiple compressed flows over a same ROHC channel. Therefore, if question 1) cannot be solved, CT3 proposes to consider the MBMS bearer as a single ROHC channel and to use different CIDs for each flow.

Question 2): does SA4 agree with this proposition? If yes, could the LARGE_CIDS flag be set to true (use of the large CID representation) if the number of flows to be transported is greater than 16 (max number of contexts when using the short CID representation)?

2. Actions:

To SA4 group.

ACTION: 	CT3 kindly asks SA4 to answer above questions and update stage 2 requirements if needed.

3. Date of Next CT3 Meetings:

CT3 Meeting #98Bis	15th October – 19th October 2018	Vilnius, Lithuania

CT3 Meeting #99	26th November – 30th November 2018	West Palm Beach, Florida (US)