3GPP TSG-SA4 Meeting #101	S4-181250

# Busan, Korea, 19th - 23rd November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.939 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (dCR) E-FLUS use cases and network assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony Mobile Communications, Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Tdoc S4-181045, which proposed the elaboration of the breaking news reporter use case, was agreed at SA4#100. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Elaboration of the breaking news reporter use case, and introduction of the Network Assistance feature for FLUS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of background information on Network Assistance for FLUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 6, new clause 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

===== 1ST CHANGE =====

# 4	FLUS Overview

FLUS defines a FLUS source entity and a FLUS sink entity that can support point-to-point transmission of speech/audio, video, and text. It defines media handling (e.g., signalling, transport, packet-loss handling, and adaptation). The goal is to ensure a reliable and interoperable service with a predictable media quality while allowing for flexibility in the service offerings.

A FLUS source entity, which may be embedded in a single UE, or distributed among a UE and separate audio-visual capture devices, may support all or a subset of the features specified in the present document.

When used as a generic framework, only the F-C procedures for establishing the FLUS session are required to be supported by the source and sink entities, and no other feature or procedure specified in the present document is mandated. Impact on the service quality and network capacity is left to the discretion of the implementation and the service utilizing the framework. For example, configuration of media formats and codecs follows the requirements of the respective service.

When offered as part of a 3GPP IMS/MTSI service, the source and sink are required to support the IMS control plane and media plane procedures, and the service quality is determined by the MTSI service policy.

When offered as part of a 3GPP non-IMS/MTSI service, the service may make use of the Network Assistance feature for uplink services.

The present document provides guidelines for the usage of FLUS and describes different user plane instantiations that can be used with FLUS.

===== END OF 1ST CHANGE =====

===== 2ND CHANGE =====

# 6	Guidelines for non-IMS-based FLUS

## 6.1	Use Case: Sharing to a Social Network Service

A user shares a 360 degree video that is being captured through a VR camera and sent as a fish eye, side-by-side 2D video. The 360 video stream is shared with a FLUS Sink in the network that relays the stream to a popular social network service (SNS).

## 6.2	Use Case: Live uplink video stream from drones or moving vehicles

The media producer for an event is using drone-mounted-360 cameras or other moving vehicles like F1 cars, sailing boats or bicycles to capture scenes from more innovative angles. The drone is flown using line of sight, i.e. the drone pilot has direct visual contact to the drone. Other vehicles may have the driver / pilot on-board.

The live video is streaming to the live ingest server and then used together with other camera feeds in a live TV broadcast.

In particular for battery powered cameras, it may be beneficial to avoid processing like 360 video stitching on the device. Instead, it may be beneficial to leverage network based post processing functions, e.g. multiple video streams are transmitted and the stitching function is executed in the network.

Use case example: An event organizer plans to use multiple drone-mounted cameras to capture live video from an event. All live video streams should be routed to an editing facility, where a program director decides on the sequencing of live video into a single linear program. The media source of each drone is configured with their own target quality (bitrate) and target delay. Each media source is configured with a unique media sink so that the program director can identify each media source.

This use case is generic in the sense that the camera is not limited to be "drone-mounted" but can be mounted to any device, vehicle, or stationary object.

## 6.3	Use Case: Breaking-News reporter

A news corporation uses 5G and mobile equipment to speed up and simplify their breaking-news operations. Either, professional cameras are equipped with 5G uplink streaming modems, or regular smartphones (with external microphones) are used for video capture. The universally available 3GPP coverage is used to stream the live video (with configurable, low delay) from the breaking news scene to the broadcast operation studio.

A news corporation negotiates a Service Level Agreement (SLA) with an operator so that a set of reporters can do either sequential or simultaneous live reports. The general frame agreement between the news corporation and the MNO foresees, that each reporter can determine their own maximum video quality (measured in bits per second). Each reporter should set their own quality, but some reporters are allowed to provide higher quality (i.e. use higher bitrates) than others.

This use case illustrates well the benefits and usage of Network Assistance for FLUS, whereby a mobile professional news team that is sent out to cover a sudden news situation. The team consists of the camera/sound man and the reporter. They arrive at the scene on their motorbike. The camera/soundman unpacks the camera, powers it on, and mounts it on his shoulder ready to shoot as soon as the reporter has made a quick assessment of the scene and is ready to report back to the TV studio.

The UE logs in to the network and establishes the secure, reliable, high bandwidth 5G connection with the TV studio, under the terms of the special subscription, or Service Level Agreement (SLA), that the TV station has with the incumbent 5G MNO in the region.

While the issue of SLA is out of scope of the FLUS specification, it is useful to consider two kinds of SLA that could be relevant here:

- SLA with guarantees for certain uplink bitrates – for this the MNO provisions RAN and network capacity in order for the TV station to be able to use FLUS to upstream their preferred media stream format reliably, anywhere where the MNO’s network is reachable.

- SLA with higher prioritisation of uplink media streams – for this the MNO agrees to prioritise uplink media streams from the TV station, over those of other users that do not have the SLA in place. This facility could also be included in the first type of SLA.

In the present use case, it is assumed the connection is ad-hoc in nature, so there is no guaranteed QoS from the MNO in this case. This corresponds to the second kind of SLA mentioned above. The objective of the news team is to deliver the best possible quality version of coverage that is possible in that location with the network capabilities and conditions at hand, allowing for possible variations during the coverage. In other variants of this use case, there might be provision for a particular quality level of audio/video stream to be delivered, for example using a QoS framework.

After the camera unit has powered up, it proceeds through the sequence of interactions with the network, so that it is ready to start streaming the best possible quality version of audio/video coverage under the current local network conditions when live coverage starts.

===== END OF 2ND CHANGE =====

===== 3RD CHANGE =====

# 10	Guidelines for Network Assistance for FLUS

## 10.1	Introduction

The feature of Network Assistance has been defined so far for 3GPP content downlink services, in particular PSS [a], including the extension of the 3GP-DASH files format specification [b] to adopt the MPEG-SAND messaging construct for its realization. While SAND was developed within the scope of DASH-based content delivery, the general concept of “network-assisted” content delivery is valid for any content container format. Hence this messaging construct is adapted to realise the equivalent functionality for uplink content provision in FLUS.

MPEG-SAND provides a generic message container format and a set of messages. The adaptation and extension of MPEG-SAND to realise downlink Network Assistance for PSS uses largely content container format-independent messages. The only parameter defined within those messages that originates from the principles of DASH is the segmentDuration message element. This element is re-purposed to mean any arbitrary regular time period of a content stream, whether in a segmented format or not.

The applicability and usage of Network Assistance for the various FLUS content formats is specified in TS 26.238 [2].

The SAND PER and status messages that are appropriate for usage to realise Uplink Network Assistance are specified in TS 26.238 [2].

As with downlink Network Assistance for PSS, DANE discovery for FLUS Network Assistance will be realised best using the DNS-based discovery method rather than methods that use the sand:Channel MPD element, which is in any case a DASH-specific tool.

The key functional entity of SAND, namely the DANE (DASH-Aware Network Element), is inappropriately named for the purposes of FLUS Network Assistance. Hence a more appropriate name for the corresponding network element is “Network Assistance Server” (NAssS). This network element is generic, in that in principle it could be used for 3GPP services other than FLUS.

## 10.2	Network Assistance within the FLUS architecture

Figure y shows an example deployment of Uplink Network Assistance (UNA) with respect to the FLUS architecture.

UNA is seen as an accessory to the core FLUS functionality. Furthermore, Network Assistance is seen as a cross-layer feature that is generally applicable to streaming services, both uplink and downlink. The NAssS can be operational in the Operator Network Edge, i.e. in the cell where the FLUS UE is located, or it can be in the Operator Network/Cloud, as is the case with the downlink Network Assistance DANE. The same holds for the FLUS Sink, but independently of the location of the NAssS.

For these reasons the Network Assistance feature for FLUS is incorporated as a separate feature within the FLUS architecture, for which an example deployment configuration is shown in figure y. The key aspect is that UNA is a separate optional feature that places a dedicated client function in the FLUS UE and dedicated server function in the operator RAN or core network.

Figure 1: FLUS Network Assistance within the FLUS architecture

## 10.3	Network Assistance for FLUS workflow

Based on the breaking news reporter use case described in clause 6.3, the following workflow is elaborated, in order to set out the basis for the Network Assistance feature for FLUS.

After the camera unit (UE / FLUS Source) has powered up, it proceeds through the sequence of interactions with the network (MNO RAN / NAssS and Content ingestion point / FLUS sink) as depicted in figure 2, so that it is ready to start streaming the best possible quality version of audio/video coverage under the current local network conditions.

Figure 2: FLUS Network Assistance work flow

- The UE registers in the Operator Network.

- The UE establishes the connection with the TV station for uplink. This step corresponds to the FLUS session establishment procedure as defined in TS 26.238 [2].

- The UE uses UNA to establish that Network Assistance is supported at the location.

- The UE establishes a UNA session with the NAssS that it locates.

- The UE uses UNA to request a bitrate recommendation for the uplink audio/video stream, to be selected from the versions available in the UE.

- The NAssS provides the bitrate recommendation.

- The UE starts/continues the uplink transmission at the recommended bitrate.

Steps 5, 6 and 7 can be repeated at regular intervals during the uplink session.

- The UE finds that there are transient problems with reaching the FLUS source to supply data packets and the source stream buffer risks getting full. It sends a boost request to the NAssS, in order to attempt to alleviate the content buffer being too full.

- The NAssS accepts the boost request and prioritises reception from the UE for the next period of uplink transmission, enabling a higher bitrate than nominally allocated to be transferred for a short period.

===== END OF 3RD CHANGE =====