3GPP TSG-SA4 Meeting #101	S4-181436

Busan, Korea, 19. – 23. November 2018	revison of S4-181293

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | 0004 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture for QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Sony Mobile Communication |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 13/11/2018 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR aims to correct the FLUS architecture wrt QoS and other system interactions for network assistance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | QoS interfaces as well as network assistance function are indicatde in the architecture |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

Definition of NassS

References for Rx, T8 and SGi

*** Next Change ***

### 4.2.3	Uplink streaming for PSS-based distribution

Figure 4.2-4a shows the most important interfaces for Evolved Packet System (EPS) and 5G System (5GS) for Live Uplink Streaming Services.

Figure 4.2-4a: Architecture for Live Uplink Streaming

An uplink streaming service requires at least a FLUS source and a FLUS sink. A FLUS sink is located at the SGi interface in case of EPS and at the N6 interface in case of 5GS. When QoS support is needed, the FLUS sink may request QoS support using Rx or T8 reference points in case or EPS and using Nnef or Npcf reference points in case of 5GS.

The architecture of uplink streaming for subsequent PSS distribution is depicted in Figure 4.2-4b.

Figure 4.2-4b: Uplink streaming for PSS

The PSS Content Source is located on the UE side and contains the FLUS source.

*** Last Change ***