3GPP TSG-SA4 Meeting #102 	S4-190057

# Bruges, Belgium, Jan 28 - Feb 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.929 | CR | - | rev | - | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VR QoE Metrics Minor Updates and Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_QoE_VR |  |  |  |  |  | Date: |  |  | 2019-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Obsolete architecture picture, wrong wording, missing reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update picture, reformulate text, add reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not consistent content. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.4, 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

================================= Start of first change =================================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[3] 	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[4]	ISO/IEC 23009-1: 2014/Amd. 1:2015/Cor.1:2015" Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats".

[5]	ITU-T P.1203: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport".

[6]	A. Singla, S. Fremerey, W. Robitza, A. Raake, “Measuring and comparing QoE and simulator sickness of omnidirectional videos in different head mounted displays”, in 9th International Conference on Quality of Multimedia Experience (QoMEX), May 2017.

[7]	A. Singla, A. Raake, W. Robitza, P. List, B. Feiten, “AhG8: Subjective Quality Evaluation for Omnidirectional (360°) Videos”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G0152, 7th Meeting, July 2017.

[8]	ITU-T P.910, “Subjective video quality assessment methods for multimedia applications”,                   International Telecommunication Union, April 2008.

[9]	ITU-R BT.500-13, “Methodology for the subjective assessment of the quality of television pictures”, International Telecommunication Union, Jan. 2012.

[10]	R. S. Kennedy, N. E. Lane, K. S. Berbaum, and M. G. Lilienthal, “Simulator sickness questionnaire: An enhanced method for quantifying simulator sickness,” The International Journal of Aviation Psychology, 1993.

[11]	E. Asbun, Y. He, Y. He, “AHG8: InterDigital Test Sequences for Virtual Reality Video Coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0039, 4th Meeting, Oct. 2016.

[12]	S. Schwarz et. al., “Tampere pole vaulting sequence for virtual reality video coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0143, 4th Meeting, Oct. 2016.

[13]	W. Sun, R. Guo, “Test Sequences for Virtual Reality Video Coding from LetinVR”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0179, 4th Meeting, Oct. 2016.

[14]	3GPP TS 26.118 "3GPP Virtual reality profiles for streaming applications"

================================= Start of next change =================================

## 4.4	Viewport-related information flow

To better understand what metrics are possible to measure, we need to understand the flow of information inside the VR streaming application. A reference model for VR QoE measurements is described later in clause 6, defining a number of observation points. However, in the current clause we will not specifically focus on observation points, but more on the flow of information, and we will instead use the following picture to illustrate a possible information flow for a simple use-case:

Figure 4.4.1: Client Reference Architecture for VR DASH Streaming Applications (from [14])

The DASH data model in Figure 4.4.2 below is also used as a basis for the use-case:

Figure 4.4.2: DASH Data Model

As seen in Figure 4.4.2, the adaptation sets are the central part for any viewport-related handling of the DASH media. Each adaptation set covers a certain spatial area of the sphere (signalled in CC), and it may also contain additional information on the relative quality ranking (QR) between the sets. Thus adaptation sets are selected depending on the user pose, and within each adaptation set there might be several representations with different encoding bitrates.

Note that depending on the scenario (e.g. single stream region-independent, single stream region-dependent, multiple stream region-dependent) multiple adaptation sets may be used at the same time, for instance a low-quality low-bitrate full-coverage set used for the full 360 view, and one or more high-quality sets which mainly covers the intended viewport. In tiled distributions each adaptation set typically contain only one tile, at a certain resolution.

Figure 4.4.3 below shows a simplified signalling diagram for a use-case with different changes of user pose:

Figure 4.4.3: Simplified signalling use-case

Although very simplified, the use-case illustrates the possible division of responsibility between the VR application and the DASH access engine. The VR application uses the sensor information and the MPD coverage and quality metadata to continuously decide which adaptation sets that shall be used. The task of the DASH access engine is to continuously fetch the media segments for the selected adaptation sets, while possibly adapting between different representations depending on the bitrate conditions in the network

In this use-case the DASH access engine does not have any knowledge about the pose or viewport orientation, or other viewport aspects such as field-of-view. It only tries to fetch suitable segments for the adaptation sets specified by the VR application, and deliver these for decoding and rendering.

Thus any metrics related to pose or viewport handling must either use specific non-viewport-related events known at DASH level (such as change of requested adaptation sets and the later delivery of the related media frames), or use combined trigger events derived from the VR application and the Decoder/Renderer (which might be difficult).

================================= Start of next change =================================

### 7.2.6	Summary

In this clause we have investigated the influence of resolutions, camera motion, motion in the content, and simulator sickness on QoE. Results show that 4K/UHD provides better QoE as compared to FHD resolution. In addition, we have shown that camera motion and motion in the content interacts with the QoE. Simulator sickness has also an impact on the QoE and vice-versa. Content that has the lowest simulator sickness scores provides the maximum QoE to the users and vice-versa.

Hence it is essential to include detection of features for camera motion and motion in the content for deriving metrics that help to estimate simulator sickness and consequently the influence of simulator sickness on the VR QoE.