3GPP TSG-S4 Meeting #102	S4-190106

# Bruges, BE, 28 January – 1st February 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.238 | CR | 0005 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Processing Description Document for FLUS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 22-Jan-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A FLUS Source is able to request media processing and distribution as part of the session establishment request. Currently, the specification only has a placeholder for the Processing Description but does not neither define format nor semantics of this processing description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes the MPEG NBMP Workflow Description as the format and semantics for the Processing Description in FLUS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Application of post-processing and distribution will remain under-specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 2, 5.3.2.7 |  |  |  |  |  |  |  |  |

1st Change

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R BS.2051-1 (06/2017): "Advanced Sound System for Programme Production".

[3]	ISO/IEC DIS 23090-2: "Omnidirectional Media Format. N16824".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[5]	3GPP TS 26.235: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[6]	3GPP TS 24.147: "Conferencing using the IM Multimedia (IM) Core Network (CN) subsystem; Stage 3”.

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[9]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[10]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[11]	IETF RFC 7231: "Hypertext transfer protocol (HTTP/1.1): Semantics and Content".

[12]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	3GPP TS 23.682: " Architecture enhancements to facilitate communications with packet data networks and applications".

[15]	3GPP TS 23.501: "System Architecture for the 5G System".

[16]	ISO/IEC CD 23090-8, MPEG-I Part 8: "Network-Based Media Processing"

2nd Change

#### 5.3.2.7	List of FLUS session properties

All FLUS session properties, except for the resource id, are always carried in an HTTP message body. The access-token is always carried as part of HTTP headers. Except for the FLUS session creation request (where the id is not present), the resource id shall be present in the URL of all requests that relate to a specific FLUS session.

In the table below, the following assertions are made:

-	Table header: C stands for Create FLUS session procedure, G is for Get FLUS session properties procedure, U is for Update FLUS session properties procedure and T is for Terminate FLUS session procedure. “I”, and “O” respectively denote “request” (going Into the FLUS sink), and response (going Out of the FLUS sink).

-	Optional (“O”) means that the property may or may not be sent/received during a REST transaction. It does not necessarily mean that the property is optional. It is possible, for example, that a session is not yet active because the FLUS source has not set the property in any previous update transaction using the PUT or PATCH HTTP method, as opposed to representing a hint on the importance of the property for the FLUS sink.

-	A property marked as optional (O) in a request message may be present in the request. When not present in the request body, the property, if present in the FLUS sink, will not be updated.

-	A property marked as optional (O) in a response message is only present in the response when a value is assigned or changed by the FLUS sink.

-	A property marked as mandatory (M) in a response message is always present in the response. The FLUS sink provides defaults, which may be modified subsequently by the content provider.

-	A blank cell in the table means “forbidden” (the property cannot be added to the request or returned by the FLUS sink, depending on the transaction direction)..

Table 5.3.2.7-1: List of FLUS session properties

| Property Name | Property Description | C I | C O | G I | G O | U I | U O | T I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| id | Identifier of the FLUS session resource. Note, the id is only provided within an HTTP body during the Create FLUS session response. Otherwise, the id is present in the message URL to identify the resource in the FLUS sink. Type; Unit; Default. Integer; None; N/A. |  | M |  |  |  |  |  |
| F-U Instantiation | Identifier of the FLUS media instantiation that is used by this FLUS session. Vendor specific enumeration values shall start with ‘vnd-‘ followed by a unique vendor name and optionally followed by additional characters. The F-U instantiation shall be provided as a globally unique URN with the parameter name "fu_instantiation". Type; Unit; Default. URI; None; All. |  |  |  | M | O |  |  |
| Entrypoint URL | Entry point URL information (e.g., SIP URL) for establishing the F-U connection to start the media streaming. Details on the Entrypoint URL is F-U instantiation specific. The JSON name of this parameter shall "entrypoint_url". |  |  |  |  |  |  |  |
| Processing Description | Provides a Workflow Description Document, as defined in [16], which describes the requested media processing and the desired distribution mechanism after the processing has been performed. The document shall be provided as a JSON object with the name "processing_description". |  |  |  | O | O |  |  |