Source:	Nokia Corporation

Title:	Proposal for MASA format

Document for:	Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

The metadata-assisted spatial audio (MASA) is currently under discussion as an audio format for the IVAS codec. An initial description for the MASA format was discussed in the SA4#98 Kista meeting [1]. In the SA4#100 Kochi meeting it was furthermore agreed, on the EVS SWG level, a definition on MASA spatial metadata [2] proposed by Nokia Corporation and Fraunhofer IIS. In the SA4#101 Busan meeting, a common metadata structure and the overall metadata structure was proposed [3].

The current input document provides a detailed description of the MASA spatial metadata use including algorithmic descriptions for analysis and synthesis. In addition, the metadata structure is clarified based on the discussions in the SA4#101 Busan meeting.

# 2. MASA spatial metadata description

As previously discussed, MASA is a parametric spatial audio format. Such formats describe spatial audio scenes using audio channels and spatial parameters. A widely known and proven model is to perform processing in time-frequency domain and to estimate spatial directions and energy ratios for each time-frequency tile. Perhaps the best-known literature example of this is Directional Audio Coding that uses a single spatial direction and a single diffuse-to-total energy ratio for each time-frequency tile. This model assumes that all sound energy in a sound scene for one time-frequency tile is coming from one spatial direction or all around the listener and this relation is described by the diffuse-to-total ratio.

In MASA, it is presented a version of this model that uses one or two spatial directions and multiple energy ratios to describe the sound scene. These parameters are found in Table 2 of this document and their use are described in clause 2.2.

First, however, we describe a complete example based on the IVAS MASA Example Software package [4] that has been made available in early July 2018 to requesting parties. While the software is not necessary for understanding the description in the present document, it can be helpful for experimentation.

For arranging access to the software, please contact:

Nokia Technologies Oy

Lasse Laaksonen

lasse.j.laaksonen@nokia.com

## 2.1 Description of the IVAS MASA Example Software

Analysis in Example Software

The example system starts with the microphone signals from an Eigenmike recording with the intention of simulating a mobile device audio capture with a commonly available microphone grid. Eigenmike contains 32 microphones and thus there are 32 signals provided. In the example MASA processing, this Eigenmike signal is converted to two separate signals: stereo transport signal and FOA signal for analysis. This can be viewed as a “mobile-device simulation”, where the transport signals could be obtained by selecting certain microphone signals (e.g., the left and right microphone signal) while still using all microphone signals for the spatial analysis. In the example, stereo transport signal is formed by selecting suitable left and right signals from the microphone grid and applying second order IIR equalization to the signals. The selected signals are signals with indices 14 and 30.

The first step for the FOA signal formation is to convert the Eigenmike signals to time-frequency domain by means of STFT (short-time Fourier transform). The further processing is then performed for each time-frequency frame. The equations presented here will have time and frequency indices, i.e., n,k, omitted for clarity. These would be included in each signal and derived parameter.

The FOA signals are formed with a set of pre-calculated filters that convert the signal from Eigenmike to FOA. Creation of such filters has been described, e.g., in [5]. The result is an FOA signal that contains four signal channels. This signal is formulated into FuMa-ordering (i.e., the traditional B-format ordering) as the public DirAC equations used in the example software have been defined for it. Thus, the signal is:

b=bwbxbybz.

The next step is to calculate intensity and energy related parameter values according to equations:

e=b22

i=Rbwbxbybz*.

It should be noted that these are not the actual physical properties but parameters that are closely related to the physical properties. Actual direction of intensity is opposite of above and scaling factors have been omitted for efficiency and SN3D scaling is assumed for the FOA signal to simplify calculations. This analysis methodology is an optimized version of the standard DirAC analysis (see [6, pages 90-91]).

After formation of these parameters, they are combined in frequency to form frequency-band based values to reduce required amount of further calculations and data. This combination is simply done by summation of all values within a frequency band as further steps do not require that absolute scale is maintained. In further processing, it is assumed that these parameters work on the desired frequency resolution.

The next step is to estimate the DOA, i.e., the spatial direction parameter, and the direct-to-total energy ratio. For spatial direction, this is done with equation:

θ=arctaniyixπ180°

for azimuth and equation

φ=arctanizix2+iy2π180°

for elevation. Here, arctan function is assumed to be the computational variant “atan2” that solves the correct quadrant automatically.

Direct-to-total energy ratio is solved with equation

rdir=ieband.

Here, ⋅ signifies averaging in time. In the example, this averaging is performed with a one-pole IIR-filter but this can also be done with a FIR filter.

Finally, spread coherence and surround coherence are calculated. First a general coherence parameter is calculated with equation:

Cgen=1-bx2+by2+bz2bw2,

where ⋅ signifies averaging over time and frequency (similarly as in direct-to-total ratio calculation). With this, spread coherence is formed with equation:

Cspr=rdirCgen

And surround coherence is formed with equation:

Csur=rdiffCgen.

Here, diffuse-to-total energy ratio is used for clarity even though the example uses the direct relation. It is assumed that rdiff=1-rdir.

Distance is not estimated in the example and it is set to value of zero. This provides us with a full set of parameters.

Synthesis in Example Software

The synthesis uses these parameters and the transport signals to synthesize the perception (but not the physical reproduction) of the original sound scene to the listener. This process starts by defining the prototype signals for synthesis. This example uses two transport signals, and these are used to form three transport signals (left, center, right) with equations:

sprot_left=strans_left

sprot_center=strans_left +stransright2

sprot_right=strans_right

These prototypes are used based on the rendering setup. As the example renders to a loudspeaker setup, the left prototype is used for the loudspeaker channels located to the left, the right prototype is used for the loudspeaker channels located to the right, and the center prototype is used for the loudspeaker located on the midline. In further processing, it is assumed that the correct prototype signal has already been selected for use.

Synthesis in the example is based on dividing the signal energy to multiple streams. These streams are direct signal stream and diffuse signal stream. The energy division between them is based on the direct-to-total energy ratio parameter.

Direct stream is synthesized with amplitude panning. The chosen method in the example is the well-known Vector-Base Amplitude Panning (VBAP). This method creates triangle mesh from loudspeakers and calculates gains for any direction by using three loudspeakers that are closest to the panning direction. In the example, this vector gain table has been pre-computed into a grid-based gain table that is accessed with quantized values of spatial direction. Thus, gains are obtained based on the azimuth and elevation as such:

gθ,φ=g1θ,φg2θ,φ⋮gNθ,φ,

where N is the number of channels in the loudspeaker setup. These gains are still time and frequency dependent as were the previous parameters.

Next, spread coherence is applied to these gains. The example synthesis applies spread coherence by obtaining gains for ±30° angles as well and then modifies the original “center” gains and the “spread” gains using the following equations based on the spread coherence value:

g'cen   =gcen1-2Cspr+2Cspr3    ,when Cspr<0.5gcen2-2Cspr                      ,when Cspr≥0.5

g'+30=g+302Cspr3    , when Cspr<0.5g+30                , when Cspr≥0.5.

g'-30=g-302Cspr3    , when Cspr<0.5g-30                , when Cspr≥0.5.

These gain vectors are added together to form the total spread gain vector.

gspr=g'cen+g'+30+g'-30.

This gain vector is then normalized such that total energy sum is one.

After this, surround coherence is applied to these gains. Surrounding coherence is applied as coherent synthesis to all directions and thus modifying direct signal gains achieves this purpose optimally. In addition, energy ratio parameters are also compensated with this process. This is shown with the following equations:

r'diff=rdiff-rdiffCsur

r'dir=rdir+rdiffCsur

rsur=rdiffCsurrdir+rdiffCsur

gfinal=gspr 1-rsur+rsurN .

Here rsur is surround coherence ratio used as a helper variable and N is again the number of channels in the loudspeaker setup. The resulting gains are normalized so that their energy sum is one.

These gains are then averaged in time with a one-pole IIR filter. This filter is usually frequency-dependent and tuned such that time constant is several cycles for the frequency. Again, FIR filter could be used as well. This filter is used to stabilize the direction rendering. After filtering, gains are again normalized back to total energy of one.

The final step is to apply the direct-to-total energy ratio to the gains:

g'final=gfinalr'dir.

These gains are then applied to the proper prototype signal matrix.

Sdir=Sprotg

For the diffuse stream, only ratio parameters and gain scaling are applied to the prototype signals:

Sdiff=Sprotr'diffN.

This step is then followed with a decorrelation step where each separate diffuse stream loudspeaker signal is processed with a different filter from a decorrelation filter bank. This is to reduce (or optimally to remove) the coherence between loudspeaker channels in the diffuse stream. This decorrelation step is done in the example after conversion to time domain with a separate FFT-based convolution. Good examples for decorrelation filter bank creation can be found in [7].

The final step is to sum the direct and diffuse stream together to obtain the output signals.

Sout=Sdir+Sdiff.

## 2.2 MASA spatial metadata parameters

The meaning and use of the MASA spatial metadata parameters are described next. This descriptions augments descriptions in previous MASA input documents. Analysis and synthesis are based on the IVAS MASA Example Software description provided above.

Direction index: Spatial direction(s)

Spatial directions represent the directional energy flows in the sound scene. Each spatial direction in tandem with direct-to-total energy ratio describes how much of the total energy for each time-frequency tile is coming from the specific direction. In general, this parameter can also be thought of as the direction of arrival (DOA).

There are multiple established methods to analyse spatial direction or DOA for spatial sound. One such method is the method in DirAC (as shown in the IVAS MASA Example Software) that is based on estimating the instantaneous active sound intensity and taking the opposite of it as the direction.

Using the spatial direction parameter in synthesis can be done in multiple ways as well. A suitable solution is to create a virtual sound source using panning suitable for the used reproduction system. For example, with loudspeakers, amplitude panning such as VBAP can be used. This is done separately for each direction and each time-frequency tile similarly as the metadata has been analysed.

Spatial direction could be represented in multiple ways. Unit vector or azimuth and elevation pairs are commonly used. In the agreed spatial metadata set a single direction index relative to a spatial grid is used. This requires less data and allows for practically uniform spacing on a sphere.

An example algorithm for using the direction index is provided in the IVAS MASA Example Software. Further description can be found in Annex A of the present input document.

In the current proposal, there can be one or two spatial directions (per frame) in the input metadata.

Direct-to-total energy ratio(s)

Direct-to-total energy ratios work together with spatial directions as described above. Each direct-to-total energy ratio corresponds to a specific spatial direction and describes how much of the energy comes from the specific spatial direction compared to the total energy. This value is also represented for each time-frequency tile separately.

According to the current proposal, there can be one or two spatial directions (per frame) in the input metadata and therefore, correspondingly, one or two direct-to-total energy ratios.

For analysis, DirAC presents a method of estimating diffuseness that is based on estimating the directional average energy compared to the total average energy [6, page 91]. This is adapted in the above example to estimate direct-to-total energy ratio. In various analysis implementations, direct-to-total energy ratio(s) could be estimated directly.

In synthesis, direct-to-total energy ratio is used to scale the output signal that will be panned using the spatial direction. This is a simple multiplication considering that energy ratios are energy parameters and a square root needs to be taken before applying to amplitude.

Spread coherence

Spread coherence is a parameter that describes the directional energy flow. It represents situations where coherent directional sound energy is coming from multiple directions at the same time. This can be represented with a single spread coherence parameter that describes how the sound should be synthesized.

In synthesis, this parameter can used such that value 0 means that sound is synthesized to single direction as directed by spatial direction, value 0.5 means that sound is synthesized to central direction and two surrounding directions as coherent, and 1 means that sound is synthesized to two surrounding directions around the spatial direction. This is explained in more detail above in the IVAS MASA Example Software description.

Diffuse-to-total energy ratio

Diffuse-to-total energy ratio represents non-directional energy flow in the sound scene. This is a complement to the direct-to-total energy ratios and in an ideal capture (or synthesized sound scene), the diffuse-to-total ratio value is always

rdiff= 1-∑rdir.

Thus, it is not common to directly analyse this energy ratio and it is instead formed as a complement of other energy ratios. In synthesis, this parameter is used together with decorrelation filters to create a diffuse stream that does not have perceptible direction.

Surround coherence

Surround coherence is an additional parameter to describe the non-directional energy flow. It represents how much of the non-directional energy should be presented as coherent reproduction instead of decorrelated reproduction. In synthesis, this is reproduced similarly to diffuse stream but the decorrelation filtering step is skipped.

Remainder-to-total energy ratio

Remainder-to-total represents all the energy that does not “belong” to the captured sound scene based on the used model. This includes possible microphone noise and other capture artifacts that have not been removed from the signal in pre-processing. This means that by considering the direct-to-total energy ratio, the diffuse-to-total energy ratio, and the remainder-to-total energy we end up with a complete energy ratio model of:

mMrdirm+rdiff+rrem=1.

Analysis of the remainder-to-total energy ratio is capture-system specific, and the parameter is included mainly for completeness to allow taking it into account where available. If this parameter is available at the synthesis, it should be reproduced such that the direct and diffuse signals mask it optimally.

Distance

Distance parameter complements the spatial direction parameter to form a complete spatial location for each time-frequency tile. In synthesis, the prime use for this parameter is to allow modification of the sound scene with head translations in addition to head rotations thus enabling, e.g., 3DoF+ or 6DoF usage.

It is the understanding of the source that this parameter would be used only with 6DoF support.

# 3. Proposed edits for MASA format metadata structure and spatial metadata

Following the proposal in [3], Figure 1 illustrates the MASA metadata structure for case of a single direction per frame. Figure 2 provides the corresponding proposal with two directions.

In the Busan meeting (SA4#101) it was asked to clarify the spatial metadata structure. It is here done by reorganizing Table 2 into two parts: Table 2a and Table 2b, which describe the spatial metadata parameters dependent on number of directions and independent of number of directions, respectively. Otherwise, Table 2a and Table 2b correspond to the agreement documented in [2].

Table 1 provides a proposal for the common metadata as seen in [3]. While the current document provides no updates to this table, the source considers future updates to it are necessary. For example, the fields ‘Frequency resolution’ and ‘Channel audio format’ still need to be defined further.

Common metadata

(Table 1)

Direction 1 spatial metadata

(Table 2a)

Direction 2 spatial metadata

(Table 2a)

Common spatial metadata

(Table 2b)

Figure 2: Illustration of the metadata structure with two directions

Figure 1: Illustration of the metadata structure with one direction

Common metadata

(Table 1)

Direction 1 spatial metadata

(Table 2a)

Common spatial metadata

(Table 2b)

Table 1. Proposed MASA format common metadata parameters (from [3])

| Field | Bits | Description |
| --- | --- | --- |
| Version | 8 | Incremental version number for the MASA metadata format. Range of values: [1, 256] |
| [Frequency resolution] | [8] | [TBD] |
| Number of directions | 1 | Number of directions in the metadata. Each direction is associated with a set of direction dependent spatial metadata. Range of values: [1, 2] |
| Channel audio format | 8 | Describes the type of channel audio input and aspects relating, e.g., to the capture configuration, for the audio signal(s) corresponding to the metadata. Coded as an index value to a lookup table. [TBD] |
| (Reserved) | (7) | (Byte alignment.) |

Table 2a. MASA format spatial metadata parameters (dependent of number of directions)

| Field | Bits | Description |
| --- | --- | --- |
| Direction index | 16 | Direction of arrival of the sound at a time-frequency parameter interval. Spherical representation at about 1-degree accuracy. Range of values: “covers all directions at about 1° accuracy” |
| Direct-to-total energy ratio | 8 | Energy ratio for the direction index (i.e., time-frequency subframe). Calculated as energy in direction / total energy. Range of values: [0.0, 1.0] |
| Spread coherence | 8 | Spread of energy for the direction index (i.e., time-frequency subframe). Defines the direction to be reproduced as a point source or coherently around the direction. Range of values: [0.0, 1.0] |
| Distance | 8 | Distance of the sound originating from the direction index (i.e., time-frequency subframes) in meters on a logarithmic scale. Range of values: for example, 0 to 100 m. (Feature intended mainly for future extensions, e.g., 6DoF audio.) |

Table 2b. MASA format spatial metadata parameters (independent of number of directions)

| Field | Bits | Description |
| --- | --- | --- |
| Diffuse-to-total energy ratio | 8 | Energy ratio of non-directional sound over surrounding directions. Calculated as energy of non-directional sound / total energy. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) |
| Surround coherence | 8 | Coherence of the non-directional sound over the surrounding directions. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) |
| Remainder-to-total energy ratio | 8 | Energy ratio of the remainder (such as microphone noise) sound energy to fulfil requirement that sum of energy ratios is 1. Calculated as energy of remainder sound / total energy. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) |

# 4. Conclusion and Proposal

This contribution provided detailed description of analysis and synthesis for the MASA format based on the spatial metadata agreed in the Kochi meeting. The description is standalone with links to literature where considered helpful. In addition, the IVAS MASA Example Software can be used to understand the details and to help with experimenting.

This contribution furthermore clarifies the structure of the common and spatial metadata taking into account the discussion in the Busan meeting.

It is the view of the source that, with these clarifications in mind, the IVAS MASA format could be agreed for the IVAS Design Constraints to provide high-quality immersive audio representation suitable for the important market segment of smartphones (or generally practical mobile devices with irregular and non-spherical multi-microphone arrays).

It is proposed to update IVAS-4 on Audio Formats and to add to IVAS-4 an Annex A documenting the MASA metadata structure including the revision of the agreed MASA spatial metadata (as given now in Tables 2a and 2b). Furthermore, the editor’s note is updated to correspond with the progress. The proposed change to IVAS-4 [8] Audio Formats is summarized as follows:

| Audio Formats | The IVAS codec shall support the following [input] formats: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. - Spatial audio, 1-4 channels and spatial metadata defined in Annex A. Editor’s Note FFS: TF resolution and common metadata for the spatial audio format remain TBD. - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] |
| --- | --- |

# References

[1]	Tdoc S4-180462: On spatial metadata for IVAS spatial audio input format. Source: Nokia Corporation.

[2]	Tdoc S4-181222: Draft report from SA4#100 EVS SWG. Source: EVS SWG Secretary (Orange).

[3]	Tdoc S4-181353: Proposal for MASA common metadata and metadata structure. Source: Nokia Corporation.

[4]	Tdoc S4-180789: Addressing open issues for spatial audio (MASA) format. Source: Nokia Corporation.

[5]	Archontis Politis, Microphone array processing for parametric spatial audio techniques, 2016
Doctoral Dissertation, Department of Signal Processing and Acoustics, Aalto University, Finland.

[6]	Pulkki V., Delikaris-Manias S., Politis A., Parametric Time-Frequency Domain Spatial Audio, 2018, 1st edition, published by John Wiley & Sons.

[7]	Kendall, G., The Decorrelation of Audio Signals and Its Impact on Spatial Imagery, 1996, Computer Music Journal 19(4), DOI: 10.2307/3680992.

[8]	Tdoc S4-181218: IVAS Design Constraints (IVAS-4), v0.0.6.

# Annex A: Explanation of the spherical index for spatial direction(s)

This section describes the means of obtaining the direction index containing the information relative to the spatial direction.

The spatial direction can be expressed, e.g., based on elevation and the azimuth. Each pair of values containing the elevation and the azimuth is first quantized on a spatial spherical grid of points and the index of the corresponding point is constructed. The spherical grid is on a sphere of unitary radius that is defined by the following elements:

- Uniform scalar quantizer for the elevation values between -90 and +90 degrees; the value 0 is contained in the codebook. The distance between consecutive elevation codewords is 0.7388 degrees. The values are symmetrical with respect to the origin. The number of positive elevation codewords is Nθ.

- For each elevation codeword value there are several equally spaced azimuth values defined such that the distance between the consecutive resulting points on the unitary sphere is the same irrespective of the elevation codeword value. One point is given by the elevation and the azimuth value. The number n(i) of azimuth values are calculated as follows:

n1=422

ni= πsin-1sinπn(1)R(i) ,i=1:Nθ

Ri=cos i-1ϕ, i=1:Nθ

ϕ=sin-123sinπn(1)61-sinπn(1)2+ sin-1233sinπn(1)

- The azimuth values for even values of i are equally spaced and start at 0.

- The azimuth values for odd value of i are equally spaced and start at πn(i).

- There is a same number of azimuth values for same absolute value elevation codewords.

The quantization in the spherical grid is done as follows:

- The elevation value is quantized in the uniform scalar quantizer to the two closest values θ1, θ2

- The azimuth value is quantized in the azimuth scalar quantizers corresponding to the elevation values θ1, θ2

- The distance on the sphere is calculated between the input elevation azimuth pair and each of the quantized pairs θ1,ϕ1, (θ2,ϕ2)

di=-(sinθsinθi+ cosθicos(ϕ-ϕi)), i=1:2

- The pair with lower distance is chosen as quantized direction.

The resulting quantized direction index is obtained by enumerating the points on the spherical grid by starting with the points for null elevation first, then the points corresponding to the smallest positive elevation codeword, the points corresponding to the first negative elevation codeword, followed by the points on the following positive elevation codeword and so on.