3GPP TSG-SA4 Meeting #102	S4-190201

# Bruges, Belgium,  29. January - 1. February 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR |  | rev | <Rev#> | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G Streaming Architecture Proposals |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | 22.1.2019 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

TS 23.401: General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access

TS 23.501, System architecture for the 5G System (5GS)

TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2

TS 36.401, Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description

TS 38.401, NG-RAN; Architecture description

TS 28.404, Management of Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements

TS 28.405, Management of Quality of Experience (QoE) measurement collection; Control and configuration

TS 28.406, Management of Quality of Experience (QoE) measurement collection; Information definition and transport

# 4	General Service Architecture

## 4.1	General Concepts

< some descriptive text. >

## 4.2	5G Unicast Media Downlink Streaming Architecture

### 4.2.1	Media Architecture

This architecture represents the media architecture between UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.2.1-1: Media Architecture for unicast media downlink streaming

Note: The yellow filled boxes can be defined by SA4. The red lines can be in scope of SA4 specifications. It is ffs, which of the in-scope interfaces and functions will be defined by SA4. The green lines are under SA2 control and SA4 may provide some usage guidelines. The black lines are not in scope of SA4.

Functions:

-	5G Media Player Functions on UE: Receiver of the unicast downlink media streaming service.

Editor's note: It might be good to break down the 5G Media Player into more subfunctions

-	External Media Functions (e.g. App): A 5G Media Function on a UE is typically controlled by an App, which is implementing the content provider desired behaviour. The App is not defined within 5G Media Streaming.

- 	Media AS: An Application Server, which hosts 5G media functions. Note, there may be different realizations of Media ASes.

-	External Media Functions: For example, functions related to Media Creation.

-	Media AF (e.g. Network Assistance (NA) Application Function (AF): A Media AF provides different control like kinds of functions to 5G Media Functions on the UE. It may relay / trigger request for different Policy or Charging principles (PCT) or to other type network functions.

-

-	Media Control Plane and Media User Plane API functions: API provided for exposing media services to external media functions.

Note: There may be several Media AFs among the Trusted Media Functions.

### [4.2.2	LTE NR Dual Connectivity for Unicast Distribution

Editor's Note: The new RAN architecture separates between single Connectivity and dual connectivity according to http://www.3gpp.org/news-events/3gpp-news/1929-nsa_nr_5g. Options 3 and 3a is based on EPC and the other options on NGC (aka 5GC)

Editor's Note: The SA2 architectures should be referenced.

Editor's Note: The O&M path for QoE is defined in TS 28.404, 28.405 and 28.406

Figure 4.2.2-1: Network Architecture for Unicast Media Downlink Streaming with LTE NR Dual Connectivity

NOTE 1: Interface E is realized by Interfaces Rx and / or T8. Rx is the reference point towards PCRF. T8 is the reference point towards SCEF.

NOTE 2: The figure depicts an untrusted Application Function (AF), which is located outside of EPC.

NOTE 3: The QoE reports in EPC may be forwarded via RRC to an eNB, which forwards the reports via O&M to the QoE Reporting server

### 4.2.3	NR Single Connectivity for Unicast Distribution

Figure 4.2.3-1: Network Architecture for Unicast Media Downlink Streaming with NR Single Connectivity

NOTE 1: Interface E is realized by Interfaces Nnef, Npcf, Naf, N5 or N33.

NOTE 2: The figure depicts an untrusted Application Function (AF), which is located outside of 5GC.

NOTE 3: Nnef, Npcf and Naf are 5GC service-based exposure interfaces. N5 and N33 are the same interfaces in reference point representation

NOTE 4: MNO trusted functions can use Npcf or N6 interfaced. External functions should use Nnef or N33 interfaces

]

## 4.3	5G Media Uplink Streaming Architecture

### 4.3.1	Media Architecture

This architecture represents the media architecture between UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.3.1-1: Media Architecture for Uplink Streaming

Note: The yellow filled boxes can be defined by SA4. The red lines can be in scope of SA4 specifications. It is ffs, which of the in-scope interfaces and functions will be defined by SA4. The green lines are under SA2 control and SA4 may provide some usage guidelines. The black lines are not in scope of SA4.

Note: It is currently assumed that interfaces between Trusted UL Media Functions (i.e. in the box labled Trusted UL Media Functions) are not defined by SA4.

Note: Red ovals indicate that the function exposes an API Provider. API invokers may exess services, provided by this function.

Functions

-	5G UL Media Functions (Trusted): Sender of the uplink streaming data.

Editor's note: The 5G UL Media Functions (Trusted will be broken into smaller subfunctions.

-	External Media Functions (e.g. App): A 5G UL Media Function (Trusted) is controlled by an App, which is implementing the content producer desired behaviour. The App is not defined within 5G Media Streaming.

-	UL Media AS: Receiver of the media stream, originating from the 5G Media Source. The UL Media AS may forward the data to other UL Media ASes and even to External UL Media Functions.

- 	External UL Media Function: External Receiver of the uplink media data.

-	Media AF, e.g. Network Assistance (NA AF): A Media Application Function, which provides different kinds of Services.

Note: There may be several UL Media AFs among the Trusted UL Media Functions.

Reference Points:

### [4.3.2	LTE NR Dual Connectivity for Uplink Streaming

Figure 4.3.2-1: Media Uplink Streaming Architecture with LTE NR Dual Connectivity

NOTE 1: Interface E is realized by Interfaces Rx and / or T8. Rx is the reference point towards PCRF. T8 is the reference point towards SCEF.

NOTE 2: The figure depicts an untrusted Application Function (AF), which is located outside of EPC.

### 4.3.3	NR Single Connectivity for Uplink Streaming

Figure 4.3.3-1: Media Uplink Streaming Architecture with NR Single Connectivity

NOTE 1: Interface E is realized by Interfaces Nnef, Npcf, Naf, N5 or N33.

NOTE 2: Nnef, Npcf and Naf are 5GC service-based exposure interfaces. N5 and N33 are the same interfaces in reference point representation

NOTE 3: MNO trusted functions can use Npcf or N6 interfaced. External functions should use Nnef or N33 interfaces

NOTE 4: The Application Function (AF) may be located inside or outside of the Operator's trust domain.

## 4.4

## ]