3GPP TSG-SA WG4 Meeting #103	S4-190357

# Newport Beach, CA, USA, 8-12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Ericsson LM) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 02/04/2019 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Improving TS editorial aspects |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | several |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

3GPP TS 26.501 V1.0.1 (2019-04)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

5G Media Streaming (5GMS);

General description and architecture

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<5G, Media, Streaming, Live, Uplink, Codec, Multimedia, transport>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definition of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Media Streaming General Service Architecture	6

4.1	Overall Media Architecture	6

4.2	5G Unicast Media Downlink Streaming Architecture	6

4.2.1	Standalone – Non-Roaming	6

4.2.2	UE Media Functions	7

4.2.3	Non-Standalone LTE NR Dual Connectivity for Unicast Distribution	9

4.2.4	NR Single Connectivity for Unicast Distribution	10

4.3	5G Media Uplink Streaming Architecture	10

4.3.1	Media Architecture	10

4.3.2	LTE NR Dual Connectivity for Uplink Streaming	11

4.3.3	NR Single Connectivity for Uplink Streaming	12

4.4	Network Slicing	12

4.5	Edge Computing	12

Annex A (informative):	Usage Guidelines for collaboration scenarios	13

Annex B (informative):	Change history	14

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the 5G Media Streaming (5GMS) architecture. The 5GMS supported services include MNO and 3rd party Media Downlink Streaming Services, and MNO and 3rd party Media Uplink Streaming Services. The 5GMS supports related network and UE functions and APIs, backwards compatible functions for EUTRAN deployments (with and without MBMS) and 5G specific features.

NOTE:	Support of 5G Media streaming over MBMS with 5GC is not considered in the current version of this specification.

The 5GMS architecture is functionally divided into independent components enabling different deployments with various degrees of integration between 5G MNOs and Content Providers. It is specified as a set of extensions to TS 23.501 "System Architecture for the 5G System".

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5GMS	5G Media Streaming

5GS	5G Systems

AF	Application Function

ABR	Adaptive BitRate

API	Application Programming Interface

App	Application

AS	Application Server

CDN	Content Delivery Network

DASH	Dynamic and Adaptive Streaming over HTTP

DRM	Digital Rights Management

GPU	Graphics Processing Unit

MNO	Mobile Network Operator

MPD	Media Presentation Description

NA	Network Assistance

NR	New Radio

NEF	Network Exposure Function

PCF	Policy and Charging Function

QoE	Quality of Experience

SBA	Service based Architecture

# 4	Media Streaming General Service Architecture

## 4.1	Overall Media Architecture

Editor's note: Overall media streaming architecture including uplink and downlink.

## 4.2	5G Unicast Media Downlink Streaming Architecture

### 4.2.1	Standalone – Non-Roaming

This architecture represents the media architecture between UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.2.1-1: Media Architecture for unicast media downlink streaming

NOTE 1: The yellow filled boxes are in scope of stage 3 specifications for 5GMS. The red lines are in scope of stage 3 for 5GMS specifications.. The stage 2 and 3 for green lines are specified elsewhere and 5GMS stage 3 may include usage guidelines. The black lines are not in scope of 5GMS stage 3.

NOTE 2: 	Red oval indicates API provider functions.

NOTE 3:	The Media AF may also interact with NEF for NEF provided API access. However, within Release 16, only interactions with Policy and Charging (PCF) is considered in 5GMS specifications.

Editor's note: The interface between Media AF and Media AS may be in scope of SA4 (currently not shown on the architecture picture)

Functions:

-	5G Media Player Functions on UE: Receiver of the unicast downlink media streaming service.

Editor's note: It might be good to break down the 5G Media Functions into more subfunctions

-	External Media Functions (e.g. App): A 5G Media Function on a UE is typically controlled by an App, which is implementing the content provider desired behaviour. The App is not defined within 5G Media Streaming.

- 	Media AS: An Application Server, which hosts 5G media functions. Note, there may be different realizations of Media ASes.

-	External Media Functions: For example, functions related to Media Creation.

-	Media AF (e.g. Network Assistance (NA) Application Function (AF)): A Media AF provides different control functions to 5G Media Functions on the UE. It may relay / trigger request for different Policy or Charging principles (PCF) or to other types of network functions.

-	Media Control Plane and Media User Plane API functions: API provided for exposing media services to external media functions.

NOTE: 	There may be several Media AFs among the Trusted Media Functions.

The following subfunctions are identified as more detailed breakdown of the Trusted Media Functions.

-	ABR Encoder, Encryption and Encapsulator

-	Manifest (MPD) Generator and Segment (DASH) Packager

-	Origin Server

-	CDN Server (Edge Servers)

-	DRM Server (DRM License)

-	Metrics Server

-	Service Directory

-	Content Guide Server

-	Ad or Replacement content server

-	MPD Proxy

The following additional functions may be relevant:

-	App Server

-	Session Management Server

The following subfunctions are identified as more detailed breakdown of the UE based Trusted Media Functions:

-	Application

-	DASH access client

-	Manifest and Segment Parser

-	Decryptor and DRM client

-	Media Decoders for audio, video, media

-	Renderer (GPU, etc.)

-	Metrics Collector

### 4.2.2	UE Media Functions

Figure 4.2.2-1 shows the functional components of a UE to access network-based media functions.

Figure 4.2.2-1: UE Media Functions

NOTE 1: 	SBA refers to principles of 5G Service based Architecture (SBA) realizations of control transactions.

NOTE 2: 	The white color indicate here that the 3GPP has created specifications for the function.

NOTE 3: 	A UE is a logical device, allowing for tethering or other type of realizations.

The following subfunctions are identified as more detailed breakdown of the UE based Trusted Media Functions:

-	Operator App or Third-party App: Uses APIs to access media services

-	Media Access Client: Accesses e.g. DASH Formatted Media

-	DRM Client (Optional): When present, the DRM client realizes the policies.

-	Media Decoders: Decodes the media like audio or video

-	Media Rendering / Presentation: Renders the media using an appropriate output device.

-	Media Session Handler: Realizes a session concept, spanning optionally over multiple stateless sessions. May optionally interact with network-based Media AFs.

-	Metrics collection and reporting: executes metrics collection and reporting.

Interfaces:

NOTE 4: 	The following device internal APIs may exist inside the UE.

- MC1: Media Control Interface(s) to configure and interact with the different UE media functions.

- MC2: Media Control Interface for media session management.

- MU1: Decoded media samples are handed over to the media renderer.

- MU2: Decrypted, compressed media samples are handed over to a trusted media decoder.

- MU3: In case of encryption, the encrypted, compressed media samples are handed over to the DRM Client.

Editor's Note: An interface to the Metrics Collection and Reporting function is TBD.

Editor's Note: Roaming, Non-3GPP Access and EPC-5GC interworking aspects are TBD

[

Editor's Note: The text in square brackets is not agreed.

### 4.2.3	Non-Standalone LTE NR Dual Connectivity for Unicast Distribution

Editor's Note: The new RAN architecture separates between single Connectivity and dual connectivity according to http://www.3gpp.org/news-events/3gpp-news/1929-nsa_nr_5g. Options 3 and 3a is based on EPC and the other options on NGC (aka 5GC)

Editor's Note: The SA2 architectures should be referenced.

Editor's Note: The O&M path for QoE is defined in TS 28.404, 28.405 and 28.406

Figure 4.2.3-1: Network Architecture for Unicast Media Downlink Streaming with LTE NR Dual Connectivity

NOTE 1: 	Rx is the reference point towards PCRF. T8 is the reference point towards SCEF.

NOTE 2: 	The figure depicts an untrusted Application Function (AF), which is located outside of EPC.

NOTE 3: 	The QoE reports in EPC may be forwarded via RRC to an eNB, which forwards the reports via O&M to the QoE Reporting server.

### 4.2.4	NR Single Connectivity for Unicast Distribution

Figure 4.2.4-1: Network Architecture for Unicast Media Downlink Streaming with NR Single Connectivity

NOTE 1: 	Interface between AF/Network Assistance and PCF/NEF is realized by Interfaces Nnef, Npcf, Naf, N5 or N33.

NOTE 2: 	The figure depicts an untrusted Application Function (AF), which is located outside of 5GC.

NOTE 3: 	Nnef, Npcf and Naf are 5GC service-based exposure interfaces. N5 and N33 are the same interfaces in reference point representation.

NOTE 4: 	MNO trusted functions can use Npcf or N6 interfaces. External functions should use Nnef or N33 interfaces.

]

## 4.3	5G Media Uplink Streaming Architecture

### 4.3.1	Media Architecture

This architecture represents the media architecture between UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.3.1-1: Media Architecture for Uplink Streaming

NOTE 1: 	The yellow filled boxes are in scope of stage 3 specifications for 5GMS. The red lines are in scope of stage 3 specifications for 5GMS.. The stage 2 and 3 for green lines are specified elsewhere and 5GMS stage 3 may include usage guidelines. The black lines are not in scope of 5GMS stage 3.

NOTE 2: 	It is currently assumed that interfaces between Trusted UL Media Functions (i.e. in the box labelled Trusted UL Media Functions) are not specified in 5GMS stage 3.

NOTE 3: 	Red ovals indicate that the function exposes an API Provider. API invokers may access services, provided by this function.

Functions:

-	5G UL Media Functions (Trusted): Sender of the uplink streaming data.

Editor's note: The 5G UL Media Functions (Trusted) will be broken into smaller subfunctions.

-	External Media Functions (e.g. App): A 5G UL Media Function (Trusted) is controlled by an App, which is implementing the content producer desired behaviour. The App is not defined within 5G Media Streaming.

-	UL Media AS: Receiver of the media stream, originating from the 5G Media Source. The UL Media AS may forward the data to other UL Media ASes and even to External UL Media Functions.

- 	External UL Media Function: External Receiver of the uplink media data.

-	UL Media AF, e.g. Network Assistance (NA AF): A Media Application Function, which provides different kinds of Services.

NOTE 4: 	There may be several UL Media AFs among the Trusted UL Media Functions.

Reference Points:

[

Editor's Note: The text is square brackets is not agreed,

### 4.3.2	LTE NR Dual Connectivity for Uplink Streaming

Figure 4.3.2-1: Media Uplink Streaming Architecture with LTE NR Dual Connectivity

NOTE 1: 	Interface between AF/Network Assistance and PCF/NEF is realized by Interfaces Rx and / or T8. Rx is the reference point towards PCRF. T8 is the reference point towards SCEF.

NOTE 2: 	The figure 4.3.2-1 depicts an untrusted Application Function (AF), which is located outside of EPC.

### 4.3.3	NR Single Connectivity for Uplink Streaming

Figure 4.3.3-1: Media Uplink Streaming Architecture with NR Single Connectivity

NOTE 1: 	Interface E is realized by Interfaces Nnef, Npcf, Naf, N5 or N33.

NOTE 2: 	Nnef, Npcf and Naf are 5GC service-based exposure interfaces. N5 and N33 are the same interfaces in reference point representation

NOTE 3: 	MNO trusted functions can use Npcf or N6 interfaced. External functions should use Nnef or N33 interfaces

NOTE 4: 	The Application Function (AF) may be located inside or outside of the Operator's trust domain.

]

## 4.4	Network Slicing

Editor's Note: This clause is TBD

## 4.5	Edge Computing

Editor's Note: This clause is TBD

Annex A (informative): Usage Guidelines for collaboration scenarios

Editor's Note: The intention of this annex is to describe the media architecture instances based on various collaboration scenarios between e.g. MNOs and Third-Party Service providers.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 03-2019 | SA-83 | SP-190037 |  |  |  | Presented to TSG SA#83 (for information) | 1.0.0 |
| 04-2019 | SA4#103 | S4-190357 |  |  |  | pCR on Editorial updates | 1.0.1 |