Source:	Nokia Corporation

Title:	Experimental results for MASA TF resolution selection

Document for:	Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

Metadata-assisted spatial audio (MASA) has been proposed as one spatial audio input format that the future IVAS codec shall support. In particular, the MASA input format is intended to allow for high-quality spatial audio reproduction from spatial audio capture using a practical mobile device (e.g., a smartphone or any related form factor). The discussion about adopting this proposal remains ongoing with a few open questions. One such question is the time-frequency (TF) resolution of the spatial metadata. In [1], the source discussed some potential alternatives for TF resolution from psychoacoustic viewpoint as well as from the viewpoint of the EVS codec which will be part of the IVAS solution.

The present contribution addresses the previous discussion via a practical listening test experiment. Based on the results, the source draws recommendations on the IVAS MASA TF resolution for consideration into IVAS Design Constraints.

# 2. Conditions for listening test

In [1], it was discussed mainly two alternative approaches for defining the MASA TF resolution. Specifically, the TF resolution relates to a filter bank used for analysis and synthesis of the metadata. One approach in [1] is based on psychoacoustics while the other is based on the filter bank used in the EVS codec with a rough approximation of the former.

It is the view of the source that allowing the reuse of the EVS filter bank (or CLDFB) is preferable to facilitate a low-complexity implementation of the immersive extension. However, the perceptual quality must not be sacrificed due to such choice. It was already noted in [1] that at the lower frequencies, the minimum bandwidth of 400 Hz in CLDFB may be limiting. This effect must be understood. If the perceptual quality can be maintained, it seems logical to utilize a TF resolution that is compatible with the EVS filter bank resolution.

In order to estimate the practical effect of the proposed TF resolution limitation for the IVAS MASA model quality, an expert listening test was performed using MUSHRA to compare the following candidate systems (with various frequency resolutions):

- Bark_24: Bark scale approximation with 24 bands (see Table A-1)

- CLDFB_24: CLDFB-based, 24 bands (see Table A-2)

- CLDFB_18: CLDFB-based, 18 bands (see Table A-3)

- CLDFB_12: CLDFB-based, 12 bands (see Table A-4)

- CLDFB_6: CLDFB-based, 6 bands (see Table A-5)

- CLDFB_3: CLDFB-based, 3 bands (see Table A-6)

- CLDFB_1: Broadband (1 band)

In addition, there are the following conditions for the MUSHRA test:

- FULL: Hidden reference based on DFT analysis with 959 frequency bands (DC and Nyquist not analyzed)

- CLDFB1_LP: Low anchor, broadband (1 band), 3.5 kHz low-pass filtered

The signal analysis and synthesis were performed using a modified version of the Matlab processing scripts from the IVAS MASA Example Software package [2, 3]. The system was modified to support a 5-ms time step (while the Example Software implements a 20-ms time step) and several different frequency band divisions as given above. The DFT size was 2 times 960 samples as with unmodified example software thus resulting in maximum frequency resolution of 961 frequency bins of which DC and Nyquist components were synthesized using the neighboring bin value.

All the input files for the Matlab analyser were Eigenmike recordings with 32 channels. There were in total 12 test samples covering various challenging and critical scenarios. Instead of simple scenes, e.g., a single talker in a quiet environment (where even the broadband model would often provide excellent quality), the test material consisted of samples with background noise ranging from clearly audible to a significant level (the actual levels were not measured) and with multiple talkers from varying directions. Some talkers were over-lapping and/or moving around the scene. The background noises and recording environments include high-level traffic noise, outdoors recording in a park with birds in trees, and a large dining hall with active background noise. One of the tested samples was acoustic music without speech. In three speech samples there was background music playing back either from a 5.1 loudspeaker system or a Bluetooth mono speaker representing both immersive and simple background music scenarios.

There were in total 8 expert listeners.

# 3. Results

Figure 1 presents the results of the expert MUSHRA listening test. The results confirm the expectation that the perceptual quality increases when increasing the number of frequency bands. A closer fit to the Bark scale also helps. The Bark scale approximation with 24 frequency bands is the best-performing system under test. The various CLDFB-based variants follow it with decreasing number of frequency bands, although the difference particularly between CLDFB_18 and CLDFB_24 is small. The lower range of the systems (CLDFB_1, CLDFB_3, and CLDFB_6) does not provide sufficient quality over all critical samples. Sample-wise results are shown in Figure 2.

Table 1 presents all the homogeneous subsets (Tukey’s HSD test). The highest-quality systems under test form two statistically equivalent groups (CLDFB_12, CLDFB_18, and CLDFB_24; and Bark_24, CLDFB_18, CLDFB_24) according to the analysis. In addition, CLDFB_12 and CLDFB_6 form one statistically equivalent group.

Figure 1. MUSHRA test results, 8 listeners.

Table 1. Tukey’s HSD test results

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CLDFB1_LP | 19,0 |  |  |  |  |  |  |
| CLDFB_1 |  | 52,6 |  |  |  |  |  |
| CLDFB_3 |  |  | 64,9 |  |  |  |  |
| CLDFB_6 |  |  |  | 80,1 |  |  |  |
| CLDFB_12 |  |  |  | 85,3 | 85,3 |  |  |
| CLDFB_18 |  |  |  |  | 88,8 | 88,8 |  |
| CLDFB_24 |  |  |  |  | 89,2 | 89,2 |  |
| Bark_24 |  |  |  |  |  | 92,1 |  |
| Full |  |  |  |  |  |  | 99,1 |

Figure 2. Sample-wise mean scores, MUSHRA test, 8 listeners

The CLDFB from EVS is a known implementation with well-understood properties including delay, complexity, and aliasing-free separation of the frequency bins. As the IVAS codec is an extension of EVS, this filter bank is directly available and a good candidate for reuse wherever appropriate. On the other hand, a higher resolution at the lower frequencies (Table A-1 vs. Table A-2) seems beneficial. There is thus good listening test justification to consider also a Bark-based filter bank. This advantage could particularly be useful at the higher end of the bit rate range. However, a new filter bank has unknown properties in terms of delay, complexity, and other properties.

As a further consideration, Figure 3 presents the AB listening test results from the SA4#97 meeting in Fukuoka [4]. This listening test was performed according to the ITU-T P.800 Comparison Category Rating (CCR) methodology with 24 listeners and 120 sample pairs comparing a parametric method (MASA) against FOA. The signals were obtained from two practical multi-microphone product configurations, one with a three-microphone capture and the other one with a four-microphone capture.

The MASA system used in the Fukuoka listening test utilized an uncompressed spatial metadata description at about 25 kbit/s. This system is roughly equivalent in frequency resolution to the Bark_24 and CLDFB_24 systems under test in the present discussion. The source thus considers that these systems represent a relevant quality level for the MASA format.

Figure 3. AB listening test results from [4].

# 4. Summary and proposal

In this input document, listening test results were presented to assess the suitability of potential TF resolutions for the spatial metadata of the IVAS MASA format that is currently under discussion. Based on the evaluation it is the recommendation of the source to adopt the CLDFB-based system with 24 frequency bands as the TF resolution to be used for MASA spatial metadata. With a 5-ms time resolution for the metadata, there would thus be in total 96 TF tiles per frame.

The source kindly asks to agree on the TF resolution and to document this agreement in the IVAS Design Constraints (IVAS-4) document.

# References

[1]	Tdoc S4-181352: On TF resolution of MASA spatial metadata. Source: Nokia Corporation

[2]	Tdoc S4-180789: Addressing open issues for spatial audio (MASA) format. Source: Nokia Corporation

[3]	Tdoc S4-190121: Proposal for MASA format. Source: Nokia Corporation

[4]	Tdoc S4-180087: On IVAS audio formats for mobile capture devices. Source: Nokia Corporation

Annex A.

Bandwidth tables for the tested cases.

Table A-1. Bark scale approximated using CLDFB for higher frequencies

| Band | LF (Hz) | HF (Hz) | BW (Hz) | Band | LF (Hz) | HF (Hz) | BW (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 100 | 100 | 13 | 2400 | 2800 | 400 |
| 2 | 100 | 200 | 100 | 14 | 2800 | 3200 | 400 |
| 3 | 200 | 300 | 100 | 15 | 3200 | 3600 | 400 |
| 4 | 300 | 400 | 100 | 16 | 3600 | 4400 | 800 |
| 5 | 400 | 500 | 100 | 17 | 4400 | 5200 | 800 |
| 6 | 500 | 600 | 100 | 18 | 5200 | 6000 | 800 |
| 7 | 600 | 800 | 200 | 19 | 6000 | 6800 | 800 |
| 8 | 800 | 1000 | 200 | 20 | 6800 | 8000 | 1200 |
| 9 | 1000 | 1200 | 200 | 21 | 8000 | 10000 | 2000 |
| 10 | 1200 | 1600 | 400 | 22 | 10000 | 12000 | 2000 |
| 11 | 1600 | 2000 | 400 | 23 | 12000 | 16000 | 4000 |
| 12 | 2000 | 2400 | 400 | 24 | 16000 | 24000 | 8000 |

Table A-2. CLDFB with 24 bands

| Band | LF (Hz) | HF (Hz) | BW (Hz) | Band | LF (Hz) | HF (Hz) | BW (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 400 | 400 | 13 | 4800 | 5200 | 400 |
| 2 | 400 | 800 | 400 | 14 | 5200 | 5600 | 400 |
| 3 | 800 | 1200 | 400 | 15 | 5600 | 6000 | 400 |
| 4 | 1200 | 1600 | 400 | 16 | 6000 | 6400 | 400 |
| 5 | 1600 | 2000 | 400 | 17 | 6400 | 6800 | 400 |
| 6 | 2000 | 2400 | 400 | 18 | 6800 | 7200 | 400 |
| 7 | 2400 | 2800 | 400 | 19 | 7200 | 7600 | 400 |
| 8 | 2800 | 3200 | 400 | 20 | 7600 | 8000 | 400 |
| 9 | 3200 | 3600 | 400 | 21 | 8000 | 10000 | 2000 |
| 10 | 3600 | 4000 | 400 | 22 | 10000 | 12000 | 2000 |
| 11 | 4000 | 4400 | 400 | 23 | 12000 | 16000 | 4000 |
| 12 | 4400 | 4800 | 400 | 24 | 16000 | 24000 | 8000 |

Table A-3. CLDFB with 18 bands

| Band | LF (Hz) | HF (Hz) | BW (Hz) | Band | LF (Hz) | HF (Hz) | BW (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 400 | 400 | 10 | 3600 | 4400 | 800 |
| 2 | 400 | 800 | 400 | 11 | 4400 | 5200 | 800 |
| 3 | 800 | 1200 | 400 | 12 | 5200 | 6000 | 800 |
| 4 | 1200 | 1600 | 400 | 13 | 6000 | 6800 | 800 |
| 5 | 1600 | 2000 | 400 | 14 | 6800 | 8000 | 1200 |
| 6 | 2000 | 2400 | 400 | 15 | 8000 | 10000 | 2000 |
| 7 | 2400 | 2800 | 400 | 16 | 10000 | 12000 | 2000 |
| 8 | 2800 | 3200 | 400 | 17 | 12000 | 16000 | 4000 |
| 9 | 3200 | 3600 | 400 | 18 | 16000 | 24000 | 8000 |

Table A-4. CLDFB with 12 bands

| Band | LF (Hz) | HF (Hz) | BW (Hz) | Band | LF (Hz) | HF (Hz) | BW (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 400 | 400 | 7 | 2400 | 2800 | 400 |
| 2 | 400 | 800 | 400 | 8 | 2800 | 3600 | 800 |
| 3 | 800 | 1200 | 400 | 9 | 3600 | 5200 | 1600 |
| 4 | 1200 | 1600 | 400 | 10 | 5200 | 8000 | 2800 |
| 5 | 1600 | 2000 | 400 | 11 | 8000 | 12000 | 4000 |
| 6 | 2000 | 2400 | 400 | 12 | 12000 | 24000 | 12000 |

Table A-5. CLDFB with 6 bands

| Band | LF (Hz) | HF (Hz) | BW (Hz) | Band | LF (Hz) | HF (Hz) | BW (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 400 | 400 | 4 | 2000 | 4000 | 2000 |
| 2 | 400 | 1200 | 800 | 5 | 4000 | 8000 | 4000 |
| 3 | 1200 | 2000 | 800 | 6 | 8000 | 24000 | 16000 |

Table A-6. CLDFB with 3 bands

| Band | LF (Hz) | HF (Hz) | BW (Hz) |
| --- | --- | --- | --- |
| 1 | 0 | 4000 | 4000 |
| 2 | 4000 | 8000 | 4000 |
| 3 | 8000 | 24000 | 16000 |