Source:	Nokia Corporation

Title:	Examples of object-based audio control metadata for IVAS

Document for:	Discussion

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

The IVAS Design Constrains [1] specify support for an object-based audio format. However, the current definition is still lacking. It is understood that object-based audio should consist of individual streams (that could be mono) and some metadata parameters (TBD).

In [2], the source presented some discussion on potential metadata parameters for the object-based audio format in IVAS. It was furthermore considered that real-time conversational use cases are likely to differentiate object-based audio for IVAS from object-based audio, e.g., for audio or audio-visual broadcast applications.

In this current input contribution, the source considers some control metadata that should be considered for IVAS. It may be still useful to further consider IVAS features that enable advanced usage scenarios. Based on such considerations it should be possible to define at least a minimum set of control metadata parameters that guarantee the codec is not limited in its practical functionality.

# 2. Discussion

It can be useful to categorize object-audio metadata to better understand the scope of various metadata parameters to be considered for IVAS. One such categorization is, where the source notes that 6DoF support has not yet been agreed for IVAS, the following one from [2]:

- Rendering metadata. This metadata describes the rendering of the waveform(s). Typical examples include, e.g., spatial direction or position, spatial extent or size, etc. These metadata parameters can most likely be defined based on views of audio experts, results from experiments, etc.

- Control metadata. This metadata describes how the object audio can be used. For example, it may be indicated that an object audio can be switched on/off or its volume relative to an overall sound scene can be freely controlled. The control metadata should preferably be based on needs identified from IVAS-specific use cases.

[

- 6DoF metadata. This metadata describes properties and features that enable 6DoF use. For example, this metadata can relate to higher interactivity between the user and the audio environment than what is generally applicable to 3DoF use. The 6DoF metadata should preferably be based on needs identified from IVAS-specific 6DoF use cases such as augmented reality (AR).

]

As also earlier noted, some broadcast style metadata can be useful in IVAS. The source, however, believes that IVAS should particularly support conversational services, where object-based audio has so far not been thoroughly considered.

In the following, it is considered control metadata based on some use case discussion.

# 3. Use case examples

In addition to performing a spatial analysis, a spatial audio capture processing can include a beamforming operation. This can be called, e.g., audio focusing. A related use case can be described as a user story:

Bob is spending a beautiful summer day in a park and sees some street performers. Bob remembers that Tina likes street music and decides to call her. Bob makes a video call, where he aims his smartphone at the performers. Bob’s smartphone applies also an audio focus, where a beamforming is used to make the audio sources in the camera view more prominent. As Tina listens to the music Bob records, she can adjust the balance between the original spatial audio and the focused audio on her smartphone UI. Tina enjoys the performance, but she likes to bring back a bit of the ambience with the birds signing in the trees. This makes Tina wish to head outside herself.

In this use case, the audio focus is not only applied to the scene in a way that the receiving user has no knowledge of the operation. It is also desired to make the original scene available to the receiving user and allow the user to select between the original and the processed scene or even to finetune the balance between them. The effect thus needs to be delivered in a way where this user control is possible.

One possibility to deliver the effect is via audio objects. However, it should be understood that the user must not be allowed to move this type of audio object relative to the scene bed (delivered using, e.g., Ambisonics, MASA, etc.). Otherwise, the sound sources may be duplicated in various parts of the scene when the audio focus is applied. One property to control is thus a restriction for the position modification of the audio object. Additionally, this type of audio object may not be relevant alone without the playback of the spatial bed. Thus, it could be considered to call such an audio object “additive only”.

A further use case relates to user calling a service number. Before getting connected with the service personnel, the user, as is often the case, gets to listen to a nice recording. And as is often the case, such recordings can be provided in several languages. For example, in Tampere, Finland where this document has been written such recording would typically be played back in Finnish, Swedish, and possibly also English. Instead of playing back different language versions sequentially, IVAS may allow delivering separate language tracks as audio objects to the receiving UE. For example, in case of two audio objects, one language may be rendered on the left-hand side of the user, while a second language may be rendered on the right-hand side of the user. The user would then be able to select or activate/deactivate a language of their choosing on the UE user interface.

In this use case, the audio objects appear as alternatives to one another. The receiving user is able to control them relative to each other or to switch between them. For example, the user could toggle between Finnish and Swedish language. In this case, no adaptation is required on the transmitting side.

# 4. Control metadata for consideration

Based on the high-level use case examples discussed above, it is noted some parameters for which it would be needed control in IVAS. These include at least:

- Restrictions of rendering parameters of audio objects (default being no restriction)

- Linking of audio objects (e.g., a pair of audio objects are alternatives in some way)

- Role of the audio object in rendering (e.g., the audio object is not meant to be played back without a spatial bed or some linked audio object)

Based on these observations, it is collected the following table of initial “audio object types” for consideration:

| “Name” | General properties | Description (E.g., use case where useful) |
| --- | --- | --- |
| Default | - Separable at the decoder - User-controllable in rendering | Potential default audio object type with full user control. Audio object is mixed in For example, this can be a voice on teleconference that listener is free to move in the space and adjust volume separately etc. |
| Restricted control | - Separable at the decoder - User control in rendering allowed but restricted - Rules on rendering provided | Audio object type with restricted user control. List of restricted rendering parameters provided. For example, the effect of audio focus can be delivered as an audio object that the user can apply to rendering or not (or, e.g., control amount of audio focus using a slider). In this case, user is not allowed to change position of audio object relative to spatial signal and object can be assumed meaningless without the spatial signal. |
| Toggled pair (“alternative and alternating”) | - Separable at the decoder - ‘Alternative sound’ for a linked audio object (with default playback of first one only) - Selecting rendering of one disables the other (“toggling”) | Audio object type allows for toggling between two specific audio objects based on user control. For example, user can for a downstream service audio select on-the-fly between two (or more) language tracks. |
| Relatively controlled pair (“alternative but concurrent”) | - Separable at the decoder - ‘Alternative sound’ for a linked audio object (with default playback of both) | Audio object type allows for relative control of two specific audio objects based on user control. For example, user can directly adjust the balance between two audio tracks. Service or content can indicate which objects are together and simplify user control. |
| Additive only (with restricted control) | - Need not be separable at decoder (although may still be preferable) - User control in rendering allowed but restricted - At least some rules on rendering provided - Audio object is mixed with linked audio based on user control - Default operation (mixing on/off) can be an encoder input | Audio object type allows for adding an enhancement or change to audio scene, where the enhancement or change may not be desirable to be presented to user without a spatial audio bed. For example, when separable at decoder, the effect of audio focus can be delivered as an audio object that the user can apply to rendering or not (or, e.g., control amount of audio focus using a slider). Default operation says whether to downmix in encoder or not, if separable transmission is not possible (e.g., due to bit rate). |

# 5. Summary

In this input document, the source presented high-level conversational example use cases for IVAS. This discussion specifically relates to use of object-based audio either alone or in conjunction with a spatial audio bed (that could be delivered using various audio formats, e.g., Ambisonics or MASA). The source considers it useful to define control metadata relating to audio-object roles for the object-based audio format for IVAS.

# References

[1]	Tdoc S4-190248: IVAS Design Constraints (IVAS-4), v0.0.7.

[2]	Tdoc S4-190122: Consideration on object-based audio metadata for IVAS. Source: Nokia Corporation