3GPP TSG-SA4 Meeting #103	S4-190453

# Newport Beach, CA, USA, 8. - 12. April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (pseudo)CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for 5G Uplink Media Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | <Source_if_WG> |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document introduces a more detailed UE model for Uplink Streaming and also a procedure for uplink streaming. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 4.3	5G Media Uplink Streaming Architecture

### 4.3.1	Media Architecture

This architecture represents the media architecture between UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.3.1-1: Media Architecture for Uplink Streaming

NOTE 1: 	The yellow filled boxes can be defined by SA4. The red lines can be in scope of SA4 specifications. It is ffs, which of the in-scope interfaces and functions will be defined by SA4. The green lines are under SA2 control and SA4 may provide some usage guidelines. The black lines are not in scope of SA4.

NOTE 2: 	It is currently assumed that interfaces between Trusted UL Media Functions (i.e. in the box labelled Trusted UL Media Functions) are not defined by SA4.

NOTE 3: 	Red ovals indicate that the function exposes an API Provider. API invokers may access services, provided by this function.

Functions:

-	5G UL Media Functions (Trusted): Sender of the uplink streaming data.

Editor's note: The 5G UL Media Functions (Trusted) will be broken into smaller subfunctions.

-	External Media Functions (e.g. App): A 5G UL Media Function (Trusted) is controlled by an App, which is implementing the content producer desired behaviour. The App is not defined within 5G Media Streaming.

-	UL Media AS: Receiver of the media stream, originating from the 5G Media Source. The UL Media AS may forward the data to other UL Media ASes and even to External UL Media Functions.

- 	External UL Media Function: External Receiver of the uplink media data.

-	UL Media AF, e.g. Network Assistance (NA AF): A Media Application Function, which provides different kinds of Services.

NOTE 4: 	There may be several UL Media AFs among the Trusted UL Media Functions.

Reference Points:

### 4.3.2	UE Media Functions

Figure 4.3.2-1: UE 5G Media Uplink Streaming Functions

NOTE 1: A UE is a logical device, allowing for tethering or other type of realizations.

Definitions:

Media Streamer Entry: a pointer e.g. a URLthat defines an entry point of a media uplink streaming session.

5G Media Uplink Streamer (5GMUS) includes at least: Media Upstream Client, Media Encoder(s), Media Capturing(s) and may include Media Configuration, Remote Control and Assistance Handlers. The 5GMUS Streamer receives a Media Streamer Entry. The 5GMUS captures the media on the provided input devices. The 5GMUS Streamer exposes some basic controls such as capture, pause, stop to the App via MUC1. The 5GMUS streamer captures the media on the provided input devices.

Third-Party Uplink Streamer: The Third-Party Uplink Streamer is a part of the application and the application uses MF1, MF2 and MUC2 APIs to exercise selected 5GMUS functions to capture and stream the media content. This type of Streamer can be downloaded and uses some selected 5GMUS functions.

The following subfunctions are identified as more detailed breakdown of the UE 5G Media Uplink Streaming Functions:

- Operator App. or Third-party App.: application which is out of scope of the present specification and uses the UE 5G Media Uplink Streaming Functions and APIs.

- Media Capturing(s): Devices such as video camera or microphone that transform an analog media signal into digital media data.

- Media Encoder(s): Compress the media data.

- Media Upstream Client: encapsulates encoded media data and pushes it upstream.

Note: this maps to the FLUS media function in the FLUS Source specified in TS 26.238 Uplink Streaming stage 3.

- Assistance Handler: requests and/or subscribes to receive assistance information from one or more Media AF in support of the streaming process.

Note: this is expected to map to the FLUS Assistance function specified in TS 26.238 Uplink Streaming stage 3.

- Media Remote Control Handler: receives control commands from a Media AF.

Note: this is expected to map to the FLUS Remote control function specified in TS 26.238 Uplink Streaming stage 3.

- Media Configuration: configures the Media AS for uplink streaming reception.

Note: this maps to the FLUS control function in the FLUS source specified in TS 26.238 Uplink Streaming stage 3.

Here are the role of the different APIs of the UE 5G Media Uplink Streaming Functions:

MUC1: Control Plane API used to configure, activate and stop the Media Capturing(s), Media Encoding(s) and Media Upstream Client functions.

MUC2: Control Plane API used to control Media Configuration, Assistance Handler, Media Remote Control Handler functions.

MF1: User Plane API between Media Capturing(s) and Media Encoder(s) functions that carries Media data to be encoded. Can also be used between a Third-Party Uplink Streamer in an App. and the Media Encoder(s) function.

MF2: User Plane API between Media Encoder(s) and Media Upstream Client functions to carry encoded Media data. Can also be used between a Third-Party Uplink Streamer in an App. and the Media Upstream Client function.

# 6	Procedures for Uplink Streaming

## 5.1	General

## 5.2	Establish an Uplink Downlink Streaming Session

Steps

1: During basic provisioning, the device is provisioned with basic information, such as the Media AF and Media AS addresses. For some devices, only the remote control information is provisioned and all additional parameters are retrieved from Remote Controller.

2: The App is started. The start may mean to start listening to remote control commands or to start actively the uplink streaming process.

3: The App instructs the Media Session Handler to start

When remote control is provisioning:

4: The transport session is established for the remote control

5: The UE starts waiting for incoming remote control commands

6: The UE receives a remote control command to start upstreaming (either now or with a timestamp). The Remote control command may contain details around the FLUS Sink and the used F-U instantiation, etc.

7: The session handler triggers the start of the uplink streaming session

8: the Media Upstream Client initiates the capture and encoder devices.

9: The media upstream client establishes the transport connection to the Media AS

10: The media upstream client establishes the media streaming session

11: The media upstream client notifies the media session handler about the start of the transport. The details around the transport session may be provided.

When client assistance is needed:

12: media session handler establishes the assistance channel to the provisioned Media AF

When server assistance is desired (e..g for QoS or charging)

13: The Media AS establishes a assistance session with the Media AF.