3GPP TSG-SA WG#4 Meeting #104	S4-190656

# Cork, Republic of Ireland, 1-5 July 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Network Assistance Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 25 June 2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are several Network Assistance procedures possible for DASH and Progressive streaming. The Media Session Handler can interact with the Media AF for different reasons (e.g. policy and charging function interaction, as identified in TR 26.891, Clause 6.4). A set of parameters is needed for the network assistance procedures. The number of parameters may depend on the use-case and the requested Network Assistance service. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

## 5.2	Establish a Unicast Media Downlink Streaming Session

### 5.2.1	General

This procedure describes the establishment of a Unicast Downlink Streaming Session. A streaming session may use DASH format, 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats.

Figure 5.2-1: High Level Procedure for progressive download for on-demand media

Steps:

1: The App triggers the Service and Content Discovery procedure.

The Content Discovery procedure only involves the App and the external Application Server and hence is out of scope of the present document.

2: A Media Player Entry is selected.

3: The App triggers the Media Session handler to start the playback. The media player entry is provided to the Media Session handler.

4: The Media Session Handler triggers the 5GMS Player to start the session.

5: The 5GMS Player establishes the transport session.

6: The 5GMS Player sends the request for the progressive download content.

7: The 5GMS Player receives the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

8: The 5GMS Player configures the rendering pipeline for media playback.

9: The 5GMS Player notifies the media session handler, providing the transport session information and some media content related information.

10: Optional Network Assistance: the media session handler establishes a media session with the Media AF. Additional parameters such as the transport session parameters are provided. See Clause 5.7 for further details.

11: Optional: 5GMS Player acquires a DRM License from an External AS.

12: The 5GMS Player receives media content and puts it into the rendering pipeline.

13: The 5GMS Player continuously receives and plays back the media content.

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.2-2: High Level Procedure for DASH content

Steps:

1: The App triggers the Service and Content Discovery procedure. The Content Discovery procedure only involves the App and the external Application Server.

2: A media content item is selected.

3: The App triggers the Media Session handler to start media playback. The Media Player Entry is provided to the Media Session handler.

4: The Media Session Handler triggers the 5GMS Player to start the session.

5: The 5GMS Player establishes the transport session for acquiring the manifest referenced by the Media Player Entry.

6: The 5GMS Player requests the manifest

7: The media access client receives the manifest

8: The 5GMS Player processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

9: The 5GMS Player notifies the media session handler about the manifest. The notification may contain parameters from the manifest

10: Optional Network Assistance: The Media Session handler establishes session with the Media AF. See Clause 5.2.2 for further details.

11: Optional: the 5GMS Player acquires the necessary DRM information, for example a DRM License.

12: The 5GMS Player configures the media rendering pipeline.

13: The 5GMS Player establishes the necessary transport sessions for the content. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

14: The media session handler is notified about the established transport sessions for the content.

15: Optional Network Assistance: The media session handler may notify the media AF about the session and the established transport sessions. See Clause 5.2.2 for further details.

16: The 5GMS Player requests initialization information. The 5GMSP repeats this step for each required initialization segment.

17: The 5GMS Player receives the initialization information.

18: The 5GMS Player requests media segments according to the manifest.

19: the 5GMS Player receives media segments and puts the information into the according media rendering pipeline.

20: Previous steps are repeated according to the manifest information.

## 5.2.2	Procedures to activate a Network Assistance Service

The Media Functions on the UE in particular the Media Session Handler, may interact with the Media AF in the network in order to receive certain network assistance services. The Media AF may offer 3GPP defined Network Assistances services as well as MNO specific services.

Editor’s Note: Authorization of the requests and security procedures are ffs.

The following set of Network Services are specified

- Conditional and dynamic policies for application instances: Provide input from the 5G Media Client so that the Media AF can interact with the Policy and Charging Function (PCF). The 5G Media Client may request usage of QoS as Network Assistance service (incl requesting a background download QoS class). Usage of different QoS classes may impact the generated traffic charges.

# Table 5.2.2-1: Network Assistance information

| Field | Description |
| --- | --- |
| Application Service Provider ID | Identifier of the media application service provider (ASP), who is offering the application and the content for the application. |
| Network Service Id | Identifier of the requested network service, such as QoS prioritization. Note that an application, offered by a single application service provider, may use different network services. |
| 5-Tuples | (wildcarded) 5-Tuples of the user plane sessions. |

Editor’s Note: A set of Network Service Ids may be defined during the Stage 3 work

### 5.2.3	Procedures to query status of a Network Assistance Service

The Media Functions on the UE in particular the Media Session Handler, may interact with the Media AF in the network in order to receive information around specific network services. The Media AF may offer 3GPP defined Network Assistances services as well as MNO specific services.

# Table 5.2.3-1: Query Request

| Field | Description |
| --- | --- |
| Application Service Provider ID | Identifier of the media application service provider (ASP), who is offering the application and the content for the application. |
| Network Service Id | Identifier of the network service, for which the status should be queried. |

# Table 5.2.3-2: Query Response

| Field | Description |
| --- | --- |