Source: 	Nokia Corporation

Title: 	Signaling Requirements for ITT4RT

Document for	Agreement

Agenda item: 	11.8

# Introduction

The purpose of this document is to identify the additional SDP signalling required for setting up a 360-degree conference call with unidirectional 360-degree video and bidirectional audio and possibly 2D-video. The use case is described in the ITT4RT permanent document [2], which consists of a conference room A and two remote users B and C. The naming, A, B, and C, is used in the same context here.

# Signaling Flows and Signaling Requirements

The high level signaling flows are depicted in Figure 1. The user C is not represented here for simplicity, but this is not a restriction for our reasoning. In this example MRF/MCU is not used.

Figure 1 Signalling flow for a 360-degree conference call with unidirectional 360- degree video from A to B.

The following parameters need to be signalled in the SDP during call setup in addition to normal MTSI call signaling [1].

- Initial viewport orientation. It is the default orientation from which to start the view at the receivers’ side.

- Decoding/Rendering metadata, e.g., region-wise packing information, projection mapping information, frame packing information, etc. It is subject of discussion whether this information is signaled via SDP and/or within SEI messages with the media stream.

- Capture Field-of-View (CFoV): as discussed during the definition of the use case, the system supports transmission of 360-degree video. However, the range of the FoV may be restricted in order to enhance user experience. The negotiation requires signaling the capture FoV of the capture device, and a response carrying the receiver’s preferred FoV (PFoV) depending on the remote UE, where the preferred FoV will be less than or equal to the captured FoV.

- Codec negotiation, including signalling for IVAS for immersive audio.

Once the call has been established, remote parties (B or C) can send viewport orientation information using RTCP reports with yaw, pitch and roll data. These may be sent at fixed intervals or event-based, triggered by changes in viewport orientation. The most efficient RTCP reporting scheme for viewport orientation information is for further study.

# Proposal

We propose that Section 2 of this document be moved to Sections 3-4 of the Permanent Document [2].

# References

[1] 3GPP TS 26.114, "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[2] “ITT4RT Permanent Document – Requirements, Working Assumptions and Potential Solutions”, 3GPP TSG SA WG4 #103 meeting, Newport Beach, U.S.A., Tdoc S4-190503.

Contact: Igor D.D. Curcio, Saba Ahsan, Nokia Technologies, Tampere, Finland, Email: igor.curcio, saba.ahsan@nokia.com.