3GPP TSG-SA4 Meeting #104	S4-190755

# Cork, Ireland, 1st Jul 2019 - 5th Jul 2019								Revision of S4-190600

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0446 | rev | 7 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 26.114-0446 rev7 on CHEM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHEM |  |  |  |  |  | Date: |  |  | 2019-06-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Defines the procedures for the CHEM feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifies the normative and informative aspects of the CHEM feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no specification for the CHEM feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 9.1, Annex X (new), Annex Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | From Rev5 (S4-190505) to Rev6: moved informative clause X.4 into new informative Annex Y. Text and note aound table Y.1 in clause Y.1.1. were removed as there was no concensus reached on this text. From Rev6 (S4-190600) to Rev7: separated ALR and non-ALR adaptation, added new SDP parameter for ALR adaptation, added negotiation features for CHEM feature, enabling MNO to disable when desired. |  |  |  |  |  |  |  |  |

### ==== Change #1 ====

[160]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[161]	3GPP TS 27.007: " Technical Specification Group Core Network and Terminals; AT command set for User Equipment (UE)".

[162]	3GPP TR 26.952: "Codec for Enhanced Voice Services (EVS); Performance characterization".

[163]	3GPP TR 26.959: "Study on enhanced Voice over LTE (VoLTE) performance".

### ==== Change #2 ====

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply:

NOTE:	A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

AMR, AMR-NB: Both names refer to the AMR codec (3GPP TS 26.071 [11]) and are used interchangeably in this specification.

CHEM: The Coverage and Handoff Enhancements using Multimedia error robustness feature.

Codec mode: Used for the AMR and AMR-WB codecs to identify one specific bitrate. For example AMR includes 8 codec modes (excluding SID), each of different bitrate.

Constrained terminal: UE that is (i) operating in radio access capability category series “M ” [162] capable of supporting conversational services, and/or (ii) a wearable device which is constrained in size, weight or power consumption (e.g. connected watches), excluding smartphones and feature phones.

### ==== Change #3 ====

# 9	Packet-loss handling

## 9.1	General

This clause specifies some methods to handle conditions with packet losses. Packet losses in general will also trigger adaptation, which is specified in clause 10.

The ‘a=bw-info’ attribute defined in clause 19 allows for negotiating how much additional bandwidth (if any) may be used for application layer redundancy in the session. When application layer redundancy is used, the media bandwidth negotiated for the session may need to be increased, i.e. by increasing the value used for the b=AS bandwidth modifier. The b=AS bandwidth modifier is however only a single value, which also applies only to the receiving direction. When an MTSI client in terminal sends the SDP, it is therefore not possible for the networks and the other client to know if the intention is to use the entire media bandwidth all the time (both with and without redundancy); or if the intention is to use the b=AS bandwidth only when redundancy is needed and to use a lower bandwidth when redundancy is not needed. It is also not possible to know what the MTSI client in terminal can do in the sending direction. The ‘a=bw-info’ attribute (see clause 19) offers an improved negotiation mechanism to better know what the MTSI client in terminal can do and what it intends to do. This is further discussed in TR 26.924 [144].

Improved error robustness can be enabled by packet-loss handling procedures of the codec or codec mode, via the adaptation procedures described in clause 10, or other mechanisms.  Annex X specifies the CHEM feature which enables the 3GPP system to exploit error robustness to avoid, delay, or reduce the need to handoff a terminal due to degradation in the media quality.  Annex Y provides example PLR threshold values that can be used for different codec configurations.

### ==== Change #4 ====

# Annex X (Normative):
Coverage and Handoff Enhancements using Multimedia Error Robustness (CHEM)

## X.1	General

The robustness of media to packet loss can be affected by the following factors in the MTSI terminal:

- codec

- codec mode

- application layer redundancy

- packet loss concealment implementation

- dejitter buffer implementation

The above factors are considered to be part of the codec configuration.

The level of robustness can affect the coverage area of the service in a network.  Furthermore, communicating the level of robustness of the media to the network enables the eNB/gNB to use this information to determine a threshold for when the terminal should be handed off to another cell, domain (circuit-switched vs. packet-switched), or radio access technology.

The MTSI terminal may support the coverage and handover enhancements using the multimedia error robustness (CHEM) feature specified in this clause.  When CHEM is supported by terminals, the terminals exchange packet loss rate (PLR) information via specified SDP parameters.  A PCF/PCRF may use these SDP PLR values to set handover thresholds in their local eNB/gNBs.

In the procedures specified in the rest of this clause, whenever the UE estimates the packet loss rate at the media receiver, the estimate can be measured either pre-dejitter buffer or post-dejitter buffer.  When packet loss rate is measured post-dejitter buffer, presentation of partial redundancy information (e.g., in EVS channel aware mode) does not count towards a correctly received packet.  Unless specifically configured using the OMA-DM Management Objects, the post-de-jitter buffer PLR estimate shall be used.

The PLR_adapt attribute to be used for this feature is specified using the following ABNF:

CHEM = "a=PLR_adapt:" [SP "ALR"]

The semantics of the above attribute and parameter are specified below.  Unsupported parameters of the PLR_adapt attribute may be ignored.

## X.2	Adaptation to Packet Losses without application layer redundancy

An MTSI terminal supporting the CHEM feature without using application layer redundancy for a media type (e.g., speech or video) shall support the following procedures:

- when sending an SDP offer, the MTSI client shall include the PLR_adapt attribute in the media description for that media type in the SDP offer

- when sending an SDP answer, the MTSI client shall include the PLR_adapt attribute in the media description for that media type in the SDP answer regardless of whether the PLR_adapt attribute was received in an SDP offer

- if, and only if, the MTSI client receives the PLR_adapt attribute in either an SDP offer or SDP answer then,

  - when the MTSI client receiving media detects packet losses higher than tolerable by the current codec mode/configuration in use and a more robust codec mode/configuration is available for the same codec, the MTSI client shall send a CMR request to the media sender to use a more robust codec mode/configuration of the same codec

  - when the MTSI client receiving media detects a packet loss rate low enough to support a codec mode/configuration of the same codec that provides better media quality than the current codec mode/configuration, and switching to the new codec mode/configuration will not cause oscillating between more robust and less robust codec modes/configurations, then the MTSI client should send a CMR request to the media sender to use the codec mode/configuration of the same codec that provides better media quality

Robustness adaptation without application layer redundancy can involve switching to a codec mode/configuration with partial-redundancy (e.g., EVS Channel Aware mode for speech), better error correction, or lower bit rate.  In the last scenario this can benefit the resulting media quality if the terminal is near the edge of coverage and the link can better support transport of lower rate codec modes/configurations.

## X.3	Adaptation to Packet Losses using Application Layer Redundancy

An MTSI terminal supporting the CHEM feature supporting use of application layer redundancy to improve robustness for a media type (e.g., speech or video) shall support the following procedures:

- when sending an SDP offer, the MTSI client shall include the ALR parameter in the PLR_adapt attribute in the media description for that media type in the SDP offer

- when sending an SDP answer, the MTSI client shall include the ALR parameter in the PLR attribute in the media description for that media type in the SDP answer only if the ALR parameter in the PLR_adapt attribute was received in an SDP offer

In addition to requesting the media sender to adapt codec mode/configuration as described in clause X.2, the MTSI terminal may request the media sender to use non-zero application layer redundancy to improve robustness to packet loss.  When using CMR to request application layer redundancy the following CMR code points shall be used only if the MTSI client receives the ALR parameter in the PLR_adapt attribute in either an SDP offer or SDP answer:

Table X.3.1: Code points for AMR

| CMR code | Application layer redundancy AMR request |
| --- | --- |
| 9 | RED 2x4.75 |
| 10 | RED 2x5.15 |
| 11 | RED 2x5.9 |
| 12 | Not used |
| 13 | Not used |
| 14 | Not used |

Table X.3.2: Code points for AMR-WB

| CMR code | Application layer redundancy AMR-WB request |
| --- | --- |
| 9 | RED 2x6.6 |
| 10 | RED 2x8.85 |
| 11 | RED 2x12.65 |
| 12 | Not used |
| 13 | Not used |
| 14 | Not used |

Table X.3.3: Code points for EVS

| CMR code | Application layer redundancy EVS request |
| --- | --- |
| 111 0000 | RED 2x7.2-NB |
| 111 0001 | RED 2x8-NB |
| 111 0010 | RED 2x9.6-NB |
| 111 0011 | RED 2x13.2-NB |
| 111 0100 | RED 2x7.2-WB |
| 111 0101 | RED 2x8-WB |
| 111 0110 | RED 2x9.6-WB |
| 111 0111 | RED 2x13.2-WB |
| 111 1000 | RED 2x13.2 CAM WB |
| 111 1001 | RED 2x13.2 CAM SWB |
| 111 1010 | RED 2x9.6-SWB |
| 111 1011 | RED 2x13.2-SWB |
| 111 1100 | RED 2x6.6-IO |
| 111 1101 | RED 2x8.85-IO |
| 111 1110 | RED 2x12.65-IO |

To enable use of application layer redundancy the max-red SDP parameter should be selected in the SDP offer and answer to give at least some room for application layer redundancy adaptation and in accordance with clauses 6.2.2.2 and 6.2.2.3.  The recommended redundancy offset for application layer redundancy should be 3 frames.

## X.4 Negotiation of End-to-End and Uplink/Downlink PLR

### X.4.1 General

In addition to the negotiated codecs and codec modes, the end-to-end quality and robustness to packet loss of a service can depend on the MTSI client implementation of functions such as packet loss concealment (PLC) and de-jitter buffer management (JBM).

A set of end-to-end and link-by-link PLR parameters is specified to provide the following:

- enable MTSI clients to indicate for each RTP payload type the effective maximum tolerable PLR resulting from the codec, codec mode, packet loss concealment, de-jitter buffer management and other implementation considerations.

- enable MTSI clients to negotiate with each other and suggest to the PCRF/PCFs how the maximum tolerable end-to-end PLR budget can be distributed across the uplink and downlink in a media transport path.  In the absence of any additional negotation or information from the terminals, the PCRF/PCFs may default to distributing the maximum tolerable end-to-end PLR equally (50-50) across the uplink and downlink.  The link-specific PLR parameters enable the terminals to suggest to the PCRF/PCFs an unequal distribution, e.g. 60% of the maximum end-to-end PLR allowed on the uplink and 40% allowed on the downlink before the terminals are handed off.

- indicate to the PCRFs/PCFs what PLR values may be used on the local uplink and downlink to the MTSI clients.

### X.4.2 Offering MTSI Client

SDP PLR attributes and parameters are defined for use with each payload type.  For each RTP payload type, an offering MTSI client supporting the CHEM feature may include the following SDP attribute in its SDP offer:

Name: MAXimum-e2e-PLR(maximum end-to-end PLR of the media decoder in the  MTSI client)

Value: MAX-e2e-PLR-value

Usage Level: media

Charset Dependent: no

Syntax:

MAX-e2e-PLR-value = payload-type SP maxe2e-PLR [“:”maxDL-PLR] [“/”maxUL-PLR]

payload-type = zero-based-integer

maxe2e-PLR = plr-value

maxDL-PLR = plr-value

maxUL-PLR = plr-value

plr-value = integer

: integer taken from IETF RFC 4566

The maxe2e-PLR represents the maximum end-to-end packet loss rate that can be properly received by the media decoder in the offering MTSI client (including effects of codec mode, packet loss concealment, de-jitter buffering, etc…) indicated by the RTP payload type number payload-type (as used in an “m=” line).

The maxDL-PLR parameter indicates the maximum packet loss rate that the offering MTSI client is able to handle on its local downlink and shall not exceed the maxe2e-PLR value in the SDP offer.  If the parameter is not included then the default value from the offering MTSI client is set to ½ of the maxe2e-PLR value included in the SDP offer.

The maxUL-PLR parameter indicates the maximum packet loss rate that the offering MTSI client is able to handle on its local uplink.  If the parameter is not included then the default value from the offering MTSI client is set to ½ of  maxe2e-PLR value the answering MTSI client will include in its corresponding SDP answer.

If maxe2e-PLR is not included in the SDP answer then its value is set to a recommended value for the codec and mode (e.g., obtain value from Annex Y if available).

The plr-value represents 1/100th of a percent (i.e. 10E-4) of packet loss as an integer.

The following table X.4.2-1 summarizes the SDP offer parameters.

Table X.4.2-1 SDP offer attributes and parameters

| Parameter | Requirement | Default Value if not included in SDP offer | Limit | Limit Condition |
| --- | --- | --- | --- | --- |
| maxe2e-PLR | Included when attribute is supported | A recommended value for the codec and mode (e.g., obtain value from Annex Y if available) |  |  |
| maxDL-PLR | Optional | ½ maxe2e-PLR value in SDP offer | No greater than maxe2e-PLR value in SDP offer | Always |
| maxUL-PLR | Optional | ½ maxe2e-PLR value in SDP answer |  |  |

### X.4.3 Answering MTSI Client

For each RTP payload type, an answering MTSI client supporting the CHEM feature may include the SDP attribute from sub-clause X.4.2 in its SDP answer with the following semantics of the parameters.

The maxe2e-PLR represents the maximum end-to-end packet loss rate that can be properly received by the media decoder in the answering MTSI client (including effects of codec mode, packet loss concealment, de-jitter buffering, etc…) indicated by the RTP payload type number payload-type (as used in an “m=” line).

If maxe2e-PLR is not included in the SDP offer then its value is set to a recommended value for the codec and mode (e.g., obtain value from Annex Y if available).

The maxDL-PLR parameter included in the SDP answer indicates the maximum packet loss rate that the answering MTSI client is able to handle on its local downlink and shall not exceed the maxe2e-PLR value in the SDP answer.

If the maxUL-PLR value included in the SDP offer is no greater than ½ the maxe2e-PLR value included by the answering MTSI client in the SDP answer, then maxDL-PLR in the SDP answer shall be set to no greater than (maxe2e-PLR value included in the SDP answer minus maxUL-PLR value included in the SDP offer).  Otherwise (the maxUL-PLR value included in the SDP offer is greater than ½ the maxe2e-PLR value included in the SDP answer) the maxDL-PLR in the SDP answer should be set to no greater than (maxe2e-PLR value included in the SDP answer minus maxUL-PLR value included in the SDP offer).  If the answerer sets the maxDL-PLR in the SDP answer to be greater than (maxe2e-PLR value included in the SDP answer minus maxUL-PLR value included in the SDP offer), the answerer shall not set maxDL-PLR greater than ½ maxe2e-PLR value included in the SDP answer.

If the maxDL-PLR parameter is not included in the SDP answer then the default value from the answering MTSI client is set to ½ of the maxe2e-PLR value included in the SDP answer.

The maxUL-PLR parameter included in the SDP answer indicates the maximum packet loss rate that the answering MTSI client is able to handle on its local uplink and shall not exceed the maxe2e-PLR value in the SDP offer.

If the maxDL-PLR value included in the SDP offer is no greater than ½ the maxe2e-PLR value included by the offering MTSI client in the SDP offer, then maxUL-PLR in the SDP answer shall be set to no greater than (maxe2e-PLR value included in the SDP offer minus maxDL-PLR value included in the SDP offer).  Otherwise (if the maxDL-PLR value included in the SDP offer is greater than ½ the maxe2e-PLR value included in the SDP offer), ten maxUL-PLR in the SDP answer should be set to no greater than (maxe2e-PLR value included in the SDP offer minus maxDL-PLR value included in the SDP offer).  If the answerer sets the maxUL-PLR in the SDP answer to be greater than (maxe2e-PLR value included in the SDP offer minus maxDL-PLR value included in the SDP offer), the answerer shall not set maxUL-PLR greater than ½ maxe2e-PLR value included in the SDP offer.

If the maxUL-PLR parameter is not included then the default value from the answering MTSI client is set to ½ of the maxe2e-PLR value included in the SDP offer.

The plr-value represents 1/100th of a percent (i.e. 10E-4) of packet loss as an integer.

The table X.4.3-1 below summarizes the usage of the SDP answer attributes and parameters.

Table X.4.3-1 Usage of the SDP answer attributes and parameters

| Attribute/ Parameter | Requirement | Default Value if not included in SDP Answer | Limit Condition | Limit |
| --- | --- | --- | --- | --- |
| maxe2e-PLR | Included when attribute is supported | A recommended value for the codec and mode (e.g., obtain value from Annex Y if available) |  |  |
| maxDL-PLR | Optional | maxe2e-PLR value included in the SDP answer minus maxUL-PLR value included in the SDP offer. If maxUL-PLR is not included in the SDP offer then the default value is ½ maxe2e-PLR value in the SDP answer. | maxUL-PLR value included in the SDP offer is no greater than ½ maxe2e-PLR value included in the SDP answer | maxe2e-PLR value included in the SDP answer minus maxUL-PLR value included in the SDP offer |
|  |  |  | maxUL-PLR value included in the SDP offer is greater than ½ maxe2e-PLR value included in the SDP answer | ½ maxe2e-PLR value in the SDP answer |
| maxUL-PLR | Optional | maxe2e-PLR value included in the SDP offer minus maxDL-PLR value included in the SDP offer. If maxDL-PLR is not included in the SDP offer then the default value is ½ maxe2e-PLR value in the SDP offer. | maxDL-PLR value included in the SDP offer is no greater than ½ maxe2e-PLR value included in the SDP offer | maxe2e-PLR value included in the SDP offer minus maxDL-PLR value included in the SDP offer |
|  |  |  | maxDL-PLR value included in the SDP offer is greater than ½ maxe2e-PLR value included in the SDP offer | ½ maxe2e-PLR value in the SDP offer |

### ==== Change #5 ====

# Annex Y (Informative): Example Maximum Packet Loss Rate (Max. PLR) Values for Setting CHEM Handover Thresholds

## Y.1	Maximum Packet Loss Rate (Max. PLR) for Speech

Based on the 3GPP EVS Selection and Characterization results that included AMR-WB, AMR-WB with G718IO, and EVS codec, this clause provides an example set of Max. PLR operating points that the terminal may indicate to the PCRF/PCF.

### Y.1.1	Max. PLR recommendation without Application Layer Redundancy

Table Y.1 provides example Maximum PLR operating points based on the EVS Selection and Characterization experiment results in TR 26.952 [162] and TR 26.959 [163].

Table Y.1: Example Maximum End-to-end Packet Loss Rate (PLR) per link for AMR-WB and EVS

| Codec | Robustness Parameter | Maximum End-to-end Packet Loss Rate |
| --- | --- | --- |
| AMR-WB | Normal | 1.5% |
| AMR-WB/G718 IO, EVS AMR-WB IO | Medium | 3% |
| EVS WB, SWB | High | 6% |
| EVS WB, SWB Channel Aware | Extreme High | 9% |

### Y.1.2	Max. PLR recommendation with Application Layer Redundancy

Application layer redundancy can work in conjunction with any of the aforementioned codec modes in Table Y.1.

Table Y.2 provides example Maximum PLR operating points with and without application layer redundancy applicable to EVS codec based on informal objective and subjective results in Annex A of TR 26.959 [163].

The example in Table Y.2 includes 100% application layer redundancy with offset 2, resulting in (2 x Bitrate). It should be noted that the relationship to path loss when operating at twice the bit rate (with 100% application layer redundancy) is not accounted in the Max. PLR recommendation in Table Y.2.

Table Y.2: Example Max. End-to-end Packet Loss Rate (PLR) with application layer redundancy for EVS codec

| Codec | Robustness Parameter | Maximum End-to-end Packet Loss Rate |
| --- | --- | --- |
| No application layer redundancy, EVS at bitrate of R kb/s | - | X % |
| With 100% application layer redundancy, EVS at bitrate of 2xR kb/s, Offset = 2. | - | X + (2 to 5) % |