3GPP TSG-SA4 Meeting #105	S4-190844

- Ljubljana, Slovenia, 12th Aug 2019 - 16th Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0001 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of configuration updates when not streaming media data |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital CE Intermediate |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  |  | Date: |  |  | 2019-07-23 |
| Category: | D |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current text has some self-contradictory sentences. Descriptions of states in clauses 6.2.2 and 6.2.3 all say that "Configuration may be updated while receiving media data”, which has no clear meaning for pause and terminated states. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the text “while receiving media data” where no data can be received. Remove whole sentence in states “terminated” (no configuration update expected there). |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intention on when configuration changes should be possible is unclear, possibly misleading the protocols definitions in stage 3. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.3 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 is re-submission of CR with identical content as discussed during SA4#104 in Cork. |  |  |  |  |  |  |  |  |  |

- 6.2.2	Sink Configuration at the Media AF / AS

The procedure allows the configuration of an Uplink Media AS for media reception. A Sink Session is created and updated. One or more media sessions are associated with a sink session.

Figure 6.2.2-1: Sink Configuration states

The states describe the Sink Configuration and the relation to the media session:

-	Sink Configuration created, no media session(s) active: The Sink Configuration entry is created at the UL Media AF and may be updated / modified before any media session(s) start.

-	Sink Configuration Provisioned, media session(s) active: The UL Media AS is receiving UL media data. Some properties of the Sink Configuration may be updated while receiving media data.

-	Sink Configuration Provisioned, media session(s) paused: The UL Media AS is temporarily not receiving UL media data. Some properties of the Sink Configuration may be updated.

-	Sink Configuration Provisioned, media session(s) terminated: The UL Media AS is not receiving UL media data anymore.

- 6.2.3	Source Configuration at the UE

The procedure allows the (remote) configuration of UE for media transmission. When remote control is used, then the application authorizes remote control e.g. by providing the remote control server address. It is assumed, that a UE can have only a single Source Configuration at a time, which can be remotely configured. One or more media sessions are associated with the Source Configuration.

Figure 6.2.3-1: Source Configuration states

The states describe the Source Configuration and the relation to the media session.

-	Source Configuration created, no media session(s) active: The Source Configuration entry is created and may be updated / modified before any media session(s) start. When remote control is enabled, then the connection to the remote control function is established.

-	Source Configuration Provisioned, media session(s) active: The Media Upstream Client is sending UL media data. Some properties of the Source Configuration may be updated while sending media data.

-	Source Configuration Provisioned, media session(s) paused: The Media Upstream Client is temporarily not sending UL media data. Some properties of the Source Configuration may be updated.

-	Source Configuration Provisioned, media session(s) terminated: The Media Upstream Client is not receiving UL media data anymore.