3GPP TSG-SA WG4 Meeting #105 	S4-190848

# Ljubljana, Slovenia, 12 - 16 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.117 | CR | pseudo | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PSS-based and EVS Media Capabilities and Operation Points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 05/08/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[3]	3GPP TS 26.071: "Mandatory Speech Codec speech processing functions; AMR Speech CODEC; General description".

[4]	3GPP TS 26.090: "Mandatory Speech Codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Transcoding functions".

[5]	3GPP TS 26.073: "ANSI C code for the Adaptive Multi Rate (AMR) speech codec".

[6]	3GPP TS 26.104: "ANSI-C code for the floating-point Adaptive Multi Rate (AMR) speech codec".

[7]	3GPP TS 26.093: "Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Source controlled rate operation".

[8]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[9]	3GPP TS 26.190: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Transcoding functions".

[10]	3GPP TS 26.173: "ANCI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[11]	3GPP TS 26.204: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; ANSI-C code".

[12]	3GPP TS 26.193: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Source controlled rate operation".

[13]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[14]	3GPP TS 26.442: "Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point)".

[15]	3GPP TS 26.443: "Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)".

[16]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[17]	3GPP TS 26.446: "Codec for Enhanced Voice Services (EVS); AMR-WB Backward Compatible Functions".

[18]	3GPP TS 26.450: "Codec for Enhanced Voice Services (EVS); Discontinuous Transmission (DTX)".

[19]	3GPP TS 26.401: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; General description".

[20]	3GPP TS 26.402: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Additional decoder tools".

[21]	3GPP TS 26.403: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Encoder specification; Advanced Audio Coding (AAC) part".

[22]	3GPP TS 26.404: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Enhanced aacPlus encoder Spectral Band Replication (SBR) part".

[23]	3GPP TS 26.405: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Encoder specification parametric stereo part".

[24]	3GPP TS 26.410: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Floating-point ANSI-C code".

[25]	3GPP TS 26.411: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Fixed-point ANSI-C code".

=====  CHANGE  =====

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Bitstream: A media bitstream that conforms to an audio/speech encoding format and certain Operation Point.

Operation Point: A collection of discrete combinations of different content formats and the encoding format.

Receiver: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAC	Advanced Audio Coding

AMR	Adaptive MultiRate

EVS	Enhanced Voice Services

=====  CHANGE  =====

# 4	Overview

The speech/audio profile media capabilities defined in this clause are primarily introduced in order to be used as content format in the context of 5G Media Streaming, but not restricted to this use case. Parameters for audio encoder/decoder, content format and transport are defined.

This specification defines:

- Media decoding capabilities: the requirements for a receiver in terms of decoding

- Operation Points: A collection of discrete combinations of different content formats and the encoding format. Operation Points are supported by

  - Bitstream Requirements: A media bitstream that conforms to an audio/speech encoding format and certain Operation Point.

  - Receiver Requirements: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

- The integration of each Operation Point in 5G Media Streaming as defined in TS26.501 [2].

=====  CHANGE  =====

# 5	Media Decoding Capabilities

## 5.1	Introduction

This clause documents speech and audio media capabilities in terms of decoding capabilities.

## 5.2	Speech Media Decoding Capabilities

## The following speech media decoding capabilities are defined:

-	AMR: All decoding requirements for the AMR speech codec as specified in 3GPP TS 26.071 [3], 3GPP TS 26.090 [4], 3GPP TS 26.073 [5] and 3GPP TS 26.104 [6]) including all 8 modes and source-controlled rate operation ‎3GPP TS 26.093 [7].

-	AMR-WB: All decoding requirements for the AMR-WB codec as specified in 3GPP TS 26.171 ‎‎[8], 3GPP TS 26.190 ‎[9], 3GPP TS 26.173 ‎[10] and 3GPP TS 26.204 [11] including all 9 modes and source-controlled rate operation ‎3GPP TS 26.193 [12].

## -	EVS: All decoding requirements for the EVS codec as specified in 3GPP TS 26.441 [13], 3GPP TS 26.445 [16], 3GPP TS 26.442 [14] and 3GPP TS 26.443 [15] as described below including functions for backwards compatibility with AMR-WB (3GPP TS 26.446 [17]) and discontinuous transmission (3GPP TS 26.450 [18]).

## 5.3	Audio Media Decoding Capabilities

## The following audio media decoding capabilities are defined:

-	eAAC+: All decoding requirements for the AAC+ audio codec as specified in 3GPP TS 26.401 [19], 3GPP TS 26.402 [20], 3GPP TS 26.403 [21], 3GPP TS 26.404 [22], 3GPP TS 26.405 [23], 3GPP TS 26.410 [24] and 3GPP TS 26.411 [25].

=====  CHANGE  =====

## 6.1	Introduction

The speech/audio profile Operation Points defined in this clause are primarily introduced in order to be used as content format in the context of 5G Media Streaming, but not restricted to this use case.

An operation point is a combination of rendering formats and media decoding capabilities.

For each Operation Point, Bitstream and Receiver requirements are detailed in the remainder of clause 6.

Table 6.1 provides an overview of the Operation Points defined in the present document.

Table 6.1: Speech/Audio Operation Points

| Operation Point name | Format Properties | Decoding Capabilities | Reference |
| --- | --- | --- | --- |
| AMR speech | Sampling frequency: 8 kHz | AMR | 6.2.2 |
| AMR-WB speech | Sampling frequency: 16 kHz | AMR-WB | 6.2.3 |
| EVS mono | Sampling frequency: 8, 16, 32, 48 kHz | EVS | 6.2.4 |
| EVS dual-mono | Sampling frequency: 8, 16, 32, 48 kHz | Two times EVS | 6.2.5 |
| eAAC+ low rate stereo | Bitrates: up to 44 kbit/s | eAAC+ | 6.3.2 |
| eAAC+ high rate stereo | Bitrates: above 44 kbit/s | eAAC+ | 6.3.3 |

## 6.2	Speech Operation Points

### 6.2.1	Introduction

### This clause defines speech operation points. For each operation point, the requirements for the bitstream as well as for the receiver are defined.

### 6.2.2	AMR

#### 6.2.2.1	Bitstream Requirements

The following restrictions apply for the AMR Operation Point.

The speech bitstream shall conform to the bitstream requirements for the AMR speech codec as specified in 3GPP TS 26.071 [3], 3GPP TS 26.090 [4], 3GPP TS 26.073 [5] and 3GPP TS 26.104 [6]) including all 8 modes and source-controlled rate operation ‎3GPP TS 26.093 [7].

The sampling frequency shall be 8kHz.

The AMR Operation Point bitstream shall be encoded such that it can be decoded by the AMR decoding capabilities as defined in clause 5.2.

#### 6.2.2.2	Receiver Requirements

Receivers conforming to the AMR Operation Point shall support decoding and rendering AMR Bitstreams.

### 6.2.3	AMR-WB

#### 6.2.3.1	Bitstream Requirements

The following restrictions apply for the AMR-WB Operation Point.

The speech bitstream shall conform to the bitstream requirements for the AMR-WB speech codec as specified in 3GPP TS 26.171 ‎‎[8], 3GPP TS 26.190 ‎[9], 3GPP TS 26.173 ‎[10] and 3GPP TS 26.204 [11] including all 9 modes and source-controlled rate operation ‎3GPP TS 26.193 [12].

The sampling frequency shall be 16kHz.

#### The AMR-WB Operation Point bitstream shall be encoded such that it can be decoded by the AMR-WB decoding capabilities as defined in clause 5.2.

#### 6.2.3.2	Receiver Requirements

#### Receivers conforming to the AMR-WB Operation Point shall support decoding and rendering AMR-WB Bitstreams.

### 6.2.4	EVS Mono

#### 6.2.4.1	Bitstream Requirements

The following restrictions apply for the EVS-Mono Operation Point.

The speech bitstream shall conform to the bitstream requirements for the AMR-WB speech codec as specified in 3GPP TS 26.441 [13], 3GPP TS 26.445 [16], 3GPP TS 26.442 [14] and 3GPP TS 26.443 [15] as described below including functions for backwards compatibility with AMR-WB (3GPP TS 26.446 [17]) and discontinuous transmission (3GPP TS 26.450 [18]).

The sampling frequency shall be one of the following: 8, 16, 32, 48 kHz.

#### The EVS-Mono Operation Point bitstream shall be encoded such that it can be decoded by the EVS decoding capabilities as defined in clause 5.2.

#### 6.2.4.2	Receiver Requirements

#### Receivers conforming to the EVS-Mono Operation Point shall support decoding and rendering EVS-Mono Bitstreams.

### 6.2.5	EVS Dual-Mono

#### 6.2.5.1	Bitstream Requirements

The following restrictions apply for the EVS-Dual-Mono Operation Point.

The speech bitstream shall conform to the bitstream requirements for the AMR-WB speech codec as specified in 3GPP TS 26.441 [13], 3GPP TS 26.445 [16], 3GPP TS 26.442 [14] and 3GPP TS 26.443 [15] as described below including functions for backwards compatibility with AMR-WB (3GPP TS 26.446 [17]) and discontinuous transmission (3GPP TS 26.450 [18]).

The sampling frequency shall be one of the following: 8, 16, 32, 48 kHz.

#### The EVS-Dual-Mono Operation Point bitstream shall be encoded such that it can be decoded by two receivers, each conforming to the EVS decoding capabilities as defined in clause 5.2.

#### 6.2.5.2	Receiver Requirements

Receivers conforming to the EVS-Dual-Mono Operation Point shall support

- decoding and rendering EVS-Dual-Mono Bitstreams.

#### Decoding and rendering of two simulatenous EVS-Mono Bitstreams.

## 6.3	Audio Operation Points

### 6.3.1	Introduction

This clause defines audio operation points. For each operation point, the requirements for the bitstream as well as for the receiver are defined.

### 6.3.2	eAAC+ low rate stereo

#### 6.3.2.1	Bitstream Requirements

#### <tbd>

#### 6.3.2.2	Receiver Requirements

#### <tbd>

### 6.3.2	eAAC+ high rate stereo

#### 6.3.3.1	Bitstream Requirements

#### <tbd>

#### 6.3.3.2	Receiver Requirements

#### <tbd>