Source:	Dolby Laboratories, Inc.

Title:	On IVAS Reference Testing over Loudspeakers

Document for:	Discussion & Agreement

Agenda Item:	7.5

# Introduction

There is an ongoing discussion in this group regarding testing procedures for the IVAS codec and multiple proposals for generation of reference signals have been put forward (cf. [1][2]). Whilst it continues to be the view of the source that the primary listening instrument for IVAS codec output will be headphones [5], the importance of testing on room multi-speaker systems is acknowledged. In this document, the source discusses some of the difficulties associated with multi-speaker testing and proposes a practical approach that can facilitate testing with a reasonable amount of effort.

# Discussion

Whilst standards for room multi-speaker layouts for immersive audio (cf. [3]) provide idealized positioning of loudspeakers within a room for a variety of layouts, with the proliferation of self-contained consumer loudspeakers with wireless capability, it is highly likely that in the near future, most immersive speaker-based consumer audio experiences will take place over non-idealized (i.e., arbitrary) loudspeaker layouts. This situation represents an infinite variety of speaker layouts to be tested if this group were to evaluate the performance of IVAS in real-world usage on immersive loudspeaker systems. Since, as mentioned previously, it is also expected that the primary listening instrument will be headphones, it does not seem sensible (let alone feasible) to test performance on every possible layout.

Nevertheless, the problem remains that certain audio formats (scene-based, objects) cannot be listened to over loudspeakers without rendering to some target layout. This is a problem that was faced recently during the evaluation of VRStream [4] candidates and a practical solution was to use a common loudspeaker layout of 7.1.4 for testing. It is the view of the source that such a solution is also suitable for IVAS, as a 7.1.4 layout is common enough to be supported by many cross-check labs and offers a sufficient number of loudspeakers to be able to reproduce a high-quality reference spatial experience.

# Conclusion and proposal

In conclusion, it is proposed that:

- If loudspeaker testing is done, a 7.1.4 loudspeaker layout should be used as the common layout for evaluating IVAS performance over loudspeakers for scene-based and object input audio content. The support of rendering to 7.1.4 loudspeaker layouts should be recommended in the IVAS codec design constraints.

- The references used for testing be generated using a common, agreed-upon object loudspeaker renderer (e.g. VBAP) and HoA loudspeaker renderer.

- These reference renders be compared against the 7.1.4 output of the IVAS default renderer.

- To acknowledge that loudspeaker tests are generally not suitable for naïve-listener test methodologies.

# References

[1]	S4-190688: “On the Importance of the Reference for IVAS Testing”, Fraunhofer IIS

[2]	S4-190675: “Proposal for the Definition of Reference and CUT Conditions for IVAS Codec Reference Testing”, Dolby Laboratories, Inc.

[3]	Recommendation ITU-R BS.2051-2 (07/2018): “Advanced sound system for programme production”

[4] 	S4-180903: “Revised WID on Virtual Reality Profiles for Streaming Media”

[5] 	S4-190673: “On the Audio Rendering Instrument for IVAS Codec Selection”, Dolby Laboratories, Inc.