3GPP TSG-S4 Meeting #107	S4-200082

# Wroclaw, Poland,  20th Jan.  - 24th Jan. 2020						Revision of S4-AHI920

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 3 for Media Session Establishment; Interactions with policy control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 25 June 2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501 considers that the Media AF “may relay or initiate a request for different Policy or Charging Function (PCF) treatment or interact with other network functions”. This pCR addresses the according session interactions. The Media Session Handler can interact with the Media AF for different reasons (e.g. policy and charging function interaction, as identified in TR 26.891, Clause 6.4). A set of parameters is needed for the network assistance procedures. The number of parameters may depend on the use-case and the requested Network Assistance service. Note, the pCR starts with a general discussion, which should not be added to draft TS 26.512. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.4.2. Procedures for dynamic policy updates

Provisioning for Dynamic Policy usage is defined in section XX.

This procedure is used by a Media Session Handler to perform the resource management of dynamic policies via the M5d interface. A dynamic policy invocation contains a Policy Template alias, flow description(s), an 5GMS Application Provider identifier (ASP Id) and potentially other parameters, according to TS 26.501, Clause 5.7.

A Policy Template alias identifies the desired policy template, which should be applied to an application flow such as specific QoS, background data or different charging treatments. The 5GMSd AF combines the information from the Policy Template with the dynamic information from the Media Session Handler to identify the N33 or N5 API call and the needed parameters. The Policy Template may contain for example the AF identifier, which is an important parameter for N33 and N5 API call. Example values for a Policy template alias are defined in Clause [].

The flow description(s) allows the identification and classification of the media traffic, such as the packet filter sets [TS 23.501 Clause 5.7.6].

The Media Session Handler creates a new dynamic policy instance by sending an HTTP POST message to the 5GMSd AF. The body of the HTTP POST message shall include 5GMSd Application Service Configuration Id, the Policy Template alias and the Flow Description. The Flow description identifies the actual application flows, which should be policed according to the policy Template. The 5GMSd AF creates a new Resource URL for the dynamic policy instance. The 5GMSd AF shall respond to the Media Session Handler with a 201 Created message, including the URL for the dynamic policy instance resource within the Location header field.

The Media Session Handler can modify the data of a dynamic policy resource using either the HTTP PUT or PATCH methods.

The Media Session Handler can delete a dynamic policy resource using the HTTP DELETE methods.

The 5GMSd AF shall trigger the appropriated actions towards other Network Functions like PCF or NEF.

Editor’s Note: Notification subscription will be added in the next version of the pCR.

**** Next Changes ****

## 5.x	DynamicPolicies API

### 5.x.1	Overview

The DynamicPolicies API is a RESTful API that allows the media session handler to request a specific policy and charging treatment. The API defines a set of data models, resources and the related procedures for the creation and management of the dynamic policy request. The corresponding JSON schema for the representation of the resources and operations defined by the API is provided in its complete form in Annex ZZ.

### 5.x.2	Data model

#### 5.x.2.1	Resource data types

##### 5.x.2.1.1	Type: DynamicPolicies

Table 5.x.2.1.1-1: Definition of type DynamicPolicies

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- |

##### 5.x.2.1.3	Type: DynamicPolicy

Table 5.x.2.1.3-1: Definition of type DynamicPolicy

| Attribute name | Data type |  | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| policyTemplateAlias | String |  | 1 | Identified the Policy template, which should be applied to the application flow(s) |  |
| flowDescription | Object | 1 | Refer to subclause 5.3.8 of 3GPP TS 29.214 for encoding |  |
| 5GMSd Application Service Configuration Id | String | 1 | Uniquely Identifies 5GMSd Application Serivice Configuration, which is linked to the Application Service Provider. |  |

##### 5.x.2.1.4	Type: DynamicPolicyReply

Table 5.x.2.1.3-1: Definition of type DynamicPolicyReply

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- |
| policyTemplateAlias | String | 1 |  |  |
| flowDescription | Object | 1 | Refer to subclause 5.3.8 of 3GPP TS 29.214 for encoding |  |
| 5GMS Application Service Configuration Id | String | 1 |  |  |
| enforcementMethod | String | 1 | Description of the Policy Enforcement Method |  |

### 5.x.3	Resource structure

#### 5.x.3.1	General

All resource URIs of this API should have the following root:

- {apiRoot}/3gpp-dynamicpolicies/v1/

All resource URIs in the subclauses below are defined relative to the above root URI.

The following resources and HTTP methods are supported for this API:

Table 5.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Meaning |
| --- | --- | --- | --- |
| Dynamic Policies | 3gpp-dynamicpolicies/v1/policies | GET | Forbidden. Should always return an error code. |
|  |  | POST | Create a new “Dynamic Policy” Resource |
| Dynamic Policy | 3gpp-dynamicpolicies/v1/policies/{policyId} | GET | Read the Dynamic Policy Resource |
|  |  | PUT | Replace an existing Dynamic Policy Resource |
|  |  | PATCH | Modify an existing Dynamic Policy Resource |
|  |  | DELETE | Delete an existing Dynamic Policy Resource |

#### 5.x.3.3	Resource: Dynamic Policies

##### 5.x.3.3.1	Introduction

<some Intro>

##### 5.x.3.3.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/policies/

This resource shall support the resource URI variables defined in Table 5.x.3.3.2-1.

Table 5.x.3.2.2-1: Resource URL variables for resource "Dynamic Policies"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122. |

##### 5.x.3.3.3	Resource Method GET

The 5GMSd AF shall return an Error code.

##### 5.x.3.2.4	Resource Method POST

The POST method creates a new Dynamic Policy resource for a given Media Session Handler. The Media Session Handler shall initiate the HTTP POST request message and the Media AF shall respond to the message. The 5GMSd AF shall construct the URI of the created resource using that URI.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the Table 5.x.3.2.4-1 and Table 5.x.3.2.4-2.

Table 5.x.3.2.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

Table 5.x.3.2. 4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | ApplicationInstance | 1 | Parameters to create a application instance. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  |  |  | 201 Created | The application instance resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table xxx also apply. |  |  |  |  |

#### 5.x.3.4	Resource: Dynamic Policy

##### 5.x.3.4.1	Introduction

<some Intro>

##### 5.x.3.4.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/policies/{policyId}

This resource shall support the resource URI variables defined in Table 5.x.3.3.2-1.

Table 5.x.3.2.2-1: Resource URL variables for resource "Dynamic Policy"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122. |
| {policyId} | Unique identifier, created by the Media AF during the create procedure (5.x.3.3.4). |

##### 5.x.3.4.3	Resource Method GET

##### 5.x.3.4.4	Resource Method PUT

##### 5.x.3.4.5	Resource Method PATCH

##### 5.x.3.4.6	Resource Method DELETE