Source: 	Nokia Corporation

Title: 	Use of Entrance Pupil Coefficients in Camera Calibration in ITT4RT and E-FLUS

Document for	Agreement

Agenda item: 	11

# Introduction

This proposal applies to both ITT4RT and E-FLUS.

In network-based stitching, different 2D captures are performed by different cameras (i.e., lenses) that may have been calibrated differently. Therefore, in light of improving video quality of the stitched output video, there is a benefit in harmonizing the camera parameters so that the 2D videos can be properly aligned prior to the 360 video production at the conferencing server.

Most cameras are affected by optical distortion due to the use of lenses, manufacturing defects, as well as cheap designs.  As there exist is a variety of lens distortion models (e.g radial, tangential, thin prism etc), it becomes complicated to harmonize separate distortion models from various devices. We provide the lens calibration technique based on Entrance Pupil which has already been adopted by the MPEG OMAF standard [1] for signaling the necessary parameters in order to facilitate network-based stitching in the ITT4RT framework. We like to remind that the model and calibration technique are generic, which allows easy integration to any image formation model used in different cameras.

# Camera lenses and entrance pupil information

Camera calibration is very important for accurate modeling of lenses. The intrinsic parameters model the internal characteristics of lens and sensors, while the extrinsic parameters model the relative position and orientation in 3D world. As a result, any image formation operation such as stitching that involves the use of the any lenses need this parameter model for accurate transformation.

Fig. 1: Camera model with Entrance Pupil (EP)

The Single Viewpoint (SVP) which assumes that all incidence camera rays pass through the optical center c, is not necessarily true for some imaging devices, especially those with greater than 90-degree Field of View (FOV) lenses. Figure 1 depicts a Non-Single Viewpoint (NSVP)  where the incidence rays originating from the world points (P(1), P(2), P(3)) do not meet at the optical center  Oc. The incidence rays formed by all these viewpoints meet at off-axis shift (c(1), c(2), c(3)) from the Entrance Pupil (EP) denoted as c which is equivalent to the optical center Oc. Only the incidence ray that coincides with the optical axis meets at the EP.

As the incidence ray increases away from the optical axis, the off-axis shift value increases as well. For most optical lenses with narrow FOV, the varying EP is quite small, and thus can be ignored during the camera calibration. However, this is not the case for larger field of view lenses which are employed in network stitching. Therefore, a SVP assumption in most camera calibration implies a limiting constraint on the entrance pupil location, which impacts the calibration parameter sensitivity and correctness.

In the MPEG OMAF [1], an image formation model that integrates entrance pupil variation for fisheye lenses was accepted, and it works for every type of lenses with strong distortion. In the proposal, the EP correction is introduced to bring NSVP to an SVP set-up.  The method gives accurate entrance pupil value and other camera parameters estimates when applied on calibration images acquired using lenses with large FOV. In [3] there is a comparison of the accuracy of the model to existing calibration models using the mean reprojection error described in the paper.

# Technical proposal

Figure 2 shows an example from the Intel’s document [2].

Fig. 2: Fisheye camera parameters from [2].

The camera parameters described in the figure can be assumed as initial camera parameters and are all in mm metric. Thereafter, these parameters from each individual camera can be eventually corrected locally or globally in an iterative manner with the provided EP coefficients. Indeed, the outcome of the optimization step should provide a global optimal value for the camera parameters in the following data structure:

struct FisheyeInfo{ float centerx,

float centery,

float wideangle,

float rotateangle,

float radius,

float4 EP}

The proposal 4 floating point EP coefficients are those that will improve the stitching result and should be signaled during an ITT4RT session. For further information about the integration to state of the art generic camera calibration, please refer to [3].

# Proposed text for the ITT4RT Permanent Document (section 6.3)

The set of camera calibration parameters includes Entrance Pupil (EP) coefficients and can be signaled using a dedicated SDP attribute (e.g., a=3gpp-itt4rt-camera-calibration:). The syntax is generic and includes the generic camera calibration parameters along withg the four floating point EP coefficients. Whenever the pararameters are not used, they can be zeroed.

EP coefficients provide a more precise model of various lens characteristics and improve the conversion from a sample location of an active area in a lens decoded picture to sphere coordinates relative to the global coordinate axes.

Please note that the formulation of the distortion correction and the EP coefficients for any lens can be derived using the calibration provided in [3]. The EP coefficients are formulated using a polynomial, and added seamlessly on whatever lens projection type specified. The use of EP coefficents is especially encouraged for wide lens cameras, e.g., for fisheye omnidirectional video.

# Proposal for E-FLUS

The camera calibration EP parameters can also be used in the FLUS specification, by means of a dedicated attribute: a=3gpp-flus-camera-calibration: and using a similar syntax as above. If SA4 agrees to this way forward, Nokia will prepare a CR to be agreed at this meeting.

# References

W18865, Text of ISO/IEC CD 23090-2 2nd edition OMAF, MPEG #128 meetig, Geneva, Switzerland.

S4-191265, ITT4RT: Further Aspects on Network-based Stitching, Intel, Nokia Corporation, Qualcomm Incorporated.

Peter Fasogbon, Emre Aksu, “Calibration of fisheye camera using entrance pupil”, http://arxiv.org/abs/1907.01759, IEEE International Conference on Image Processing (ICIP), 2019.

Contact: Peter Fasogbon, Igor Curcio, Saba Ahsan, Emre Aksu,  Nokia Technologies, Finland. Emails: firstname.lastname@nokia.com.