## Source:	Fraunhofer IIS

## Title:	Stereo Performance Requirements Proposal for IVAS

## Document for:	Discussion & Agreement

## Agenda Item:	7.5

# 1	Introduction

The current IVAS Performance Requirements document (IVAS-3, [1]) still contains open points for stereo operation. This document discusses these open points and provides proposals on completion.

This contribution is a follow-up on AHEVS-501 [2], which was discussed during the EVS SWG Call #64.

# 2	Discussion

The source sees the following points on Stereo operation which are either open or unclear:

- DTX: The current draft just lists “as provided”. The source thinks this is not enough.

- FER: Concrete numbers for frame error rates are missing

- Content Category: The content is currently only described as “Certain correlated content”. The source is of the opinion that this is not precise enough. Further on, also exact details on uncorrelated content are missing

Further on, the source proposed to stick to the concept currently captured in the current draft, namely to compare IVAS Stereo operation against a) EVS dual-mono and b) in addition evaluate the quality of a mono compatible output by comparison against an EVS dual-mono downmix.

This should also apply to features such as DTX and PLC:

- The performance of IVAS PLC shall be evaluated by comparing IVAS against EVS dual mono at the frame error rate. Example: IVAS at 6% FER shall be compared against EVS dual mono at the same FER

- The performance of IVAS DTX shall be evaluated by comparing IVAS against EVS dual mono with DTX enabled for both codecs.

# DTX

As for EVS mono, also for IVAS Stereo operation DTX is considered as an important feature. Its importance is not only for saving bitrate, but also to indicate active/inactive frames, which is particularly useful e.g. in conferencing bridges to select or prioritize active over inactive participants. It was thus already included in the IVAS design constraints [3], and should thus also be reflected in the performance requirements. Since the indication of active/inactive participants is seen as an important feature across all bitrates, it is proposed to consider this aspect in the performance requirements, too.

  - FER

The current performance requirements draft lists simply “All” for FERs for Stereo operation. This is however considered as relatively vague. The source proposed instead to replace it by the same FERs as used in EVS, namely 0%FER (for testing the clean channel conditions) and 3%, 6% FER for testing impaired channel conditions.

For EVS, testing of those FERs has lead to a codec with high quality and robustness for both, clean and impaired channel conditions. Thus, the source sees no reason to deviate from these assumptions.

  - Content Category

The current Performance Requirement content only lists “Certain correlated content” in the content category. The source if of the opinion that this is not precise enough. It is thus proposed to a) extend this by listing the categories explicitly and b) also clarifying the terms.

Similar as for EVS, the following categories are proposed:

- Clean speech, i.e. speech recordings that do not contain background noise: This is considered as the predominant content for communication scenarios, also due to the noise-reduction typically implemented in the frontend-processing.

- Noisy speech, i.e. speech recordings that contain background noise: Similar as EVS and previous speech coding standards, it is further on expected that IVAS is also robust for speech over background noise, e.g. due to a high amount of background noise in the environment.

- Mixed content and music: There is an extremely high amount of (preproduced) mixed and music stereo content available, which could be used e.g. as holding tones, ring tones, in bridges, for streaming-applications etc. It is thus considered unavoidable to test the performance for this content category

It is proposed to stick to the same sound pressure levels as for EVS mono and also test various levels for clean speech, to make sure the codec exhibits equal performance also for different sound pressure levels.

Further on, it is proposed to limit the background noise types for noisy speech to car and office noise, and – compared to EVS mono – do not consider street noise: First, characteristics and performance of car and street noise were relatively similar during the EVS standardization. Second, the source considers use-cases with street noise background as relatively unlikely: Most relevant use-cases for IVAS Stereo operation are stereo telephony/conferencing use-cases. It is considered unlikely that these take place on the street. Office and car however are considered important environments for these use-cases.

In addition, it must be clarified that the expected performance gain of 2x EVS mono is only expected in case the content exhibits correlation between left and right channel. It is proposed to add a clarification on the term “Correlated (…) content”, namely “Correlated stereo content shall not consist of two independent sources put into left and right channel, with no or very little correlation or leakage between both channels.”

Further on, as already outlined at the last call, a corresponding table for Uncorrelated Content was added, following the high-level description “No other content shall be degraded with respect to dual mono operation (…)”.

  - Other

In addition, several other issues/missing pieces were found when reviewing the performance requirements once again:

- 13.2 kbit/s Stereo Operation: Stereo operation for 13.2 kbit/s SWB was missing. As evident from EVS deployment, 13.2 kbit/s SWB is one of the most used bitrates for EVS, and many mobile networks were designed for this operating point. It is also evident from other codecs such as MPEG-D USAC that good stereo performance is possible at this bitrate. The source consequently proposes to also mandate 13.2 kbit/s SWB operation for IVAS and add corresponding performance requirements

- Bandwidth for 9.6 kbit/s EVS reference: The current performance requirements proposal originates from EVS performance requirements, where no SWB operation was foreseen for 9.6 kbit/s. EVS however supports SWB for 9.6 kbit/s. It is thus proposed to reflect this also in the IVAS design constraints, i.e. use EVS 9.6 kbit/s SWB for IVAS SWB/FB operation instead of EVS 9.6 kbit/s WB.

# 3	Proposed Changes

*** Start change 1 ***

## 6.1 Stereo operation

IVAS stereo operation shall provide a benefit over the case when it is not included, i.e. mono operation.

High-level definition of stereo requirements

- For correlated stereo content (e.g. speech in conferencing use case, mixed/music), IVAS stereo operation shall be not worse than EVS dual-mono operation at (at least) next higher bit rate or better than EVS dual-mono operation at the same rate. Table 1 describes the details of these requirements. Correlated stereo content shall not consist of two independent sources put into left and right channel, with no or very little correlation or leakage between both channels.

- No other content shall be degraded with respect to dual mono operation. This includes two independent sources put into left and right channel, with no or very little correlation or leakage between both channels. Such content is later on referred to as “Uncorrelated Content”.

- The IVAS stereo operation with mono compatible decoding shall be not worse than the stereo to mono downmix of dual-mono at the same bit rate.

For stereo operation, the IVAS codec will be compared for information to the 3GPP audio codecs in characterization, taking into account complexity, delay, etc.

Table 1: Stereo requirements.

Dual mono is abbreviated with a “2x”. It is assumed that the IVAS codec and the EVS reference codec are running in the bandwidth mode as outlined in the BW column, if not otherwise specified. For testing of the requirements, DTX shall be activated synchronously in both, IVAS CuT and EVS reference codec. For testing of the requirements, frame errors shall be applied synchronously to both, IVAS CuT and EVS reference codec.

Note: for all bit rates / audio bandwidth with no requirement, the following applies: If provided it should be characterized; characterization results shall be used for the decision of inclusion

| Category | BW | Bitrate (kbit/s) | FER | DTX | Stereo Requirements | Mono compatible requirements |
| --- | --- | --- | --- | --- | --- | --- |
| Correlated Stereo Clean speech -26,-16,-36 dBov Correlated Stereo Noisy Speech (Car, Office) -26dBov Correlated Stereo Mixed content and Music single level | NB | 7.2 | x%=0%,3%, 6% |  |  |  |
|  |  | 8 |  |  |  |  |
|  |  | 9.6 |  |  |  |  |
|  |  | 13.2 |  |  |  |  |
|  | WB | 7.2 | x%=0%,3%, 6% | On/Off |  |  |
|  |  | 8 |  |  |  |  |
|  |  | 9.6 |  |  |  |  |
|  |  | 13.2 |  |  | NWT EVS @ 2x 8 kbps OR BT EVS @ 2x 7.2 kbps |  |
|  |  | 16.4 |  |  | NWT EVS @ 2x 9.6 kbps OR BT EVS @ 2x 8 kbps | NWT stereo downmix of EVS @ 2x 8 kbps |
|  |  | 24.4 |  |  | NWT EVS @ 2x 16.4 kbps OR BT EVS @ 2x 13.2 kbps | NWT stereo downmix of EVS @ 2x 13.2 kbps |
|  |  | 32 |  |  | NWT EVS @ 2x 24.4 kbps OR BT EVS @ 2x 16.4 kbps | NWT stereo downmix of EVS @ 2x 16.4 kbps |
|  |  | 48 |  |  | NWT EVS @ 2x 32 kbps OR BT EVS @ 2x 24.4 kbps | NWT stereo downmix of EVS @ 2x 24.4 kbps |
|  |  | 64 |  |  | NWT EVS @ 2x 48 kbps OR BT EVS @ 2x 32 kbps | NWT stereo downmix of EVS @ 2x 32 kbps |
|  |  | 96 |  |  | NWT EVS @ 2x 64 kbps OR BT EVS @ 2x 48 kbps | NWT stereo downmix of EVS @ 2x 48 kbps |
|  | SWB | 13.2 | x%=0%,3%, 6% | On/Off | NWT EVS-WB @ 2x 8 kbps OR BT EVS-WB @ 2x 7.2 kbps | NWT stereo downmix of EVS @ 2x 7.2 kbps |
|  | SWB FB | 16.4 | x%=0%,3%, 6% | On/Off | NWT EVS-SWB @ 2x 9.6 kbps OR BT EVS-WB @ 2x 8 kbps | NWT stereo downmix of EVS @ 2x 8 kbps |
|  |  | 24.4 |  |  | NWT EVS @ 2x 16.4 kbps OR BT EVS @ 2x 13.2 kbps | NWT stereo downmix of EVS @ 2x 13.2 kbps |
|  |  | 32 |  |  | NWT EVS @ 2x 24.4 kbps OR BT EVS @ 2x 16.4 kbps | NWT stereo downmix of EVS @ 2x 16.4 kbps |
|  |  | 48 |  |  | NWT EVS @ 2x 32 kbps OR BT EVS @ 2x 24.4 kbps | NWT stereo downmix of EVS @ 2x 24.4 kbps |
|  |  | 64 |  |  | NWT EVS @ 2x 48 kbps OR BT EVS @ 2x 32 kbps | NWT stereo downmix of EVS @ 2x 32 kbps |
|  |  | 96 |  |  | NWT EVS @ 2x 64 kbps OR BT EVS @ 2x 48 kbps | NWT stereo downmix of EVS @ 2x 48 kbps |
|  |  | 128 |  |  | NWT EVS @ 2x 96 kbps OR BT EVS @ 2x 64 kbps | NWT stereo downmix of EVS @ 2x 64 kbps |
| Uncorrelated Stereo Clean speech -26,-16,-36 dBov Uncorrelated Stereo Noisy Speech (Car, Office) -26dBov Uncorrelated Stereo Mixed content and Music single level | NB | 7.2 | x%=0%,3%, 6% |  |  |  |
|  |  | 8 |  |  |  |  |
|  |  | 9.6 |  |  |  |  |
|  |  | 13.2 |  |  |  |  |
|  | WB | 7.2 | x%=0%,3%, 6% | On/Off |  |  |
|  |  | 8 |  |  |  |  |
|  |  | 9.6 |  |  |  |  |
|  |  | 13.2 |  |  | NWT EVS @ 2x 7.2 kbps |  |
|  |  | 16.4 |  |  | NWT EVS @ 2x 8.0 kbps |  |
|  |  | 24.4 |  |  | NWT EVS @ 2x 13.2 kbps |  |
|  |  | 32 |  |  | NWT EVS @ 2x 16.4 kbps |  |
|  |  | 48 |  |  | NWT EVS @ 2x 24.4 kbps |  |
|  |  | 64 |  |  | NWT EVS @ 2x 32 kbps |  |
|  |  | 96 |  |  | NWT EVS @ 2x 48 kbps |  |
|  | SWB | 13.2 | x%=0%,3%, 6% | On/Off |  |  |
|  | SWB FB | 16.4 | x%=0%,3%, 6% | On/Off |  |  |
|  |  | 24.4 |  |  | NWT EVS @ 2x 13.2 kbps |  |
|  |  | 32 |  |  | NWT EVS @ 2x 16.4 kbps |  |
|  |  | 48 |  |  | NWT EVS @ 2x 24.4 kbps |  |
|  |  | 64 |  |  | NWT EVS @ 2x 32 kbps |  |
|  |  | 96 |  |  | NWT EVS @ 2x 48 kbps |  |
|  |  | 128 |  |  | NWT EVS @ 2x 64 kbps |  |

*** End of changes ***

# 4	Summary

Several clarifications and additions to the Stereo performance requirements were provided. It is proposed to accept the changes as outlined in Section 4 and incorporate them into the IVAS Performance Requirements document [1], namely on:

- Clarification of correlated and uncorrelated content

- Clarification on DTX and FER conditions

- Definition of content types for correlated and uncorrelated content

- Bandwidth of the 9.6 kbit/s EVS reference condition for SWB tests

- Inclusion of the 13.2 kbit/s IVAS condition for SWB

- Additional table for uncorrelated content which reflects the high-level requirements

- Removal of square-brackets, editors around the proposed Section 6.1

References:

[1] 	3GPP S4-191064, “IVAS Performance Requirements (IVAS-3)”, Version 0.0.4
[2]	3GPP AHEVS-501, Fraunhofer IIS, “Stereo Performance Requirements Proposal for IVAS”
[3]	3GPP S4-191307, “IVAS Design Constraints (IVAS-4)”, Version 0.2.0