3GPP TSG-WG SA4 Meeting #107	S4-200239

# Wroclaw, PL                                                                                                                        (revision of S4-200235)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev | 3 | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provisioning Interface Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | B |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The stage 3 specification for the Ingest interface is specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR specifies the Ingest interface for 5GMS3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The content ingest for 5GMS3 will be lacking. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[X]					DASH-IF, Specification of Live Media Ingest, https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

[Y]	Standard ECMA-262, 5.1 Edition / June 2011, ECMAScript Language Specification.

[Z]	IETF RFC 6234, US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF)

Second Change

### 4.2.X Procedures for M1d Provisioning

#### 4.2.X.1	Overview

A 5GMSd-aware Application Provider may use the procedures in this clause to provision the network for downlink media sessions that are operated by the application provider.  These sessions may be DASH streaming sessions, progressive download sessions, or any other type of media streaming or distrubtion (e.g. HLS).

The M1d offers three different sets of procedures:

- Media Ingest for CDN distribution: designed as an API that matches the functionality of public CDNs.

- Configuration for dynamic policy: allows the configuration of policy templates that can be applied to downlink sessions of the application provider.

- Configuration for reporting: permits the MNO to collect QoE and consumption reports about the downlink sessions.

A 5GMSd-aware Application Provider may use any of these procedures and in any combination it wishes, to support its downlink media sessions.

#### 4.2.X.2	Provisioning Procedures

Prior to configuring ingest, dynamic policies, or reporting, the Application Provider shall create a new Provisioning session. The following CRUD operations are used to manage a provisioning session.

4.2.X.2.1 Create Provisioning

4.2.X.2.2 Read Provisioning

4.2.X.2.3 Update Provisioning

4.2.X.2.4 Delete Provisioning

#### 4.2.X.3	Ingest Procedures

These procedures are used by the 5GMSd-aware Application Provider and the 5GMSd AF on M1d to provision downlink media ingest.

##### 4.2.X.3.1 Create Ingest

This procedure is used by the application provider to provision a new Ingest session. The application provider shall use the HTTP POST method for this purpose. Upon successful creation, the resource Id of the newly created Provisioning session shall be returned in the body of the reply.

If the ingest uses the Push method, i.e. the Pull attribute is set to False, then the Path and EntryPoint shall not be provided and are read-only. The TargetDomain is assigned by the Ingest Media AF and is read-only.

##### 4.2.X.3.2 Read Ingest Properties

This procedure is used by the application provider to obtain the properties of the ingest session from the Media AF. The application provider uses the GET method for this purpose.

##### 4.2.X.3.3 Update Ingest Properties

The update operation is invoked by the application provider to update the ingest session. All available parameters, except the Origin and Target domains, may be updated. The application provider shall use the HTTP PATH or HTTP PUT for the update operation.

##### 4.2.X.3.4 Delete Ingest

This operation is requested by the application provider to end the ingest session and the related distribution. The AF will release any associated resources, purge any cached data, and delete any corresponding configurations. The Media AF shall use the HTTP Delete method for this purpose.

#### 4.2.X.4	Policy Procedures

TBD.

#### 4.2.X.5	Reporting Procedures

TBD.

### 4.2.Y	M2d Procedures

#### 4.2.Y.1	Overview

These procedures are used by the 5GMSd-aware Application Provider and the 5GMSd AS on M2d to ingest content. These procedures depend on the selected Ingest Protocol and are described in section 5.X.4.

## 5.X	Ingest API

### 5.x.1	Overview

### 5.x.2	Data Model

The Ingest Object for media ingest is provided below:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| origin | Object | 1 .. 1 | Describes the origin that will be used to ingest the content for distribution. |
| name | string | 1 .. 1 | A name associated with this origin |
| path | string | 1 .. 1 | The relative path which will be used to address the media resources. This Path is provided by the Media AF in the case of Push. |
| pull | boolean | 1 .. 1 | Indicates whether to use Pull or Push for ingesting the content. |
| protocol | string | 1 .. 1 | Identifier of the ingest protocol. The set of supported protocols are defined in clause 5.4.2.3. |
| entryPoint | string |  | An entry point to ingest the content. The semantics of the entry point are dependent on the selected Protocol. In case of Push (Pull flag is set to False), this parameter is returned by the Provisioning AF to the content provider and provides the entry point for pushing the content. In case of Pull (Pull flag is set to True), the EntryPoint shall be provided by the origin to the Media AF to indicate the location from which content is to be pulled. The URL shall be used as the base URL, in the case of Pull. A request received by the Media AS is mapped to a URL using the provided base URL to fetch the content from the origin server. |
| cache | [Object] | 1 .. 1 | Defines the Cache configuration for the content of this Distribution. |
| pattern | string |  | A pattern that will be used to match the object’s URL to the Cache settings. The pattern shall be defined as a regular expression as specified by [Y]. |
| originCacheHeaders | boolean |  | Indicates whether to apply the cache directives for the object at ingest to the distribution. This is only applicable if the Ingest protocol is HTTP. |
| cacheDirectives | Object |  | If the cache directives are not inherited from the origin, this provides instructions on how to cache the content that matches the pattern. |
| no-cache | boolean |  | If set to true, this indicates that the objects that match this pattern are not be cached. |
| http-method-filter | [string] |  | The methods to which the Cache rules apply. The filter shall be provided as a regular expression as specified in [Y]. |
| status-code-filter | [integer] |  | The status codes to which the Cache rules apply. The filter shall be provided as a regular expression as specified in [Y]. |
| cdn-ttl | integer |  | The TTL value to set to the objects that match the pattern and the other filters. |
| configuration | Object | 1 .. 1 | Specifies the distribution method and configuration for the ingested content |
| contentPreparation | Object | 0..1 | Defines the requested content preparation. A set of templates for content preparation may be provided for selection. |
| mimeType | string |  | This field provides the MIME type of the content preparation description document. If the MIME type is not understood, the Media AF shall reject the provisioning request. |
| content | string |  | This field provides the content preparation description document. The document shall be Base64 encoded. |
| targetDomain | string | 1 | The target domain name, which will be used to access the content from the CDN. This will replace the domain name of the |
| originDomain | string | 1 | The origin domain that is redirected to the target domain. This domain name can be exposed to the UEs instead of the target domain. |
| geoFencing | Object | 0..1 | Limit access to the content to the indicated geographic areas. |
| urlSignature | Object | 0..1 | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content. |
| pattern | string |  | A pattern that shall be used to match the object’s URL for which URL signing is activated. An object of which the URL does not match this pattern will not be URL signed. The pattern shall be defined as a regular expression as specified by [Y]. |
| tokenName | string |  | The name of the query parameter that carries the token. |
| passphraseName | string |  | The name of the query parameter that carries the passphrase. |
| passphrase | string |  | The shared secret between the eMAS and the MAS for this Distribution. |
| ttlName | string |  | The name of the query parameter that carries the TTL field. |
| useIPAddress | boolean |  | If set to true, the IP address of the UE is matched to the IP address in the signature. Access to the object is only allowed when they match. |
| ipAddressName | string |  | The name of the query parameter that carries the IP address of the client. |
| certificateId | string |  | When content is distributed using TLS, the certificate for the Origin Domain is shared with the Media AF. This provides the id of the certificate to use. |

### 5.x.3	Resource structure

The Ingest and Distribution API is accessible through this URL paths:

{apiRoot}/3gpp-m1d/v1/provisioning/{provisioning-id}/

The following operations and the corresponding HTTP methods are supported:

| Operation | Resource URI | HTTP Method | Description |
| --- | --- | --- | --- |
| Create a new Ingest Session | ingest | POST | This is used to create a new Ingest resource. |
| Fetch an Ingest Session | ingest/{resource-id} | GET | This operation is used to read the Ingest configuration. |
| Update an Ingest Session | ingest/{resource-id} | PUT/PATCH | This operation is used to update the Ingest configuration. |
| Delete an Ingest Session | ingest/{resource-id} | DELETE | This operation is used to delete an existing Ingest. |
| Purge Cache | ingest/{resource-id}/purge | POST | This operation is used to invalidate some or all cached objects of this Ingest. |
| Fetch list of supported ingest protocols | protocols | GET | This operation is used to retrieve a list of supported Ingest Protocols. |
| Create a new Content Preparation Template | preparation-templates | POST | This operation is used to create a new Content Preparation Template that defines how the ingested content is to be prepared for ingest sessions. |
| Create a new Certificate | certificates | POST | This operation is used to add a new certificate that is used for the distribution of the content using TLS. |

### 5.X.4	Operations

#### 5.X.4.1	Overview

In this clause, we define the behaviour that is expected from the distribution when the Ingestion Configuration is defined. The main operations that are performed affect the caching and purging of cached content as well as the processing for media preparation and at the edge.

#### 5.X.4.2	Caching

The Provisioning Sessionmay specify caching rules that will be applied during the distribution of the objects. The distribution shall use the Pattern in the Cache object to determine which caching directives apply to that object. In case an object’s URL matches more than one Cache object, the first match shall apply. In case no Cache object is identified as a match, the Media AF shall apply default caching directives based on the object type. The OriginCacheHeaders flag may only be set to true if the Ingest protocol is based on HTTP and the Ingest mode is Pull. If the OriginCacheHeaders flag is set to true, then the CacheDirectives object shall not be present.

If a cache directive shall either indicate a “no-cache” request or a request for the CDN to cache it for “cdn-ttl” seconds. The “cdn-ttl” is applicable based on the time of ingest. For an HTTP-based ingest, this corresponds to the Date header field in the HTTP request/response that carries the object. At the time t_ingest + cdn-ttl, the object is considered as stale and should not be served from the cache. The Ingest Media AS shall compensate for any synchronization skew between the origin and its own clock. This can be for instance done by using the max-stale cache directive.

The cdn-ttl value may be signalled by the CDN using the Expires, max-age or s-maxage directives.

When distributing the object using HTTP, a “no-cache” request may be translated into a “no-cache” and “no-store” Cache-Control directive and/or a “max-age=0” Cache-Control directive.

By default, all origin HTTP header fields shall be assumed as not forwarded by the CDN, unless specified by the flag Origin CacheHeaders.

#### 5.X.4.3	Purging

The application provider may performs a Purge operation on a particular Provisioning Sessionto invalidate some or all cached objects of that Provisioning Session. The pattern parameter in the body of the request shall be provided and shall contain a regular expression that matches the URLs of the objects that the application provider wants to purge from the cache.

The Ingest Media AF shall immediately invalidate all matching objects in the CDN by declaring them as stale. Any request for a purged object will trigger the fetching and caching of the newer version from the origin in case of a Pull-based ingest. For Push-based Ingest, the request shall be responded to by a 404 HTTP response, unless a new version of the object is pushed by the origin.

#### 5.X.4.4	Processing

The ContentPreparation object shall include a content-type element that provides the MIME type of that document. If the format of the ContentPreparation document is not understood, the creation of the Ingest shall fail with error code 501. If the Ingest Media AF fails at provisioning the required resources based on the ContentPreparation document, it shall fail with a status code of 503.

#### 5.X.4.5	URL Signing

The URL signing procedure allows the content provider to prevent deep linking and unauthorized access to the data. It works by embedding a signature of a set of query parameters of the URL into the URL query part itself. The Media AS verifies the signature before allowing access to the distributed media data. Both the Media AS(s) used in the distribution and the origin use a shared secret that is encoded as part of the query parameter hash but not shared with the end client.

If UseIPAddress is set to True, then the signature and query part of the URL shall contain the client IP address. The parameter name shall then be provided by ipAddressName.

The shared secret shall be provided in the request as a string of length between 6 and 50 characters. The parameter name for the passphrase shall be provided by passphraseName.

The expiry time of the signed URL shall be indicated by the parameter with the name provided in ttlName.

The token shall be provided in the URL as a query parameter with the name given by tokenName. The token shall be the SHA512 encoding of the query part of the URL, including the passphrase as follows:

The query part shall be set as: configuration[ttlName]=ttl_value&configuration[ipAddressName]=client_ip_address

The token shall be calculated as:

token=SHA512(configuration[ttlName]=ttl_value&configuration[ipAddressName]=client_ip_address&configuration[‘passphraseName]=passphrase)

Finally, the final URL query part is composed as follows:

query= configuration[ttlName]=ttl_value&configuration[ipAddressName]=client_ip_address&configuration[tokenName]=token

The order of the parameters as used for the creation of the token shall be:

- TTL: expiry time of the signed URL

- ipAddress: if the UseIPAddress flag is set to true, otherwise it is not present

- passphrase: The shared passphrase

The Media AS shall use the SHA512 as defined in [Z] as the hash algorithm for creating the token.

#### 5.X.4.6	 Geofencing

The Application Provider may wish to limit the access to its media content to certain areas or to non-roaming UEs. Geofencing is used to configure the zone from which content is accessible.

### 5.X.5	Media Ingest Interface

This specification defines a single DASH Ingest protocol, the “dash-if-ingest” as specified by [8]. The Pull flag of the Origin element shall be set to false, indicating that a Push-based protocol is used. The entry point element shall be set to the URL that will be used to upload the DASH segments and MPD. The entry point URL shall not contain a path. The path for the URL is provided by the Path element.