3GPP TSG-SA4 Meeting #107	S4-200330

# Wroclaw, Poland,  Jan 20 - 24, 2020                                               revision of S4-200297

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0009 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Metrics collection parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2020-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The metrics collection configuration parameters are not clearly specified for the different configuration options. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct some text in the table in clause 4.2.3. - Add a table with parameters and options in clause 5.5 - Add sub-clauses to 5.5 keep consistency with other functions in 26.501 - Clarify text for step 7 in second figure in clause 5.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear which metrics parameters that are valid for different configuration options. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ================ First change =================

### 4.2.3	Service Access Information

The Service Access Information is the set of parameters and addresses, which are needed by the 5GMSd Client to activate the reception of a downlink streaming session.

The Service Access Information may be provided together with other service announcement information using M8d. Alternatively, the 5GMSd Client fetches the Service Access Information from the 5GMSd AF.  The Service Access Information contains at least the baseline information. Additional information is added, depending on offered features.

Table 4.2.3-1: Parameters of baseline service access information

| Parameters | Description |
| --- | --- |
| Media player entry | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| URL signing information | …. |

When consumption reporting is activated, then the parameters from Table 4.2.3-2 are present.

Table 4.2.3-2: Parameters for consumption reporting configuration

| Parameters | Description |
| --- | --- |
| Report interval | Identify the interval when the consumption report messages are sent by the 5GMS Media Session Handler. |
| Server address | A list of 5GMSd AF addresses where the consumption reporting messages are sent by the 5GMS Media Session Handler. |
| Location reporting | Identify whether the 5GMS Media Session Handler provides the location data to the Media AF (in case of MNO or trusted third parties) |

Table 4.2.3-3: Parameters for dynamic policy invocation configuration

| Parameters | Description |
| --- | --- |
| Server address | A list of 5GMSd AF addresses which offer the APIs for dynamic policy invocation sent by the 5GMS Media Session Handler. |
| 5GMSA Provider Id | 5GMSA Provider Id, which should be used for this application (only present, when the value is not hard-coded into the application). |
| Policy type indication | A list of policy type indication values, which the 5GMSd Client is authorized to use. |

Table 4.2.3-4: Parameters for metrics configuration

| Parameters | Description |
| --- | --- |
| Server address | A list of 5GMSd AF addresses to which metric reports shall be sent. |
| DNN | The DNN which shall be used when sending metrics report. If not specified, the default DNN shall be used. |
| Reporting interval | The sending interval between metrics reports. If not specified, a single final report shall be sent after the streaming session has ended. |
| Sample percentage | The percentage of streaming sessions that shall report metrics. If not specified, reports shall be sent for all sessions. |
| Streaming source filter | A list of URL patterns for which metrics reporting shall be done. If not specified, reporting shall be done for all sessions. |
| Metrics | A list of metrics which shall be reported. |

Table 4.2.3-5: Parameters for Network Assistance configuration

FFS

## ================ Next change =================

## 5.5	Metrics collection and reporting

### 5.5.1	General

Metrics collection and reporting can be done in different ways, depending on the relation between the Service Provider and the Network Operator. The following clauses shows simplified signalling examples for two different use-cases.

### 5.5.2	Out-of-band-controlled reporting

In the first use-case, shown in figure 5.5.2-1 below, the MNO is controlling the metric reporting, using the out-of-band configuration and reporting option. In this case the metrics are configured via the RAN and the 5G control plane.

Figure 5.5.2-1: Metrics collection using the out-of-band 5G control plane option

The different steps are explained below:

1: Overall metrics configuration is done on the network level, for instance defining which geographical areas that shall have metrics collection active, and how often metrics shall be reported.

2: The metric configuration(s) is sent from the OAM to the RAN, which does not forward that information to the UE at this stage.

3: Time passes, and it assumes that the UE moves around during that period.

4: The UE enters an area (cell, location area, etc.,) which is inside the geographical constraint. This is discovered by the RAN, and it now needs to activate metrics reporting for the UE.

5: The actual metric configuration is sent from the RAN to the metrics handler function in the media session handler, via the 5G control plane.

6: Additional time passes, and the UE has a metrics configuration, but no streaming session has started.

7: A streaming session is started.

8: The session setup is done in conjunction with signalling transactions (not shown here).

9: A new metrics job is created in the media player.

10: A reference to the new metrics job is returned.

11: The configuration for the metrics collection job is sent to the media player (i.e. which metrics should be measured).

12: Media is delivered and rendered, and...

13: ...more media is delivered...

14: The configured metrics reporting interval has passed, and the metrics handler now requests the collected metrics from the media player.

15: The media player returns the collected metrics.

16: The metrics are reported via the 5G control plane.

17: The session continues...

18: more media is delivered, and then the session is finished.

19: The metrics handler requests the final metrics collected.

20: The media player returns the final collected metrics.

21: The metrics are reported to the OAM via the 5G control plane.

22: The metrics collection job is deleted.

23: Time passes, the UE moves around.

24: The UE leaves the geographical area specified by the metrics configuration.

25: RAN sends metrics (de)configuration to the UE, to stop future metrics collection.

### 5.5.3	In-band-controlled reporting

The second use-case, shown in figure 5.5.3-1 below, illustrates a scenario where the metrics collection is configured in-band. Thus together with the media there are metadata which describe how metrics shall be collected, and how they shall be reported. The Media OAM and Media /AS functions can both be either trusted or untrusted (but not mixed).

Figure 5.5.3-1: Metrics collection using the in-band metadata option

The different steps are explained below:

1: The content servers are set up, including any metrics configuration.

2: If the Media OAM and 5GMSd AS are located in a trusted domain, metadata regarding the metrics collection and reporting are sent to the AS. The procedure for configuring the 5GMSd AS, when the Media OAM and 5GMSd AS reside in an untrusted domain, is unspecified.

3: Time passes during which no streaming session is active...

4: A streaming session starts.

5: The App and the session handler initializes a new session.

6: The media player requests any metadata, from the 5GMSd AS, which is needed for downloading the media content.

7: Metadata is returned to the media player  and also forwarded to the session handler and (potentially) to the App.

8: A new metrics job is created in the media player.

9: A reference to the new metrics job is returned.

10: Based on the metadata, the configuration for the metrics collection job is sent to the media player (i.e. which metrics should be measured).

11: The streaming session is running.

12: Media is delivered and rendered, and...

13: more media is delivered...

14: The configured metrics reporting interval has passed, and the metrics handler now requests the collected metrics from the media player.

15: The media player returns the collected metrics.

16: The metrics are reported according to the reporting method defined in the configuration, for instance HTTP upload.

17: The session continues...

18: more media is delivered, and then the session is finished.

19: The metrics handler requests the final metrics collected.

20: The media player returns the final collected metrics.

21: The metrics are reported.

22: The metrics collection job is deleted.

### 5.5.4	Metrics collection configuration parameters

Table 4.2.3-4 in clause 4.2.3 describes the metrics collection configuration parameters used in step 5 in Figure 5.5.2-1 and step 7 in Figure 5.5.3-1. Note that some of the parameters are only relevant for a specific reporting option, as shown in table 5.5.4-1 below.

Table 5.5.4-1: Metrics collection configuration parameters and options

| Parameters | Out-of-band-controlled reporting | In-band- controlled reporting |
| --- | --- | --- |
| Server address |  | X |
| DNN |  | X |
| Reporting interval | X | X |
| Sample percentage | X | X |
| Streaming source filter | X |  |
| Metrics | X | X |

## ================ End of changes =================