3GPP TSG-SA4 Meeting #108-e	S4-200504

# 2nd – 9th April, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.131 | CR | 0081 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying nature of headset requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Testing organizations raised concerns on how conformance testing should be performed either with the headset supplied with the device or the manufacturers default headset (GCF explains in S4-200537). - Further questions were raised as to how test cases were validated when testing results could differ depending on a number of factors such as the positioning methods and type of headsets that might not be same as those used for validation. - Many manufacturers do not supply wired headsets with their UEs and many consumers use their own headsets rather than the ‘out of the box’ headsets. - With the variety of headsets currently bundled with the UEs or used by consumers, digital/analog, wired/wireless, or earphone/earpiece, what a ‘headset’ means, as stated as “an apparatus for telephony comprising essentially a “hands-free” handset which is typically secured to the head of the wearer” in ITU-T P.380, is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is clarifed that the requirements for headsets are for measuring performance (with a UE and certain headsets), rather than testing conformance (with a UE and any headsets). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The implementers and operators find difficulty in the testing of the UEs and headsets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start change 1 ***

4	Interfaces

The interfaces required to define terminal acoustic characteristics are shown in TS 26.132. These are the air interface and the point of interconnect (POI). The interfaces are shown for one-channel (mono) operation, interfaces for two-channel (stereo) operation is for further study.

The Air Interfaces for GSM, 3G, LTE and NR are specified by GSM 05, 3GPP 45, 3GPP 25, 3GPP 36 and 3GPP 38 series specifications, and the Air Interface for WLAN access to EPC is specified by WLAN access to EPC as defined in TS 23.402 [19] and TS 24.302 [20]. MTSI speech aspects are specified by TS 26.114 [17].

Measurements can be made using the system simulator (SS) described in TS 26.132.

The POI with the public switched telephone network (PSTN) is considered to have a relative level of 0 dBr.

Five classes of acoustic interface are considered in this specification:

-	Handset UE including softphone UE used as a handset;

-	Headset UE including softphone UE used with headset;

-	Desktop-mounted hands-free UE including softphone UE with external loudspeaker(s) used in hands-free mode;

-	Vehicle-mounted hands-free UE including softphone UE mounted in a vehicle;

-	Hand-held hands-free UE including softphone UE with internal loudspeaker(s) used in hands-free mode.

(See definition of softphone in Clause 3.1)

NOTE 1:	The requirements and performance objectives for a softphone UE shall be derived according to the following rules:

-	When using a softphone UE as a handset: requirements and performance objectives shall correspond to handset mode.

-	When using a softphone UE with headset: requirements and performance objectives shall correspond to headset mode.

-	When a softphone UE is mounted in a vehicle: requirements and performance objectives shall correspond to vehicle-mounted handsfree mode.

-	When using a softphone UE in hands-free mode:

-	When using internal loudspeaker(s), requirements and performance objectives shall correspond to hand-held hands-free.

-	When using external loudspeaker(s), requirements and performance objectives shall correspond to desktop-mounted hands-free.

NOTE 2:	The requirements for headsets are for measuring performance, rather than testing conformance.

*** End change 1 ***