3GPP TSG-WG SA4 Meeting # 108-e	S4-200542

# E-Meeting, 6th -10th April 2020

| CR-For v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev | 0 | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dynamic Policy Configuration Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The external application provider configures a set of dyamic policy templates that may be applied to the streaming sessions offered by it. It defines these as a set of operation points for the service, each of which with corresponding QoS requirements and charging configurations. These policies are also shared with the Session Handler on the UE to assist the client with the selection of the proper operation point. This CR defines the Policy Template resource and the operations that are performed over the M1d/M1u. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines the policy template configuration API that is offered for the provisioning of downlink and uplink streaming session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 solution will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# First Change

### 4.2.4	Dynamic Policy Configuration procedures

### 4.2.4.1	General

These procedures are used by the 5GMSd Application Provider to configure the policy templates for streaming sessions of this provisioning session.

### 4.2.4.2	Create Policy Template

This procedure is used by the 5GMSd/5GMSu Application Provider to create a new Policy Template. The Application provider shall use the HTTP POST method for this purpose.

Since policy templates require MNO verification, a policy template that is newly created may not be used immediately. Upon creation, a policy template shall be in state “unverified”. Once it has been validated by the MNO, it takes the “ready” state, indicating that it can be used by the associated streaming sessions. Finally, a policy template may be “suspended” by the MNO, e.g. in case of a violation of the usage terms or for some other reasons, which renders it unusable. The 5GMSd/5GMSu AF shall verify the status of a policy template prior to using it.

If the procedure is successful, the 5GMSd/5GMSu AF shall generate a resource id to uniquely identify the newly created Policy Template. In that case, it shall respond with a 201 (Created) HTTP response message and provide the URL to the newly created resource in the Location header field.

### 4.2.4.3	Read Policy Template

This procedure is used by the 5GMSd/5GMSu Application Provider and other 5GMSd/5GMSu AFs to query the properties of an existing Policy Template resource from the 5GMSd/5GMSu AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response that includes the Policy Template in the response message body.

### 4.2.4.4	Update Policy Template

The update operation is invoked by the 5GMSd/5GMSu Application Provider to modify the properties of an existing Policy Template. All available parameters except the state may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

Note that the update may potentially cause the change of the Policy Template to change into “unverified”, which makes it temporarily unusable.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message that includes the Policy Template in the response message body.

### 4.2.4.5	Delete Policy Template

This operation is used by the 5GMSd/5GMSu Application Provider to destroy a Policy Template resource that is owned by its provisioning session. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd/5GMSu AF will remove the policy template from the provisioning session and any currently active streaming sessions using the policy template will be migrated to a different policy template of the same provisioning session.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message.

# Second Change

### 5.5	Policy Template API

### 5.5.1 Overview

The 5GS offers network assistance to multimedia sessions for which provisioning has been performed using the M1d by the content provider with the Provisioning 5GMSd AF. A new media streaming session is detected by the Session Handler, which in turn will intiate a network assistance session with the Assistance 5GMS AF over the M5d interface.

A Policy Template (PT) provides a description of a set of operation points and their respective QoS and charging requirements. The PT is configured as part of the Provisioning procedures with the 5GMS AF and information about the configured policy templates are shared with the Network Assistance 5GMS AFs.

### 5.5.2	Data Model

The PT resource is defined according to the following table:

| Parameter | type | Description |
| --- | --- | --- |
| policyId | Integer | Provides an MNO-wide unique Identifier of the policy template. |
| state | Enumeration of strings | A policy template may be in “ready, “unverified”, or “suspended” state. Only a policy template in “ready” state may be used for streaming sessions of the provisioning session. |
| OperationPoints | Array of Objects |  |
| OPId | Integer | Provides an indentifier for the current operation point |
| QoS | Object | This object providesthe QoS requirements that apply to this operation point. |
| MaxGBR | Integer | The maximum value for guaranteed bitrate. Note that this implies that GBR = MBR for the operation point that is provisioned for this session. Traffic that exceeds the MaxGBR will be shaped, which may result in excessive delays and packet drops. |
| AveragingWindow | Integer | The averaging window over which the MaxGBR value is calculated. The value is provided in milliseconds. |
| MaxBurst | Integer | The maximum burst that is allowed for the session at this operation point. The MaxBurst is provided in bytes. |
| BurstAveragingWindow | Integer | The averaging window over which the burst is calculated. This value is typically an order of magnitude smaller than the AveragingWindow and is meant to allow for short-lived bursts in the traffic. |
| RouteSeletion | Object | Provides information to describe how traffic of this Provisioning is to be identified and filtered. |
| NSSAI | Integer | Identifier of the network slice used for this operation point. |
| DNN | String | Name of the Data Network that is used for this session at this operation point. |
| TrafficDescriptor | Object | Contains information about the traffic descriptors that are used to identify and filter the traffic. The traffic descriptor shall be formatted as described in 6.6.2.1-2 of TS 23.503. |
| Charging | Object | Provides information about the charging policy to be used for this operation point. |
| sponsor_identifier | String | Provides the identifier for the provider of the sponsored data connection as defined in 23.203[X]. |

### 5.5.3	Resource structure

The policy template API is accessible through this URL paths:

{apiRoot}/3gpp-m1d/v1/provisioning/{provisioning-subresource}

The following operations and the corresponding HTTP methods are supported. In each case, the sub-resource path specified in the second column shall be substituted into {provisioning-subresource} in the above URI template:

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template | policy-templates | POST | This is used to create a new Policy Template resource. |
| Fetch a Policy Template | policy-templates/{policy-template-id} | GET | This operation is used to retrieve an existing Policy Template resource. |
| Update a Policy Template | policy-templates/{policy-template-id} | PUT, PATCH | This operation is used to modify the configuration of an existing Policy Template. |
| Delete a Policy Template | policy-templates/{policy-template-id} | DELETE | This operation is used to delete an existing Policy Template. |