SA WG4 Meeting #108E (e-meeting)	S4-200591

# E-meeting, 2-9 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0016 | rev | - | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and clarifications for consumption reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2020-3-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Step 12 is missing in the figure 5.6-1. - In Clause 5.6.3, it is not clear about the criteria for location reporting in the consumtion reporting. - Other editorial changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding step 12 in figure 5.6-1. - In clause 5.6.3, it is propsed to clarify that the location reporting is only enabled if location reporting is requested by the Media AF and is allowed to be reported by the 5GMS Client. - Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some misunderstandings remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1, 5.6.2, 5.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

**************** First Change ****************

5.6	Consumption reporting

5.6.1	Consumption reporting procedures

Figure 5.6-1: Consumption reporting

Steps:

The first phase is the initialisation phase:

1: The App is started.

2: A media content item is selected.

3: The App triggers the Media Session Handler to start playback. The media player entry is provided.

4: The 5GMSd AF initializes the parameters for the consumption reporting configuration (e.g. frequency).

5: The Media Session Handler triggers the consumption reporting.

6: The Media Session Handler starts the 5GMS Player with the Entry URL.

The second phase is media playback:

The user preferences may be changed:

7: The App selects/changes the user preferences.

8: The 5GMS Player transmits user preferences to the consumption reporting.

When Media is playing, the consumption reporting parameters may be updated.

9:The 5GMSd AF updates the parameters for the consumption reporting.

10: The Media Session Handler triggers an update to the Consumption reporting.

When Media is playing

11: The 5GMS player regularly accesses to the media content.

12: In case of changes to the consumed media properties, the 5GMS Player transmits the changes to the consumption reporting function.

13: The consumption reporting regularly sends report(s) to the 5GMSd AF.

The last phase is to stop the media:

14: The App triggers the Media Session Handler to stop the video.

15: The Media Session Handler stops the Consumption reporting.

16: The consumption reporting may send the last report(s) to the 5GMSd AF.

17: The Media Session Handler stops the 5GMS player.

5.6.2	Consumption reporting parameters

Table 5.6.2-1 describes the parameters used in step 4 of clause 5.6.1.

Table 5.6.2-1: Parameters for consumption reporting configuration

| Parameters | Description |
| --- | --- |
| Report interval | Identify the interval when the consumption report messages are sent by the 5GMS Media Session Handler. |
| Server address | A list of addresses where the consumption reporting messages are sent by the 5GMS Media Session Handler. |
| Location reporting | Identify whether the 5GMS Media Session Handler provides the location data to the Media AF (in case of MNO or trusted third parties) |

Table 5.6.2-2 describes the parameters used in steps 13 or 16 of clause 5.6.1.

Table 5.6.2-2: Consumption reporting parameters

| Parameters | Description |
| --- | --- |
| Media player entry pointer | Identify the Media player entry. In case of DASH, the media player entry pointer can be a URL of the MPD. |
| Consumption reporting client ID | Identify the identifier of the UE that consumes the data. [The value of this identifier should be global unique ID such as MSISDN] |
| Location type | Identify the UE location type. This parameter is only used when the location reporting is enabled for the UE or for the Downlink Streaming session with a condition that the UE allows to share its location within operator’s trust domain. The location type can be CGI, ECGI or NCGI as defined in TS 23.003 [9]. |
| Location | Identify the UE location. This parameter is only used when the location reporting is enabled for the UE or for the Downlink Streaming session with a condition that the UE allows to share its location within operator’s trust domain. |
| Media consumed | Identify the media consumed. In case of DASH, the AdaptationSet@id may be used. |
| Start time | Define the time when the consumption starts. |
| Duration | Define the duration of the consumption of the media consumed. |

5.6.3	Triggering consumption reporting

If consumption report is supported and activated, the 5GMS Client shall send consumption reporting message when any of the following conditions occurs:

- Start of 5GMS consumption of a Downlink Streaming session;

- Stop of 5GMS consumption of a Downlink Streaming session;

- Upon determining the need to report ongoing 5GMS consumption

  - The "ongoing" report is performed at periodic intervals as set by the reportInterval attribute of the ConsumptionReportingConfiguration

- Upon determining a location change if location reporting is requested by the Media AF and is allowed to be reported by the 5GMS Client.

NOTE:	Whenever the consumption reporting message is sent, the 5GMS Client is expected to reset its corresponding 'report interval' timer to the value of that attribute and begin count down of the timer.  Whenever the 5GMS Client stops the consumption of the same session, it is expected to disable its corresponding 'report interval' timer.

**************** END CHANGES **************