3GPP TSG-SA4 Meeting #108-e	S4-200604

# 2nd – 9th April, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | - | rev | - | Current version: | 16.4.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discovery Server for FLUS Sinks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_FLUS |  |  |  |  |  | Date: |  |  | 2020-03-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 7.1.1 describes the resources for Uplink Streaming Control Interface. One of the resource described is a Sink resource. For managing Sink resources, there is a need for an API end-point, and therefore a functional entity to act as the API end-point. Detailed Justification is provided in S4-200603. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide clarification on FLUS sink discovery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Insufficient specification for FLUS Uplink Streaming Control Interface |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 5.3.2, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Start change 1 ***

### 4.2.1	General

Figure 4.2-1 depicts the architecture showing the relevant entities for providing an uplink streaming service.

Figure 4.2-1: FLUS architecture

The uplink streaming service architecture is based on a FLUS source located in a UE and a FLUS sink located in either another UE or the network.

The FLUS source receives media content from one or more capture devices. In the context of this specification, the capture devices are considered as parts of a UE or are connected to it.

When the FLUS sink is located in a UE, the FLUS sink shall forward media content to a decoding and rendering function.

When the FLUS sink is located in the network, the FLUS sink may forward media content to a processing or distribution sub-function. The processing and distribution sub-functions are not in scope of the present specification. The FLUS sink may act as a Media Gateway Function (MGW) and/or an Application Function (AF).

The F reference point connects a FLUS source and a FLUS sink. The F reference point enables the establishment and control of a single FLUS session.

The F reference point also enables the FLUS sink and the FLUS source to mutually authenticate and authorize each other.

FLUS source and FLUS sink are each split into:

-	Media Source and Sink (F-U end-points),

-	Control Source and Sink (F-C end-points),

-	Remote Controller and Remote Control Target (F-RC end-points) and

The UE, the FLUS source and the FLUS sink are considered to be logical functions. Functions are not required to be located in the same physical device; they can be spread over multiple physical devices and interconnected via other interfaces.

Multiple F-RC end-points can exist in a single FLUS source. F-RC end-points are independent of a FLUS sink and depend on the offered service.

The F reference point shall support security functionality for confidentiality protection for all subfunctions.

Details of the functions are shown in Figure 4.2-2.

Figure 4.2-2: Sub-functions of FLUS

Note that in the above diagram, hatched-filled boxes represent FLUS control plane functionality, solid line boxes represent mandatory functionality, and dashed-line boxes corresponds to optional functionality. Also, note that F-C and F-U denote FLUS control and FLUS user plane interactions, respectively, and do not represent reference points.

Control Source and Sink (F-C): FLUS control plane functionality including the associated processing by FLUS sink of the uploaded media for subsequent downstream distribution, plus FLUS media instantiation selection. F-C may also support configuration of static metadata for the session.

Remote Controller and Remote Control Target (F-RC): The Remote Control Target on the FLUS source receives control messages. The messages affect the behaviour of the Media Source in the FLUS source. Examples of commands issued to the Remote Control Target are start or stop of the media upstreaming process in FLUS source.

Media Source and Sink (F-U): Media plane functionality which includes setup of one or more media sessions and subsequent media data transmission via media streams. In some cases, a media session establishment protocol (e.g. IMS session set-up for MTSI-based FLUS instantiation) is necessary.

NOTE: F-C is not needed when the FLUS sink is an MTSI client that is only capable of rendering. In such event, logical control plane functions such as media and session descriptions and support for FLUS session establishment are encapsulated in FLUS user plane functionality.

F-C represents the interactions associated with the creation and modification of the configuration of the FLUS sink. F-C allows the Control Source to:

- select a FLUS media instantiation,

- provision static metadata associated with each media session present in the FLUS session,

- discover, select and configure the processing and distribution sub-functions.

In addition to the delivery of control messages, F-RC represents the interactions associated with the discovery, creation, modification and selection of the FLUS sink configuration by the Remote Controller. F-RC allows the Remote Controller to:

- provide the Media Sink information to the FLUS source,

- select a FLUS media instantiation,

- determine capture device settings and other FLUS source parameters.

The FLUS media instantiation is defined as part of the FLUS session. The user plane (F-U) may also contain the media stream establishment procedures when needed. Multiple media streams may be established for one FLUS session.

A media stream may contain media components of a single content type, e.g., audio, or media components of different content types, e.g., audio and video. A FLUS session may be composed of more than one media stream containing the same content type, e.g., multiple media streams of video.

The FLUS Source retrieves the list of FLUS sinks by communicating with a Sink Discovery Server. The Sink Discovery Server provides the list of all FLUS sinks in the form of URLs or FQDNs to the FLUS Source. The FLUS Source shall use a known URL to reach the Sink Discovery Server.

## *** End change 1 ***

## *** Start change 2 ***

#### 5.3.2.X	FLUS sink discovery

This procedure allows a Control Source to discover list of FLUS sinks

Figure 5.3.2.x-1: Get List of FLUS sinks

1. The Control Source sends the Get FLUS Sinks request to the Sink Discovery Server.

2. The Sink Discovery Server provides a list of FLUS sinks in response.

## *** End change 2 ***

## *** Start change 3 ***

### 7.1.1	Resources

F-C defines a set of resources managed at the FLUS sink that are controlled by the FLUS source. The set of resources are defined in the following table:

Table 7.1.1-1: Resources for managing FLUS sessions at FLUS sink

| Resource Name | Resource Type | Description |
| --- | --- | --- |
| Sink | Instance resource | Represents a single FLUS sink as described in section 7.1.1.1. The FLUS source can query the properties of that sink. However, a FLUS source cannot create, update, or terminate a FLUS sink. The creation, update, and termination of FLUS sinks are controlled exclusively by the network |
| Sinks | Collection Resource | Represents a collection of FLUS sink resources. |
| Session | Instance resource | Represents a single FLUS session resource as described in section 7.1.1.2. The FLUS source can provision or modify a single session at the FLUS sink. |
| Sessions | Collection Resource | Represents a collection of FLUS session resources. |

## *** End change 3***

## *** Start change 4 ***

## 7.2	Discovery

### To query the list of available Sink Discovery servers, the FLUS client shall use one of the following procedures:

-	The operator provides one or more Sink Discovery Server URLs to the FLUS source.

-	DNS resolution using FQDN: flus.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org

## *** End change 4 ***

## *** Start change 5 ***

## 7.X	FLUS sinks retrieval

GET /flus/v1.0/sinks

### Using this Get FLUS sinks procedure, the FLUS source enquires about the list of FLUS sinks at the Sink Discovery Server.

### To retrieve the list of FLUS sinks, the FLUS source shall use the HTTP GET method on the "sinks" collection resource as follows:

-	the request URI with the "path" is set to "/flus/v1.0/sinks"

-	the Host field is set to the FQDN of the Sink Discovery Server

### The URL in the request is set according to the information retrieved using the Discovery procedure described in clause 7.2.

The response shall be a JSON document that represents an array of objects, each of which contains a FQDN or URL to the entry-point to the FLUS Sink.

The response from the Sink Discovery Server to the FLUS source shall contain the following:

-	The Content-Type field specifying the MIME type (JSON) using which the information about list of sinks is encoded.

-	The Content-Length is the length of the content body.

The above header fields are followed by the content body in the format indicated by the Content-Type field.

The possible response messages from the Sink Discovery Server to the FLUS source are shown in Table 7.x-1.

Table 7.x-1: Response status code, message, and contents for FLUS sink retrieval using HTTP GET

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The Sink Discovery Server provides the list of FLUS sinks to the FLUS source. |
| 403 Forbidden | Request cannot be fulfilled | The Sink Discovery Server may include optional text to indicate why the request could not be fulfilled |
| Note: In addition to the above response codes, the Sink Discovery Server can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## *** End change 5 ***

## *** End of changes ***