Source:	Orange

Title:	dCR 26445 Clarifications on ch-aw-recv

## Document for:	Agreement

## Agenda Item:	7.3

## Introduction

It was found that the specification of ch-aw-recv given in TS 26.445 [1] is ambiguous.

We propose to remove this ambiguity by updating the corresponding text.

This topic was initially addressed at SA4#106 based on S4-191210 [2]. Comments on [2] were taken into account to produce a revised proposal.

## Problem statement

We address the following excerpt of clause A.3.1 of TS 26.445 [1]:

---

ch-aw-recv:	Specifies how channel-aware mode is configured or used for the receive direction. Permissible values are -1, 0, 2, 3, 5, and 7. If ch-aw-recv is -1, channel-aware mode is disabled in the session for the receive direction. If ch-aw-recv is 0 or not present, partial redundancy (channel-aware mode) is not used at the start of the session for the receive direction. If ch-aw-recv is positive (2, 3, 5, or 7), partial redundancy (channel-aware mode) is used at the start of the session for the receive direction using the value as the offset (See NOTE below). Partial redundancy is supported only when the bit-rate is 13.2 kbps and the bandwidth is wb or swb.

NOTE 4:	If a positive (2, 3, 5, or 7) value of ch-aw-recv is declared for a payload type and the payload type is accepted, the receiver of the parameter shall send partial redundancy (channel-aware mode) at the start of the session using the value as the offset. If ch-aw-recv=0 is declared or not present for a payload type and the payload type is accepted, the receiver of the parameter shall not send partial redundancy (channel-aware mode) at the start of the session. If ch-aw-recv=-1 is declared for a payload type and the payload type is accepted, the receiver of the parameter shall not send partial redundancy (channel-aware mode) in the session.  If ch-aw-recv is not present or a non-negative (0, 2, 3, 5, or 7) value of ch-aw-recv is declared for a payload type and the payload type is accepted, partial redundancy (channel-aware mode) can be activated or deactivated during the session based on the expected or estimated channel condition through adaptation signaling, such as CMR (see Annex A.2) or RTCP based signaling (see clause 10.2 of [13]).  If ch-aw-recv is not present or a non-negative (0, 2, 3, 5, or 7) value of ch-aw-recv is declared for a payload type and the payload type is accepted, the partial redundancy offset value can also be adjusted during the session based on the expected or estimated channel condition through adaptation signaling.

NOTE 5:	The frame erasure rate indicator for the channel-aware mode has two permissible values (LO, HI) and this indicator has to be initialized to HI, as specified in clause 5.8.4.

---

This note has several problems:

- Formally speaking, a ‘shall’ should not be used in a note (several instances of ‘shall’ are also present in the same note).

- The first sentence is ambiguous as it may be understood as mandating 13.2 CAM as the initial codec mode. For instance, if the session was negotiated as br=9.6-24.4, bw=swb, ch-aw-recv=3, this requirement seems to force an initial codec mode at 13.2 CAM (offset=3), which would be counterproductive if the bearers are set such that the call could start at 24.4 kbit/s.

- One may note that partial redundancy is only available at 13.2 kbit/s in WB and SWB and adaptation requests may be change CAM configuration. Moreover, the initial codec mode for EVS is also addressed in clause 7.5.2.1.8 of TS 26.114 [3].

Based on the discussion that took place at SA4#106, we propose not to modify the existing text in TS 26.445 A.3.1. Instead, we propose to address two additional notes.

## Proposal

We propose the following draft change request to TS 26.445 A.3.1:

---

[…]

ch-aw-recv:	Specifies how channel-aware mode is configured or used for the receive direction. Permissible values are -1, 0, 2, 3, 5, and 7. If ch-aw-recv is -1, channel-aware mode is disabled in the session for the receive direction. If ch-aw-recv is 0 or not present, partial redundancy (channel-aware mode) is not used at the start of the session for the receive direction. If ch-aw-recv is positive (2, 3, 5, or 7), partial redundancy (channel-aware mode) is used at the start of the session for the receive direction using the value as the offset (See NOTE below). Partial redundancy is supported only when the bit-rate is 13.2 kbps and the bandwidth is wb or swb.

NOTE 4:	If a positive (2, 3, 5, or 7) value of ch-aw-recv is declared for a payload type and the payload type is accepted, the receiver of the parameter shall send partial redundancy (channel-aware mode) at the start of the session using the value as the offset. If ch-aw-recv=0 is declared or not present for a payload type and the payload type is accepted, the receiver of the parameter shall not send partial redundancy (channel-aware mode) at the start of the session. If ch-aw-recv=-1 is declared for a payload type and the payload type is accepted, the receiver of the parameter shall not send partial redundancy (channel-aware mode) in the session. If ch-aw-recv is not present or a non-negative (0, 2, 3, 5, or 7) value of ch-aw-recv is declared for a payload type and the payload type is accepted, partial redundancy (channel-aware mode) can be activated or deactivated during the session based on the expected or estimated channel condition through adaptation signaling, such as CMR (see Annex A.2) or RTCP based signaling (see clause 10.2 of [13]).  If ch-aw-recv is not present or a non-negative (0, 2, 3, 5, or 7) value of ch-aw-recv is declared for a payload type and the payload type is accepted, the partial redundancy offset value can also be adjusted during the session based on the expected or estimated channel condition through adaptation signaling.

NOTE 5:	The frame erasure rate indicator for the channel-aware mode has two permissible values (LO, HI) and this indicator has to be initialized to HI, as specified in clause 5.8.4.

NOTE 6:	Partial redundancy with a positive (2, 3, 5, or 7) value of ch-aw-recv can only be sent at the start of the session when the initial codec mode is set to 13.2 kbit/s in WB or SWB.

NOTE 7:	If ch-aw-recv=-1 is not declared for a payload type and the payload type is accepted, the value of ch-aw-recv may be modified during the session by an adaptation request.

[…]

---

## References

- 3GPP TS 26.445, Codec for Enhanced Voice Services (EVS); Detailed algorithmic description

- 3GPP Tdoc S4-191210, dCR 26445 Clarifications on ch-aw-recv, Source: Orange

- 3GPP TS 26.114, IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction