Source:	Ericsson LM, Fraunhofer IIS, VoiceAge Corporation

Title:	Proposed Corrections to EVS Source Code

## Document for:	Agreement

## Agenda Item:	7.3

## Introduction

This document lists all proposed changes to the EVS source code, namely for

- TS 26.442 (Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point))

- TS 26.443 (Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point))

- TS 26.452 (Codec for Enhanced Voice Services (EVS); ANSI C code; Alternative fixed-point using updated basic operators)

for Releases 12 – 17. The proposed changes are described as diff to the source code in the corresponding CRs. This document is intended as an overview over all proposed changes. Further on, it should be mentioned that the proposed CRs are based upon [1].

## Proposed Changes

## Handling of possibly corrupt SID frames in AMR-WB IO/EVS decoder

In [2], two field issues related to the handling of possibly corrupt SID frames in the AMR-WB IO/EVS decoder were described. In the course of the investigations, the following issues were found:

- AMR-WB IO: All zero AMRWB SID_update results in a valid LP filter with extremely high LP-filter-gain

- For both, AMR-WB IO and EVS operating mode, especially corrupted SID frames might lead to instable behavior and eventually overshoots

- JBM: Bitstream-reading path was not in sync with the corresponding G.192 bitstream reading path. Further on, it was found that especially handling of AMRWB SID_first,  AMRWB SID_update is not covered in the JBM path.

To overcome the identified issues, the following solutions are proposed:

- Special treatment of all-zero AMR-WB IO SID_update frames, to avoid overshoots: All zero AMRWB SID are treated as erroneous received and the decoder is switched to concealment mode for these frames.

- Various bugfixes on JBM path of bitstream reading to align with the G.192 bistream reading path. Note: As SID_first frames are not signalled in G.192 format (which the EVS network simulation and the corresponding JBM path is based upon), the reference code is not able to detect such frames are not actually correctly unless good zero length frames can be signalled.

- Added safety check for LP-CNG ISF/LSF decoding (EVS, AMR-WB IO) to avoid overshoots due to instable filter coefficients. This consists of two measures on decoder-side (applies to SID frames only):

  - Limitation of the increment of the CNG energy index, to avoid sudden energy peaks

  - In case nevertheless a big energy increment is detected after ISF/LSF dequantization, the ISF/LSF coefficients of the previous frame are used instead (fixed-point code only)

Impact:

- The above described changes affect TS 26.442, TS 26.443 and TS 26.452.

- The above described changes have no effect on the testvectors as contained in TS 26.444.

## Wrong arithmetic in MSLVQ Encoder Module

During the development of TS 26.452, wrong arithmetic in the MSLVQ Encoder Module (function index_lvq_fx()) was discovered. A corresponding patch was already submitted as part of TS 26.452, but kept intentionally disabled, to keep TS 26.442 and TS 26.452 in sync. It is proposed to correct this now in both versions simultaneously.

Impact:

- The above described changes affect TS 26.442 and TS 26.452.

- The above described changes affect testvectors (encoder) as contained in TS 26.444.

## Improvements in Decoder-Robustness based on Torture Testing

Additional Tests have been carried to test the decoder robustness, in particular under combined conditions of a) bitrate/bandwidth switching b) bit-errors c) frame-errors/JBM delay/error profiles. Several issues could be identified:

- Reading of uninitialized memory in EVS_RX_GetSamples() due to an unnecessary call of read_indices_from_djb_fx(). To overcome this issue it is proposed to remove this call.

- Reading of uninitialized memory at several decoder instances in combination with bitrate switching and channel-aware mode concealment: It was found out that partial redundancy concealment after an HQ-Core or TCX High-Bitrate frame can lead to reading of uninitialized memory in the concealment path. These scenarios can happen only after bitrate switching and loss of at least the first 13.2 kbit/s CAM frame. It is thus proposed to skip the partial redundancy payload for such cases in the concealment and continue the regular concealment until the first valid 13.2 kbit/s CAM frame is received.

- Potential division by 0 in find_best_delay(). It is proposed to add the same handling as in the fixed-point code. (floating-point code only)

Impact:

- The above described changes affect TS 26.442, TS 26.443 and TS 26.452.

- The above described changes have no effect on the testvectors as contained in TS 26.444.

## Usage of log10(0) in shb_DTX()

During Testing it was found that log10(0) was occasionally used in shb_DTX() in the floating point source code, which might trigger a range error for certain implementations and which is certainly undesired. It is proposed to overcome this issue by addition a special handling of this situation in a similar manner as in the fixed-point code, i.e. by making sure that the affected variable is at least of value FLT_MIN.

Impact:

- The above described changes affect TS 26.443.

- The above described changes affect testvectors (encoder) as contained in TS 26.444.

## Hardening of Code

Various places could be identified where either memory could be accessed (reading/writing) out of bounds, uninitialized memory was read or errors were not handled correctly:

- decision_matrix_dec(): Make sure, a valid "start_idx" in decision_matrix_dec() is set when the bitstream is corrupted by bit errors

- tcx_ltp_post(): Set gain of TCX LTP to zero, in case of active concealment and non-consistent LTP parameters (e.g. due to bitrate switching)

- dec_prm(): Add a safety check for corrupt bitstreams wrt the calculations in  BITS_ALLOC_config_acelp()

- E_ACELP_codearithp(): Avoid buffer overflows in E_ACELP_codearithp() in case of corrupt input

- root_a_over_b(): A safety check for input variables of value “inf” was added, to avoid an endless loop (floating-point code only)

- speech_music_classif_fx(): Additional code was added to initialize the output variable *sp_aud_decision0; otherwise it might get read without initialization (but never evaluated).

Impact:

- The above described changes affect TS 26.442, TS 26.443, TS 26.452.

- The above described changes have no effect on the testvectors as contained in TS 26.444.

## Cleanup

Further on, two smaller issues on code clean-up could be identified:

- The static memory synth_history[] was found to be dimensioned too big in the floating-point code. The proposed change brings the floating-point code inline with the fixed-point code.

- The function IGFCommonFuncsWriteSerialBit() was declared twice in prot.h. The second declaration was removed.

Impact:

- The above described changes affect TS 26.443.

- The above described changes have no effect on the testvectors as contained in TS 26.444.

## Editorial Changes

Several improvements are proposed to commandline-help output related to MIME format. Further on, formatting is applied to the source code TS 26.452, to be consistent also in comparison to TS 26.442 and TS 26.443.

Impact:

- The above described changes affect TS 26.442, TS 26.443, TS 26.452.

- The above described changes have no effect on the testvectors as contained in TS 26.444.

## Conclusion

It is proposed to agree on the above outlined changes and incorporate them via the corresponding CRs into TS 26.442, TS 26.443 and TS 26.452 and also update the testvectors in TS 26.444 accordingly.

## References

[1] S4-200592, “Corrections to EVS Fixed-Point Source Code”

[2] S4-200128, “Two field issues with EVS“