3GPP TSG-SA4#109-e meeting	S4-200848

# 20th May – 3rd June 2020, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | <CR#> | rev | - | Current version: | 1.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GMS3: Default downlink profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 26.511 specification is a collection of 5GMS profiles with dedicated media requirements. Each profile is associated to a service scenario. In the absence of indication of a particular profile, the default profile requirements are applicable. Discussions happened on the level of requirement associated to HEVC. It was found agreeable to mandate HEVC for full HD services with HDR. The source observes that the choice for selecting HEVC when deploying media services is motivated more by the 10-bit decoding capability supported by HEVC than the support of HDR (e.g. 4K SDR TV) |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to indicate in the default downlink profile: - an 8-bit default profile in which AVC fullHD is mandatory and HEVC fullHD is recommended - a 10-bit default profile in which only HEVC is specified and mandatory in fullHD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Including HDR/WCG as a precondition to support HEVC creates confusion to network operators and service providers intending to deploy video services using HEVC but not neccesarily requesting the support of HDR/WCG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2; 5.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 5.2.2	Video decoding

If the 5GMS client supports the reception of 8-bit video,

-	AVC-FullHD-Dec decoding capability shall be supported as defined in clause 4.2.1.1.

If the 5GMS client supports the reception of 10-bit video,

-	HEVC-FullHD-Dec decoding capability shall be supported as defined in clause 4.2.2.1.

=====  CHANGE  =====

#### 5.2.7.2	Video media profiles

If the 5GMSd client supports the reception of 8-bit video, then the following applies:

-	the AVC-HD playback requirements as defined in clause 4.2.1.3.1.4 shall be supported.

-	the AVC-FullHD playback requirements as defined in clause 4.2.1.3.2.4 shall be supported.

If the 5GMSd client supports the reception of 10-bit video, then the following applies:

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 shall be supported.