3GPP TSG-WG SA4 #109-e	S4-200887

# 20th May – 3rd  June 2020

| CR-For v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | – | rev | – | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | M1d Procedure for Dynamic Policy Provisioning (Stage 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc., Ericsson LM, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | B |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The external application provider configures a set of policy templates that may be applied to the streaming sessions offered by it. This CR defines the Policy Template resource and the operations that are performed over the M1d/M1u. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines the policy template configuration API that is offered for the provisioning of downlink and uplink streaming session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 solution will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# First Change

### 4.2.4	Policy Template Configuration procedures

### 4.2.4.1	General

These procedures are used by the 5GMS Application Provider to configure the Policy Templates for streaming sessions of a particular Provisioning Session.

Since Policy Templates require 5GMS System operator verification, a Policy Template that is newly created cannot be used immediately. Upon creation, a Policy Template shall be in the pending state. Once it has been validated by the 5GMS System operator, it shall take the ready state, indicating that it may be applied to streaming sessions. If it is subsequently updated by the 5GMS Application Provider, a Policy Template shall return to the pending state, awaiting revalidation by the operator of the 5GMS System. Finally, a Policy Template may be suspended by the 5GMS System operator, e.g. in case of a violation of the usage terms or for some other reasons, which renders it unusable.

The 5GMSd/5GMSu AF shall verify the status of a Policy Template prior to allowing a Dynamic Policy Instance to instantiate it. Only Policy Templates in the ready state are eligible to be instantiated in this way.

Figure 4.2.4.1-1 below is a state diagram showing the life-cycle of a Policy Template:

Figure 4.2.4.1-1: Policy Template State Diagram

### 4.2.4.2	Create Policy Template

This procedure is used by the 5GMS Application Provider to create a new Policy Template. The HTTP POST method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall generate a resource identifier to uniquely identify the newly created Policy Template. In that case, it shall respond with a 201 (Created) HTTP response message and provide the URL to the newly created resource in the Location header field.

The default state of a newly created Policy Template is pending.

### 4.2.4.3	Read Policy Template

This procedure is used by the 5GMS Application Provider and other 5GMSd/5GMSu AFs to query the properties of an existing Policy Template resource from the 5GMSd/5GMSu AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response that includes the Policy Template in the response message body.

### 4.2.4.4	Update Policy Template

The update operation is invoked by the 5GMS Application Provider to modify the properties of an existing Policy Template. All available properties except state may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

Any update to the policy template will change the state back into the pending state, which makes it temporarily unusable.

Editor’s Note: It is FFS, whether ANY update or SOME updates of the Policy Template will trigger a state change to pending.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message that includes the Policy Template in the response message body. Modifications to read-only properties, such as changes to the state of a Policy Template, shall be rejected with a 403 (Forbidden) HTTP response.

### 4.2.4.5	Delete Policy Template

This operation is used by the 5GMS Application Provider to destroy a Policy Template resource. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd/5GMSu AF will remove the Policy Template from any Provisioning Sessions that reference it.

Currently active streaming sessions using the destroyed Policy Template, if any exist, shall be stopped by the removal of the Policy Template.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message.

# Second Change

## 7.9	Policy Templates Provisioning API

### 7.9.1	Overview

The Policy Templates Provisioning API allows a 5GMS Application Provider to configure a set of Policy Templates within the scope of a Provisioning Session that can subsequently be applied to media streaming sessions belonging to that Application Provider using the Dynamic Policies API specified in clause 11.5. A Policy Template is used to specify the traffic shaping and charging policies to be applied to these media streaming sessions.

A Policy Template, identified by its policyTemplateId, represents a set of PCF/NEF API parameters which defines the service quality and associated charging for the media streaming sessions. The Policy Template is configured as part of the Provisioning procedures with the 5GMS AF and is then used by the 5GMS AF to request specific QoS and charging policies for that session from the PCF or NEF.

### 7.9.3	Data model

#### 7.9.3.1	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in Table 7.9.3-1 below:

Table 7.9.3-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Visibility | Description |
| --- | --- | --- | --- | --- |
| policyTemplateId | Integer | 1..1 |  | Unique identifier of this Policy Template within the scope of the Provisioning Session. |
| state | Enumeration of Strings | 1..1 |  | A Policy Template may be in the pending, ready, or suspended state. Only a Policy Template in the ready state may be instantiated as a Dynamic Policy Instance and applied to streaming sessions. |
| apiEndPoint | String | 1..1 | MNO Admin | The API endpoint that should be invoked when activating a Dynamic Policy Instance based on this Policy Template. |
| apiType | Enumeration of Strings | 1..1 | MNO Admin | N5: Npcf Policy Authorization Service. N33: AsSessionWithQoS or CHargableParty. |
| externalReference | String | 1..1 |  | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. |
| QoSSpecification | Object | 0..1 |  | Specifies the network quality of service to be applied to streaming sessions at this Policy Template. |
| marBwUl | BitRate | 0..1 |  | As defined in clause 8.6.2.7 of TS 29.514. |
| marBwDl | BitRate | 0..1 |  |  |
| maxPacketLossRateDl | PacketLossRateRm | 0..1 |  |  |
| maxPacketLossRateUl | PacketLossRateRm | 0..1 |  |  |
| maxSuppBwDl | BitRate | 0..1 |  |  |
| maxSuppBwUl | BitRate | 0..1 |  |  |
| minDesBwDl | BitRate | 0..1 |  |  |
| minDesBwUl | BitRate | 0..1 |  |  |
| mirBwUl | BitRate | 0..1 |  |  |
| mirBwDl | BitRate | 0..1 |  |  |
| ApplicationSessionContext | Object | 1..1 |  | Specifies information about the application session context to which this Policy Template can be applied. |
| afAppId | AfAppId | 0..1 | Read-Only | As defined in clause 5.6.2.3 of TS 29.514. |
| sliceInfo | Snssai | 0..1 |  |  |
| dnn | Dnn | 0..1 |  |  |
| aspId | AspId | 0..1 |  |  |
| ChargingSpecification | Object | 0..1 |  | Provides information about the charging policy to be used for this Policy Template. |
| sponId | SponId | 0..1 |  | As defined in clause 5.6.2.3 of TS 29.514. |
| sponStatus | SponsoringStatus | 0..1 |  |  |
| gpsi | [Gpsi] | 0..* |  | List of UEs permitted to instantiate this Policy Template. |

Editor’s Note: The parameter externalReference is for further study. It may be a provisioning parameter of the Media Player and / or a Media Session Handler to assist mapping of external references to a policyTemplateId.

Editor’s Note: The ChargingSpecification object may contain any charging related information, such as sponId or afChargeId.

### 7.9.2	Resource structure

The Policy Template Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.9.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column shall be appended to the above base URL template:

Table 7.9.2-1: Operations supported by the Policy Template Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template | policy-templates | POST | Used to create a new Policy Template resource. |
| Fetch a Policy Template | policy-templates/‌{policyTemplateId} | GET | Used to retrieve an existing Policy Template resource. |
| Update a Policy Template |  | PUT, PATCH | Used to modify the configuration of an existing Policy Template. |
| Delete a Policy Template |  | DELETE | Used to delete an existing Policy Template resource. |

# Third Change

# Annex X:	AF QoS Mapping

### X.1	Overview

This annex defines the mapping from a media streaming description into Dynamic Policy Instance properties for different types of media streaming description.

### X.2	DASH mapping

Editor’s Note: This clause should describe the mapping from DASH to Dynamic Policy Instances.