3GPP SA4 Video Meeting #109e	S4-200892

# Online,  <Start_Date> - <End_Date>

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex on performance evaluation for FS_5GVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ATEME |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Need for formal objective video quality metrics documentation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce PSNR and metric template for the annex |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.3.2, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== CHANGE 1 =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[3]	3GPP TS 26.116: "Television (TV) over 3GPP services; Video profiles".

[4]	3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[5]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[6]	3GPP TR 26.928: "Extended Reality over 5G".

[7]	Recommendation ITU-T H.264 (06/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014/Amd.3 2016: "Information technology - Coding of audio-visual objects - Part 10: Advanced Video Coding".

[8]	Recommendation ITU-T H.265 (11/2019): "High efficiency video coding" | ISO/IEC 23008-2:2020: "High Efficiency Coding and Media Delivery in Heterogeneous Environments - Part 2: High Efficiency Video Coding".

[9]	2020 Mobile Internet Phenomena Report, accessible here: https://www.sandvine.com/download-report-mobile-internet-phenomena-report-2020-sandvine, February 2020.

[10]	2019 Ericsson Mobility Report, accessible here: https://www.ericsson.com/4acd7e/assets/local/mobility-report/documents/2019/emr-november-2019.pdf, November 2019.

[11]	T. Fautier, "New Codecs for 5G", DASH-IF Workshop on "Media Streaming meets 5G", December 2019, accessible here: https://dashif.org/docs/workshop-2019/04-thierry%20fautier%20-%20Harmonic%20Codec%20Comparison%205G%20Media%20Workshop_Final%20v3.pdf

[12]	Bitmovin Video Developer Report, accessible here: https://go.bitmovin.com/video-developer-report-2019, September 2019.

[13]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[14]	Recommendation ITU-R BT.709-6 (06/2015): "Parameter values for the HDTV standards for production and international programme exchange".

[15]	Recommendation ITU-R BT.2020-2 (10/2015): "Parameter values for ultra-high definition television systems for production and international programme exchange".

[16]	Recommendation ITU-R BT.2100-1 (06/2017): "Image parameter values for high dynamic range television for use in production and international programme exchange".

[A]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sürhing, “JVET Common Test Conditions and software reference configuration for SDR video”, Document JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

[B]	A. Segall, E. Francois, W. Husak, S. Iwamura and D. Rusanovskyy, “JVET Common Test Conditions and evaluation procedure for HDR/WCG video”, Document JVET-P2011, 16th Meeting: Geneva, CH, 1–11 Oct. 2020.

[C]	J. Strom, K. Andersson, R. Sjöberg, A. Segall, F. Bossen, G. Sullivan, J.-R. Ohm and A. Tourapis, “Summary information on BD-rate experiment evaluation practices”, Document JVET-R2016, 18th Meeting: by teleconference, 15–24 April 2020.

[D]	ISO/IEC JTC 1/SC 29/WG 11 MPEG Video, “Common Test Conditions for Essential Video Coding”, Document ISO/IEC JTC 1/SC 29/WG 11 N19003, Brussels, Belgium – January, 2020

===== End of CHANGE 1 =====

===== CHANGE 2 =====

## 5.3	Key Performance Indicators and Metrics

This clause defines performance metrics as used in section 7, to numerically and objectively express a Decoder's reaction to compressed video data that is possibly modified due to erasures. Only objective metrics are considered which can be computed from sequences being available in a 3G format as described in Annex C by using the method detailed in annex B.

The following section provides a general description of the quality metrics. For the exact computation with the availability of sequences in 3G format please refer to annex C.

The following acronyms are utilized throughout the remainder of this subclause:

-	OrigSeq: The original video sequence that has been used as input for the video encoder.

-	ReconSeq: The reconstructed video sequence, the output of a standard compliant decoder that operates on the output of the video encoder without channel simulation, i.e. without any errors. Timing alignment between the OrigSeq and ReconSeq are assumed.

-	ReceivedSeq: The video sequence that has been reconstructed and error-concealed by an error-tolerant video decoder, after a) the video encoder operated on the OrigSeq and produced an error free packet stream file as output, b) the channel simulator used the error free packet stream file and applied errors and delays to it so to produce an error-prone packet file which is used as the input of the error-tolerant video decoder. For comparison purpose, a constant delay between OrigSeq and the ReceivedSeq is assumed, whereby this constant delay is removed before comparison.

Each of the following metrics generates a single value when run for a complete video sequence.

The APSNR metric described in subclause 5.3.1 is evaluated for all sequences. The additional metrics described in subclause 5.3.2 are specific to HDR content and is provided only that type of content.

### 5.3.1	Average Peak Signal-to-Noise Ratio (APSNR)

The average Peak Signal-to-Noise Ratio (APSNR) calculated between all pictures of the OrigSeq and the ReconSeq or the ReceivedSeq, respectively. First, the Peak Signal-to-Noise Ratio (PSNR) of each picture is calculated with a precision sufficient to prevent rounding errors in the future steps. Thereafter, the PSNR values of all pictures are averaged. The result is reported with a precision of two digits.

NOTE:	This is the traditional metric referred to as PSNR in the academic literature and in the context of video compression research.

Only the luminance component of the video signal is used.

### In case that results from several ReceivedSeq are to be combined, the average of all PSNR values for all ReceivedSeq is computed as the final result.

### 5.3.2	HDR quality metrics: PSNRL100, DE100 and wPSNR

For HDR/WCG content represented with Hybrid Log-Gamma (HLG) transfer function as per BT.2100/HLG [16], conventional PSNR as shown in subclause 5.3.1 computed on the luma and chroma codewords (Y’CbCr samples) was found relevant for quality assessment.

For HDR/WCG content represented with ST 2084/PQ transfer function, as per BT.2100/PQ [16], two groups of quality metrics have been found to more appropriate, these metrics are:

- CIELab domain metrics:

  - PSNRL100: reflects a fidelity level for the luminance component. This metric is computed in the CIELab colour space domain of the input and output sample values of the codec.

  - DE100: reflects a fidelity level for the chromaticity. This metric is computed in the CIELab colour space demain of the input and output sample values of the codec.

These metrics have been found useful to provide information on combined performance on bit-rate saving, thus avoiding PSNR YUV combined metric computation. Computation of these metrics requires to convert OrigSeq and ReconSeq in the CIELab colour space domain.

- Y’CbCr domain metric:

  - wPSNR: reflects a fidelity level for each Y’, Cb and Cr color components independently, similarly to PSNR. wPSNR metric is calculated in the Y’CbCr domain (with BT.2100/PQ representation) on luma and chroma components samples of the input and ouput samples. Weighted multiplier, based on the luma sample value, is applied to model the perceptual sensitivity of Human Visual System for the given representation. The table of weighting multiples can be found in [C].

In case that results from several ReceivedSeq are to be combined, the average values of these metrics for all ReceivedSeq are computed as the final results.

===== End of CHANGE 2 =====

===== CHANGE 3 =====

Annex B
Objective quality metrics

## B.1	Introduction

This Annex provides documentation on objective quality metrics used for codec characterization in this Technical Report. Performance evaluation is conducted using methodology and good practices established in MPEG, thus this annex references relevant documents. In addition, the quality evaluation tools and associated scripts are provided in section B.6

## B.2	APSNR

The PSNR value for each frame is computed as described in document [C], Section 3.1 (see Equations 1 and 2).

## The APSNR value is computed as the average of per-frame PSNR value, as described in document [C], Section 3.2 (see Equation 3).

## B.3	PSNRL100

## The PSNRL100 is computed as described in [B], Section A.3.

## B.4	DE100

## The DE100 is computed as described in [B], Section A.2.

## B.5	wPSNR

The wPSNR is computed as described in [B], Section A.1.

## B.6	Quality evaluation tools and scripts

TBD: Link to 3gpp gitbhub, scripts, simulations instructions, etc …

===== End of CHANGE 3 =====