3GPP TSG-SA 4 Meeting #90-e	S4-200902

# E-meeting, 20th May – 3rd June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | <CR#> | rev |  | Current version: | # 16.4.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MQTT for Remote Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Ericsson LM, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_FLUS |  |  |  |  |  | Date: |  |  | 18-05-2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This draft CR introduces the functionality of remote control through the MQTT protocol for the IMS-baed and the Generic FLUS. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No remote control functionality will be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R BS.2051-1 (06/2017): "Advanced Sound System for Programme Production".

[3]	ISO/IEC 23090-2:2019: "Information technology - Coded representation of immersive media - Part 2: Omnidirectional media format".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[5]	3GPP TS 26.235: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[6]	3GPP TS 24.147: "Conferencing using the IM Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[9]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[10]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[11]	IETF RFC 7231: "Hypertext transfer protocol (HTTP/1.1): Semantics and Content" (June 2014).

[12]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	3GPP TS 23.682: " Architecture enhancements to facilitate communications with packet data networks and applications".

[16]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[17]	ISO/IEC DIS 23090-8: "Information technology -- Coded representation of immersive media -- Part 8: Network based media processing" (under development).

[18]	IETF RFC 4574: "The Session Description Protocol (SDP) Label Attribute" (August 2006).

[19]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[20]	3GPP TS 38.321,: "NR; Medium Access Control (MAC) protocol specification".

[21]	OASIS Standard, MQTT Version 5.0, https://docs.oasis-open.org/mqtt/mqtt/v5.0/mqtt-v5.0.html

Second Change

# 9	Remote Control

## 9.1	General

The FLUS Source may support a remote control interface through the F-RC. The Remote Control Target in the FLUS Source may subscribe to one or more remote controll channels. The Remote Control Target in the FLUS Source and the Remote Controller shall implement the MQTT protocol as specified in [21] for remote control.

The Remote Control Target shall connect at the session start-up to the MQTT Broker and may provide authorization credentials at subscription.

The Remote Control Target shall subscribe at the session start-up to one or more MQTT topics to receive remote controll commands. The MQTT Broker may restrict subscription to MQTT topics.

The Remote Controller shall connect to the MQTT Broker and provide authorization credentials. The MQTT Broker shall restict publication to only authorized clients using authorization rules.

If remote control is supported, the Remote Controller and the Remote Control Target shall be provisioned with the MQTT broker address. In the IMS-baed FLUS, the “a=remote_control_url:” session level attribute shall be used to signal the broker’s address. The URI scheme shall be a “wss” or “mqtts”.

The MQTT Topics shall be structured as defined in Clause 9.2.

The MQTT Messages shall be formated according to Clause 9.3.

## 9.2	Usage of MQTT

The Topic shall be formatted according to the following ABNF syntax:

Topic=session_id “/” protocol  [ “/” remote_controller_id ] [“/” media_source_id]

The session_id shall be the SIP session identifier for the IMS-based FLUS or the F_C session resource id for the Generic FLUS.

The protocol shall be a unique identifier of the application remote control protocol. The protocol defines the format of the payload message.  This document does not define any protocol.

The remote_controller_id shall be a unique identifier of the remote controller. A Remote Control Target in the FLUS Source may receive message from different Remote Control Targets.

The media_source_id shall contain the unique identifier of the media source in that session. When the media_source_id is omitted, then the message should be applied to all media sources of that FLUS source. In case no source_id is provided in the PUBLISH or SUBSCRIBE messages, then all remote control targets are targetted by that message.

A Remote Control Target shall at least subscribe to the “session_id/protocol/#” topic, enabling the reception of message from any authorized Remote Controller and for any Media Source.

If remote control is supported, the FLUS Sink shall supply the MQTT broker websocket address during the session setup to the FLUS Sink. In F-C, the “remote_control_url” element is used to signal the MQTT broker address, to which the FLUS Source should subscribe. In the IMS-baed FLUS, the “a=remote_control_url:” session level attribute shall be used to signal the broker’s address. The URL shall be a “wss” addres.

## 9.3	Message Format

The following message format for information exchange (e.g. the request and response) using F-RC shall be used when delivering a message.

The remote control message shall be formatted as a json [RFC8259] formatted object.

-	The json object contains four name / value pairs:

-	The first name / value pair represents the start line of an HTTP message. The name is “startLine” and the value is string according to RFC 8259.

-	The second name / value pair represents the protocol identifier. The name is “protocol” and the value is string according to RFC 8259.

- 	The third name / value pair represents the optional headers. The name is “headers”. The value is a json array of json object.

-	Each json object of the array contains the key and the value of a single HTTP header.

-	The Host HTTP header may be omitted, since the WebSocket Connection is already established.

-	The fourth name / value pair represents the optional body. The name is “body” and it is formatted according to the Content-Type header field. The content type of the body object is defined by the individual API.