3GPP TSG-SA4 Meeting #110-e	S4-20xxxx

# Electronic, Online, 19th–28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | – | rev | – | Current version: | 1.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provisioning Sessions API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-08-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Complete the stage 3 design for Provisioning Sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing refernece to TS 29.514 (for AspId type). - Add missing clauses for Overview, Resource structure and Data model for Provisioning Sessions API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 design for M1 will be incomplete in Release 16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

...

[Z]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

SECOND CHANGE

## 7.2	Provisioning Sessions API

### 7.2.1	Overview

The Provisioning Sessions API is used by the 5GMS Application Provider to instantiate and manipulate Provisioning Sessions in the 5GMS System, as described in clause 4.3.2. Having created a Provisioning Session, the 5GMS Application Provider can then go on to provision other 5GMS features in the context of that Provisioning Session, using the APIs specified in clause 7.3 et seq.

### 7.2.2	Resource structure

The Provisioning Sessions API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/

Table 7.4.2-1 specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the above URL base path.

Table 7.2.2-1: Operations supported by the Provisioning Sessions API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Provisioning Session |  | POST | Used to create a new Provisioning Session resource. Returns a {provisioningSessionId}. |
| Retrieve Provisioning Session | {provisioningSessionId} | GET | Used to retrieve a Provisioning Session resource for inspection. |
| Destroy Provisioning Session |  | DELETE | Used to destroy an existing Provisioning Session resource. |

### 7.2.3	Data model

#### 7.2.3.1	ProvisioningSession resource

The data model for the ProvisioningSession resource is specified in table 7.2.3.1-1 below:

Table 7.2.3.1-1: Definition of ProvisioningSession resource

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | C: R R: R | An identifier for this Metrics Reporting Configuration that is unique within the scope of the enclosing Provisioning Session. |
| aspId | AspId | 0..1 | C: W | The identity of the Application Server Provider, as specified in clause 5.6.2.3 of TS 29.514 [Z]. |
| serverCertificateIds | Array(String) | 0..N | R: RO | A (possibly empty) array of Server Certificate identifiers currently associated with this Provisioning Session. |
| contentPreparation‌TemplateIds | Array(String) | 0..N | R: RO | A (possibly empty) array of Content Preparation Template identifiers currently associated with this Provisioning Session. |
| contentHosting‌ConfigurationId | String | 0..1 | R: RO | The Content Hosting Configuration identifier currently associated with this Provisioning Session, if any. |
| consumptionReporting‌ConfigurationId | String | 0..1 | R: RO | The Consumption Reporting Configuration identifier currently associated with this Provisioning Session, if any. |
| metricsReporting‌ConfigurationIds | Array(String) | 0..N | R: RO | A (possibly empty) array of Metrics Reporting Configuration identifiers currently associated with this Provisioning Session. |
| policyTemplateIds | Array(String) | 0..N | R: RO | A (possibly empty) array of Policy Template identifiers currently associated with this Provisioning Session. |

END OF CHANGES