3GPP TSG-SA4 Meeting #111e	S4-201329

11-20 November 2020

Agenda item	11.5

Source: 	Intel

Title: 	On SEI Messages for ITT4RT

Document for	Discussion and Agreement

# Introduction

A draft CR to TS 26.114 introducing video support for ITT4RT in S4aM200591 was reviewed during the ITT4RT telco on Oct 21st and partially agreed after online edits. This draft CR is made available in document S4-201320 during SA4#111-e.

In this contribution, we propose some further updates to the SEI message requirements for ITT4RT video bitstreams. Our proposed text updates to the latest draft CR in S4-201320 are presented in Section 2.

# Proposed Text Updates to draft CR to TS 26.114

# X.3	Immersive Video Support

ITT4RT-Rx clients in terminals offering video communication shall support decoding capabilities based on:

-	H.264 (AVC) [R3] Progressive High Profile, Level 5.1 with the following additional restrictions and requirements on the bitstream:

-    the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

-    the bitstream does not contain more than 10 slices per picture.

-	H.265 (HEVC) [R4] Main 10 Profile, Main Tier, Level 5.1.

ITT4RT-Tx clients in terminals offering video communication shall support encoding up to the maximum capabilities (e.g., color bit-depth, luma samples per second, luma picture size, frames per second) compatible with decoders compliant with the following:

-	H.264 (AVC) [R3] Progressive High Profile, Level 5.1 with the following additional restrictions and requirements on the bitstream:

-    the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

-    the bitstream does not contain more than 10 slices per picture.

-	H.265 (HEVC) [R4] Main 10 Profile, Main Tier, Level 5.1.

For H.264/AVC and H.265/HEVC random access point (RAP) definitions refer to TS 26.116 [X], clauses 4.4.1.1 and 4.5.1.2.1, respectively. RAPs shall be present in the video bitstreams delivered from ITT4RT-Tx clients to ITT4RT-Rx clients at least once every 5 seconds. It is recommended that RAPs occur in the video bitstream on average at least every 2 seconds. The time interval between successive RAPs is measured as the difference between their respective decoding time values. If viewport adaptation is offered, then RAPs should occur even more frequently to enable transitioning across these viewport-optimized bitstreams.

For 360-degree video delivery across ITT4RT clients, the following components are applicable:

-	The RTP stream contains an HEVC or an AVC bitstream with omnidirectional video specific SEI messages. In particular, the omnidirectional video specific SEI messages as defined in ISO/IEC 23008-2 [R4] or ISO/IEC 14496-10 [R3] may be present for the respective HEVC or AVC bitstreams.

-	The video elementary stream(s) shall be encoded following the requirements in the Omnidirectional Media Format (OMAF) specification ISO/IEC 23090-2 [R1], clauses 10.1.2.2 (viewport-independent case) or 10.1.3.2 (viewport-dependent case) for HEVC bitstreams and clause 10.1.4.2 for AVC bitstreams. Furthermore, the general video codec requirements for AVC and HEVC in clause 5.2.2 of TS 26.114 also apply.

ITT4RT-Rx clients are expected to be able to process the VR metadata carried in SEI messages for rendering 360-degree video. Relevant SEI messages contained in the elementary stream(s) with decoder rendering metadata may include the following information as per ISO/IEC 23008-2 [R4] and ISO/IEC 14496-10 [R3]:

-	Region-wise packing information, e.g., carrying region-wise packing format indication and also any coverage restrictions

-	Projection mapping information, indicating the projection format in use, e.g., Equi-rectangular projection (ERP) or Cubemap projection (CMP)

-	Padding, indicating whether there is padding or guard band in the packed picture

-	Frame packing arrangement, indicating the frame packing format for stereoscopic content

-	Content pre-rotation information, indicating the amount of sphere rotation, if any, applied to the sphere signal before projection and region-wise packing at the encoder side

-	Fisheye video information, indicating that the picture is a fisheye video picture containing a number of active areas captured by fisheye camera lens.  This information enables remapping of the colour samples of the pictures onto a sphere coordinate space

In particular, the equirectangular projection SEI message (payloadType equal to 150 shall be present in the video bitstream delivered from ITT4RT-Tx clients to ITT4RT-Rx clients at every RAP, with the erp_guard_band_flag set to 0. In case viewport-dependent

-	the region-wise packing SEI message (for details see clauses 5.1.5.11 and 5.1.5.12)

-	the frame-packing arrangement SEI message (for details see clause 5.1.5.13)

If viewport-dependent processing (VDP) capability is successfully negotiated by the ITT4RT-Tx client and ITT4RT-Rx client, then the video bitstream at every RAP shall contain either:

-	the equirectangular projection SEI message (payloadType equal to 150) with the erp_guard_band_flag set to 0, or

-	the cubemap projection SEI message (payloadType equal to 151).

In order to optimize the spatial resolution of specific viewports, the ITT4RT-Tx client and ITT4RT-Rx client may negotiate the use of region-wise packing. If this is the case, the video bitstream at every RAP may include the region-wise packing SEI message (payloadType equal to 155).

If stereoscopic video capability is successfully negotiated by the ITT4RT-Tx client and ITT4RT-Rx client, then the video bitstream at every RAP shall include frame packing arrangement SEI message (payloadType equal to 45) with the following restrictions:

-	The value of frame_packing_arrangement_cancel_flag is equal to 0.

-	The value of frame_packing_arrangement_type is equal to 4.

-	The value of quincunx_sampling_flag is equal to 0.

-	The value of spatial_flipping_flag is equal to 0.

-	The value of field_views_flag is equal to 0.

-	The value of frame0_grid_position_x is equal to 0.

-	The value of frame0_grid_position_y is equal to 0.

-	The value of frame1_grid_position_x is equal to 0.

-	The value of frame1_grid_position_y is equal to 0.

[With regards to the negotiation of SEI messages for carriage of decoder rendering metadata, procedures specified in IETF RFC 7798 [6] on the RTP payload format for HEVC may be reused. In particular, RFC 7798 can allow exposing SEI messages related to decoder rendering metadata for omnidirectional media in the SDP using the 'sprop-sei' parameter, which allows to convey one or more SEI messages that describe bitstream characteristics. When present, a decoder can rely on the bitstream characteristics that are described in the SEI messages for the entire duration of the session. Intentionally, RFC 7798 does not list an applicable or inapplicable SEI messages to be listed as part of this parameter, so the newly defined SEI messages for omnidirectional media in ISO/IEC 23008-2 can be signalled. It is expected that both MTSI clients and MTSI gateways support RTP payload formats for VR support.

Editor’s Note: It is to be decided whether SEI messages for omnidirectional video needs to be explicitly negotiated or whether ITT4RT-MTSI clients must support these messages]

Furthermore, ITT4RT-Tx clients supporting fisheye video shall include the following omnidirectional video specific SEI messages as defined in ISO/IEC 23008-2 [R4] and ISO/IEC 14496-10 [R3] in the video elementary streams corresponding to HEVC or AVC bitstreams::

- the fisheye video information SEI message (for fisheye video).

]