Source:	Nokia Corporation

Title:	Improved text on SDP for ITT4RT Draft CR

## Document for:	Agreement

## Agenda Item:	11.5

# 1. Introduction

The document proposes improved text for some SDP attributes in the ITT4RT draft CR. Note that the source of the 360-degree video is not the source of all possible overlays e.g., when the 2D video of an ITT4RT-Rx client is offered as an overlay by the ITT4RT MRF. . In this case the itt4rt_group attribute can be used by the ITT4RT MRF to indicate  the media sources that will be used as overlays. The ITT4RT MRF may be able provide overlay configuration based on these overlays. The text reflects the possibility of this exchange.

# 2. Changes to CR

--------------------------------------------------START OF CHANGE1 --------------------------------------------

- X.6.1	General

Based on the architecture described in clause X.2, an SDP framework for immersive video and immersive voice/audio exchange for ITT4RT is presented to negotiate codec support, SEI messages for decoder rendering metadata, as well as RTP/RTCP signaling necessary for viewport dependent processing.

The SDP attributes 3gpp_360video, 3gpp_overlay, 3gpp_360bg shall be used to indicate respectively a 360-degree video stream, a spherical overlay and a 360-degree background image (or series of images or video). The detailed definition and usage of these SDP attributes are presented in the clauses below.

Editor’s Note: Additional text on the presence of multiple 360-degree video participants and their handling in the SDP may be required for clarification in this section.

--------------------------------------------------END OF CHANGE1 --------------------------------------------

--------------------------------------------------START OF CHANGE2 --------------------------------------------

X.6.2.5 Overlay and 360-degree video

An ITT4RT-Tx or ITT4RT MRF client that sends an SDP message with at least one 360-degree video/audio and at least one overlay [shall] include in SDP the attribute itt4rt_group before any media lines. The itt4rt_group attribute is used to group 360-degree media and overlay media using the mid attribute as defined according to the ABNF below:

a = itt4rt_group: <mid1> SP <mid2> SP <mid3> …

The list of mids in the itt4rt_group shall include at least one mid associated with 360-degree media and at least one mid associated with an overlay as defined by the mid attribute in the corresponding media description.

An ITT4RT-Tx or ITT4RT MRF client and an ITT4RT-Rx client may negotiate the overlays that can be associated with the 360-degree video offered by the ITT4RT-Tx or ITT4RT MRF client using the itt4rt_group attribute. An ITT4RT-Tx or MRF client may indicate in an offer the overlays to be grouped with the 360-degree video using the itt4rt_group attribute.  2D video received from an ITT4RT-Rx client can be offered as an overlay by the ITT4RT MRF to other ITT4RT-Rx clients.  The overlays that are acceptable shall be retained in the answer and the ones that are not acceptable shall be removed. An ITT4RT-Tx or ITT4RT MRF client may offer overlay configuration options using the 3gpp_overlay attribute based on the list of overlays provided in the itt4rt_group attribute.

The order of the media included in the itt4rt_group indicates the synchronization source with the first media always being the synchronization anchor when synchronization is required.

--------------------------------------------------END OF CHANGE2 --------------------------------------------

# 4. Proposal

# The proposal is to incorporate the proposed changes to the draft CR.

Contact: Igor Curcio, Saba Ahsan, Nokia Technologies, Finland. Emails: igor.curcio, saba.ahsan@nokia.com.