SA WG4 Meeting #S4-112E	S4- 210106

1 - 10 Febrary, 2021, Electronic meeting

Title:		Draft LS on Application ID Usage in NEF Related Service API

Response to: 	-

Release:	Rel-16

Work Item:	5GMS3

Source:	Huawei Technologies Co. Ltd. (to be SA4)

To:		TSG CT3, TSG SA2

Cc:

Contact person:	Qi Pan

panqi8@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

SA4 has been studying the 5G Media Streaming (5GMS) Protocols in the Rel-16.

In the dynamic policy invocation procedures, the Application Identifier can be used as an alternative traffic detection parameter of the full flow descriptions.

When the Policy Template relates to QoS, the 5GMSd AF may use the Nnef_AFsessionWithQoS service as defined in TS 23.502 [3] clause 5.2.6.9. (The complete call flow is described in TS 23.502 clause 4.15.6.6);

When the Policy Template relates to a different charging scheme, the 5GMSd AF may use the Nnef_ChargeableParty service as defined in TS 23.502 [3] clause 5.2.6.8.

There are some ambiguous places need to be clarified in the SA2 and CT3 specifications.

Q1: As the flow descriptions and the application id can both be used for traffic detection, we noticed that in the Nnef_AFsessionWithQoS service, the flow description is still mandatory as a required input as defined in TS 23.502 clause 5.2.6.9. Is the flow description still mandatory when the Application ID is used ?

Q2: In the procedures for setting up an AF Session with Required QoS and changing the Chargeable Party, there is a description in TS 29.122 that the SCEF/NEF shall/may map the SCS/AS/AF Identifier to the AF Application ID. There are several questions need to be clarified.

- Generally one SCS/AS identifier corresponds to different application id. So how does this mapping work ?

- When shall the SCEF/NEF shall map the SCS/AS/AF Identifier to the AF Application ID and when may the SCEF/NEF SCS/AS/AF Identifier to the AF Application ID?

- And is there any difference between AF Application ID and Application ID?

# 2	Actions

To SA2

ACTION: 	SA4 kindly asks SA2 to provide feedback for Q1.

To CT3

ACTION: 	SA4 kindly asks CT3 to provide feedback for Q2.

# 3	Dates of next TSG SA WG2 meetings

SA4#113E		6 April – 14 April 2021				             Electronic meeting

SA4#114E		19 May – 28 May 2021					Elewctronic meeting