3GPP TSG SA4 Meeting #113e	TD S4-210551

## 6-14 April 2021

- Source:	Qualcomm Incorporated; Tencent; Fraunhofer HHI; Ericsson LM; ATEME; Beijing Xiaomi Electronics; InterDigital, Inc.; AT&T; Nokia Corporation; Dolby Laboratories; Samsung Electronics Co. LTD; Sony Europe B.V.; Huawei Technologies Co., Ltd.

- Title:	New Work Item on “8K Television over 5G”

- Document for:	Agreement

Agenda Item:	11.10

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 8K Television over 5G

## Acronym: 8K_TV_5G

## Unique identifier:

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 890009 | Operation Points for 8K VR 360 Video over 5G | This work item introduced 8K decoding capabilities for VR in 5G Media streaming. |
| 870011 | Feasibility Study on 5G Video Codec Characteristics | This study item provides anchors for existing 3GPP codecs for different service scenarios and provides a framework to add new scenarios as well as to characterize new codecs. |

## 3	Justification

Since the initial development and last update of TV Video Profiles defined in 3GPP TS 26.116, TV and mobile device capabilities have improved and nowadays they support higher decoding capabilities. In particular, new TV sets and 5G mobile devices entering the market since 2020 support up to 8K video decoding as well as 8K display capabilities.

8K is recently trialed and introduced in several services. In addition, other ecosystem support is happening, such as 8K encoders are announced, 8K TV sets are shipped and content is produced in 8K. Some good information, background and motivation is for example collected by the 8K Association. Furthermore, it evident that distribution of 8K TV content is feasible with 5G. As an example it was shown that with HEVC around 40 MBit/s on sports content can be achieved. Most of the existing services are build on H.265/HEVC.

In order to provide full interoperability for 8K TV services in the context of 5G, it is important to define the relevant 8K operation points, bitstream requirements and decoding capabilities. This is addressed in this work item.

In addition, it is relevant for 5G Systems to also understand the deployment characteristics and typical traffic characteristics. Finally, the 8K TV scenario is preferably added together with HEVC encoded anchors to the 5G Video codec characterization framework in order to support 5G system design.

## 4	Objective

The objective of this Work Item is to specify an HEVC-based 8K TV operation point in TS 26.116 as well as the corresponding media decoding capabilities for 5GMS in TS 26.511 in order to enable support for up to 8K video.

More specifically, this work item aims to conduct the following normative work in TS 26.116 and TS 26.511, toward fulfilling this objective:

-	Define new 8K TV operation point(s) for TV Video profiles with conforming bitstream requirement based on H.265/HEVC Main-10 Profile Main Tier Profile with the following constraints:

-	In addition to the luminance resolutions already included in TS 26.116 for UHD services, the specification is expected to support 16:9 aspect ratio and luminance resolution up to 7680x4320 pixels, including 5120 x 2880 pixels.

-	The specification is expected to enable an 8K operation point that is within the profile level constraints of H.265/HEVC Main-10 Profile, Main Tier and Level 6.1 decoding capabilities.

-	The specifications is expected to support conformance points with 10-bit BT.2020 non-constant luminance colorimetry with SDR, HDR PQ and HDR HLG.

-	Define the relevant ISO BMFF encapsulation, CMAF media profile and DASH signaling for the new 8K TV operation point.

-	Communicate with relevant organizations including MPEG, CTA WAVE, DVB, SCTE and ATSC to develop a CMAF profile for 8K HEVC.

- 	Include the newly defined decoding capabilities and associated profiles and operation points into 5G Media Streaming.

-	Provide a new scenario for TR26.955 to document characterization of 8K TV.

-	Document typical traffic characteristics of such 8K TV video services.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |
| TS 26.116 | Operation Points for 8K Television over 5G (Rel-17) | SA#94 (Dec 2021) |
| TS 26.511 | Decoding Capabilities and Operation Points for 8K Television over 5G (Rel-17) | SA#94 (Dec 2021) |
| TR 26.925 | Typical traffic characteristics for 8K Television over 5G (Rel-17) | SA#95 (Mar 2022) |
| TR 26.955 | 8K TV Scenario: Reference Sequences and Anchors (Rel-17) | SA#95 (Mar 2022) |

## 6	Work item Rapporteur(s)

Thomas Stockhammer, tsto@qti.qualcomm.com

## 7	Work item leadership

SA4

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated

Tencent

Fraunhofer HHI

Ericsson LM

ATEME

Beijing Xiaomi Electronics

Interdigital

AT&T

Nokia Corporation

Dolby Laboratories

Samsung Electronics Co. LTD

Sony Europe B.V.

Huawei Technologies Co., Ltd.

https://8kassociation.com/

https://dashif.org/docs/workshop-2019/04-thierry%20fautier%20-%20Harmonic%20Codec%20Comparison%205G%20Media%20Workshop_Final%20v3.pdf