3GPP TSG SA WG4 #117	S4-220104

14th - 23rd February 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | - | rev | - | Current version: | 1.4.6 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on AV1 Results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2022-02- |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update to AV1 configuration and results |  |  |  |  |  |  |  |  |
| Summary of change: |  | Minor edits; explicate some scenario 3 config settings; partial results for AV1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

## 8.4	AOMedia Video 1 (AV1)

End Change 1

#### 8.4.2.2	Scenario 1: Full HD Streaming

...

Change 2

##### 8.4.2.2.5	AV1 Test Results

AV1 test streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-1-FHD/AV1/

AV1 test metrics are provided with the appropriate keys as defined in Table 8.4.2.2.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-1-FHD/AV1/Metrics/

End Change 2

#### 8.4.2.3	Scenario 2: 4K-TV

...

Change 3

##### 8.4.2.3.5	AV1 Test Results

AV1 test streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-2-4K/AV1/

AV1 test metrics are provided with the appropriate keys as defined in Table 8.4.2.3.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-2-4K/AV1/Metrics/

End Change 3

#### 8.4.2.4	Scenario 3: Screen Content Scenario

...

Change 4

##### 8.4.2.4.2	Common Parameters and Settings

The following common Scenario 3 command line options are provided in Table 8.4.2.4.2-1.

Table 8.4.2.4.2-1 Common Scenario 3 command line options

| --lag-in-frames=0 | No future frame is used as reference in low delay configuration. Key-frame-filtering is a motion compensated temporal filtering. It is only enabled in random access configuration. When --lag-in-frames=0, key frames will not be filtered. |
| --- | --- |
| --deltaq-mode=0 | disable block based adaptive quantization, so all blocks within a frame use the same quantization parameter |
| --enable-onesided-comp=0 | disable compound prediction for low delay content. Unidirectional prediction Inter frame, only. |
| --max-reference-frames=4 | restrict number of reference frames to 4. |
| --enable-obmc=1 | Enable OBMC |
| --enable-dist-wtd-comp=1 | Enable distance weighted inter compound prediction |
| --enable-diff-wtd-comp=1 | Enable difference weighted inter compound prediction |
| --enable-interintra-comp=1 | Enable inter/intra compound prediction |
| --enable-interinter-wedge=1 | Enable wedge-based inter compound prediction |
| --enable-interintra-wedge=1 | Enable wedge-based inter/intra compound prediction |
| --enable-masked-comp=1 | Enable mask based inter compound prediction |
| --enable-smooth-interintra=1 | Enable smooth inter/intra compound prediction |

End Change 4

...

Change 5

##### 8.4.2.4.7	AV1 Test Results

AV1 test streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/AV1/

AV1 test metrics are provided with the appropriate keys as defined in Table 8.4.2.4.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/AV1/Metrics/

Editor’s Note:

- Results are still missing

End Change 5

#### 8.4.2.5	Scenario 4: Messaging and Social Sharing

...

Change 6

##### 8.4.2.5.5	AV1 Test Results

AV1 test streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-4-Sharing/AV1/

AV1 test metrics are provided with the appropriate keys as defined in Table 8.4.2.5.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-4-Sharing/AV1/Metrics/

End Change 6

#### 8.4.2.6	Scenario 5: Online Gaming

...

Change 7

##### 8.4.2.6.7	AV1 Test Results

AV1 test streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-5-Gaming/AV1/

AV1 test metrics are provided with the appropriate keys as defined in Table 8.4.2.6.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-5-Gaming/AV1/Metrics/

End Change 7