3GPP TSG-SA4 Meeting #119-e	S4-220625

Online, 11th – 20th May 2022                                                                    revision of S4-220625

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0021 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for fixing api-version in Open API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 5th May 2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR corrects the version reference in the 5GMS APIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Usage of apiVersion instead of concrete versions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Future updates to the API would require changes throughout the document. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 7.2.2, 7.3.2, 7.4.2, 7.5.2, 7.6.2, 7.7.2, 7.8.2, 7.9.2, 11.3.2, 11.4.2, 11.5.2, 11.6.2, C.3.1, C.3.2, C.3.3, C.3.4, C.3.5, C.3.6, C.3.7, C.3.8, C.4.2, C.4.3, C.4.4, C.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 6.1	HTTP resource URIs and paths

The resource URI used in each HTTP request to the API provider shall have the structure defined in subclause 4.4.1 of TS 29.501 [22], i.e.:

{apiRoot}/{apiName}/{apiVersion}/{apiSpecificResourceUriPart}

with the following components:

-	{apiRoot} shall be set as described in TS 29.501 [22].

-	{apiName} shall be set as defined by the following clauses.

-	{apiVersion} shall be set to "v2" in this release of the specification.

-	{apiSpecificResourceUriPart} shall be set as described in the following clauses.

2nd Change

### 7.2.2	Resource structure

The Provisioning Sessions API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/

3rd Change

### 7.3.2	Resource structure

The Server Certificates Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

3rd Change

### 7.4.2	Resource structure

The Content Preparation Templates Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

4th Change

### 7.5.2	Resource structure

The Content Protocols Discovery API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

5th Change

### 7.6.2	Resource structure

The Content Hosting Provisioning API is accessible through this URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

6th Change

### 7.7.2	Resource structure

The Consumption Reporting Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

7th Change

### 7.8.2	Resource structure

The Metrics Reporting Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

8th Change

### 7.9.2	Resource structure

The Policy Template Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

9th Change

### 11.3.2	Reporting procedure

Consumption reports shall be submitted to one of the URLs selected from the ClientConsumptionReporting‌Configuration.‌serverAddresses array of the ServiceAccessInformation resource (see clause 11.2.3). The path of the URL should conform to the following general format:

{apiRoot}/3gpp-m5/{apiVersion}/consumption-reporting/{aspId}

10th Change

### 11.4.2	Reporting procedure

Metrics reports related to a specific metricsReportingConfigurationId shall be submitted to one of the URLs selected from the ClientMetricsReportingConfiguration.serverAddresses array of the ServiceAccessInformation resource (see clause 11.2.3). The path of the URL should conform to the following general format:

{apiRoot}/3gpp-m5/{apiVersion}/metrics-reporting/{provisioningSessionId}/{metricsReportingConfigurationId}

11th Change

### 11.5.2	Resource structure

The Dynamic Policies API is accessible through the following URL base path:

{apiRoot}/3gpp-m5/{apiVersion}/dynamic-policies/

12th Change

### 11.6.2	Resource structure

The Network Assistance API is accessible via the following URL base path:

{apiRoot}/3gpp-m5/{apiVersion}/network-assistance/

13th Change

## C.3.1	M1_ProvisioningSessions API

openapi: 3.0.0 info: title: M1_ProvisioningSessions version: 1.0.0 description: ¦ 5GMS AF M1 Provisioning Sessions API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ProvisioningSessions description: '5G Media Streaming: Provisioning (M1) APIs: Provisioning Sessions' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions: post: operationId: createProvisioningSession summary: 'Create a new Provisioning Session' responses: '201': description: 'Provisioning Session Created' headers: Location: description: 'URL including the resource identifier of the newly created Provisioning Session.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' /provisioning-sessions/{provisioningSessionId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' get: operationId: getProvisioningSessionById summary: 'Retrieve an existing Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' delete: operationId: destroyProvisioningSession summary: 'Destroy an existing Provisioning Session' responses: '204': description: 'Provisioning Session Destroyed' # No Content components: schemas: ProvisioningSession: type: object description: "A representation of a Provisioning Session." required: - provisioningSessionId - provisioningSessionType - contentProtocols properties: provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' provisioningSessionType: $ref: 'TS26512_CommonData.yaml#/components/schemas/ProvisioningSessionType' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' serverCertificateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 contentPreparationTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 metricsReportingConfigurationIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 policyTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1

14th Change

## C.3.2	M1_ServerCertificatesProvisioning API

openapi: 3.0.0 info: title: M1_ServerCertificatesProvisioning version: 1.0.0 description: ¦ 5GMS AF M1 Server Certificates Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ServerCertificatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Server Certificates Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/certificates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: The resource identifier of an existing Provisioning Session.' post: operationId: createOrReserveServerCertificate summary: 'Create or reserve a Service Certificate resource' description: 'Without the optional csr query parameter, an X.509 certificate is generated and this is returned. If the csr query parameter is present, a Certificate Signing Request is instead generated and returned, allowing the X.509 certificate to be generated by the invoker and later uploaded.' parameters: - in: query name: csr schema: type: string description: 'When present, return a Certificate Signing Request instead of generating an X.509 certificate' responses: '200': description: 'Server Certificate Created' headers: Location: description: 'URL of the newly created Server Certificate resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' content: 'application/x-pem-file': schema: type: string /provisioning-sessions/{provisioningSessionId}/certificates/{certificateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: The resource identifier of an existing Provisioning Session.' - name: certificateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Server Certificate' put: operationId: uploadServerCertificate summary: "Upload the X.509 certificate for a previously reserved Server Certificate resource" requestBody: required: true content: application/x-pem-file: schema: type: string responses: '204': description: 'Server Certificate Updated' get: operationId: retrieveServerCertificate summary: 'Retrieve the X.509 certificate representation of the specified Server Certificate resource' responses: '200': description: 'Success' content: 'application/x-pem-file': schema: type: string '204': description: 'Awaiting Upload' '404': description: 'Not Found' delete: operationId: destroyServerCertificate summary: 'Destroy an existing Server Certificate resource' responses: '204': description: 'Server Certificate Destroyed'

15th Change

## C.3.3	M1_ContentPreparationTemplatesProvisioning API

openapi: 3.0.0 info: title: M1_ContentPreparationTemplatesProvisioning version: 1.0.0 description: ¦ 5GMS AF M1 Content Preparation Templates Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentPreparationTemplatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Preparation Templates Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-preparation-templates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentPreparationTemplate summary: 'Create (and optionally upload) a new Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template of any type' required: true content: '*/*': schema: type: string responses: '201': description: 'Content Preparation Template Created' headers: Location: description: 'URL of the newly created Content Preparation Template.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/content-preparation-templates/{contentPreparationTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: contentPreparationTemplateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Content Preparation Template.' get: operationId: retrieveContentPreparationTemplate summary: 'Retrieve the specified Content Preparation Template of the specified Provisioning Session' responses: '200': description: 'Success' content: '*/*': schema: type: string put: operationId: updateContentPreparationTemplate summary: 'Update the specified Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template of any type' required: true content: '*/*': schema: type: string responses: '204': description: 'Updated Content Preparation Template' '404': description: 'Not Found' patch: operationId: patchContentPreparationTemplate summary: 'Patch the specified Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template patch of any type' required: true content: application/merge-patch+json: schema: type: string application/json-patch+json: schema: type: string responses: '200': description: 'Patched Content Preparation Templates' content: '*/*': schema: type: string '404': description: 'Not Found' delete: operationId: destroyContentPreparationTemplate summary: 'Destroy the specified Content Preparation Template of the specified Provisioning Session' responses: '204': description: 'Destroyed Content Preparation Template' '404': description: 'Not Found'

16th Change

## C.3.4	M1_ContentProtocolsDiscovery API

openapi: 3.0.0 info: title: M1_ContentProtocolsDiscovery version: 1.0.0 description: ¦ 5GMS AF M1 Content Protocols Discovery API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentDiscovery description: '5G Media Streaming: Provisioning (M1) APIs: Content Protocols Discovery' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/protocols: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' get: operationId: retrieveContentProtocols summary: 'Retrieve the set of Content Protocols supported by the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentProtocols' components: schemas: ContentProtocolDescriptor: type: object required: - termIdentifier properties: termIdentifier: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' descriptionLocator: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' ContentProtocols: type: object properties: downlinkIngestProtocols: type: array items: $ref: '#/components/schemas/ContentProtocolDescriptor' minItems: 1 uplinkEgestProtocols: type: array items: $ref: '#/components/schemas/ContentProtocolDescriptor' minItems: 1 geoFencingLocatorTypes: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1

17th Change

## C.3.5	M1_ContentHostingProvisioning API

openapi: 3.0.0 info: title: M1_ContentHostingProvisioning version: 1.0.0 description: ¦ 5GMS AF M1 Content Hosting Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentHostingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Hosting Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentHostingConfiguration summary: 'Create (and optionally upload) the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '201': description: 'Content Hosting Configuration Created' headers: Location: description: 'URL of the newly created Content Hosting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' get: operationId: retrieveContentHostingConfiguration summary: 'Retrieve the Content Hosting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' put: operationId: updateContentHostingConfiguration summary: 'Update the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '204': description: 'Updated Content Hosting Configuration' '404': description: 'Not Found' patch: operationId: patchContentHostingConfiguration summary: 'Patch the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' delete: operationId: destroyContentHostingConfiguration summary: 'Destroy the current Content Hosting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Content Hosting Configuration' '404': description: 'Not Found' /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/purge: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: A unique identifier of the Provisioning post: operationId: purgeContentHostingCache summary: 'Purge the content of the cache for the Content Hosting Configuration of the specified Provisioning Session' requestBody: description: 'The regular expression pattern for resources to purge from the cache' required: true content: application/x-www-form-urlencoded: schema: properties: pattern: description: 'Keyword' type: string value: description: 'The regular expression' type: string responses: '200': description: 'Content Purged' components: schemas: IngestConfiguration: type: object properties: path: type: string pull: type: boolean protocol: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' entryPoint: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' PathRewriteRule: type: object required: - requestPattern - mappedPath properties: requestPattern: type: string mappedPath: type: string CachingConfiguration: type: object properties: urlPatternFilter: type: string CachingDirectives: type: object required: - urlPatternFilter - noCache properties: statusCodeFilters: type: array items: type: integer noCache: type: boolean maxAge: type: integer format: int32 DistributionConfiguration: type: object required: - canonicalDomainName - domainNameAlias properties: contentPreparationTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' canonicalDomainName: type: string domainNameAlias: type: string pathRewriteRules: type: array items: $ref: '#/components/schemas/PathRewriteRule' cachingConfigurations: type: array items: $ref: '#/components/schemas/CachingConfiguration' GeoFencing: type: object required: - locatorType - locators properties: locatorType: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' locators: type: array items: type: string description: 'Format of individual locators depends on the locatorType.' minItems: 1 UrlSignature: type: object required: - urlPattern - tokenName - passphraseName - passphrase - tokenExpiryName - useIPAddress properties: urlPattern: type: string tokenName: type: string passphraseName: type: string passphrase: type: string tokenExpiryName: type: string useIPAddress: type: boolean ipAddressName: type: string certificateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' # Schema for the resource itself ContentHostingConfiguration: type: object required: - name - ingestConfiguration - distributionConfigurations properties: name: type: string ingestConfiguration: $ref: '#/components/schemas/IngestConfiguration' distributionConfigurations: type: array items: $ref: '#/components/schemas/DistributionConfiguration'

18th Change

## C.3.6	M1_ConsumptionReportingProvisioning API

openapi: 3.0.0 info: title: M1_ConsumptionReportingProvisioning version: 1.0.0 description: ¦ 5GMS AF M1 Consumption Reporting Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ConsumptionReportingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Consumption Reporting Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/consumption-reporting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: activateConsumptionReporting summary: 'Activate the consumption reporting procedure for the specified Provisioning Session by providing the Consumption Reporting Configuration' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '201': description: 'Consumption Reporting Configuration Created' headers: Location: description: 'URL of the newly created Consumption Reporting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' get: operationId: retrieveConsumptionReportingConfiguration summary: 'Retrieve the Consumption Reporting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' put: operationId: updateConsumptionReportingConfiguration summary: 'Update the Consumption Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '204': description: 'Updated Consumption Reporting Configuration' '404': description: 'Not Found' patch: operationId: patchConsumptionReportingConfiguration summary: 'Patch the Consumption Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '200': description: 'Patched Consumption Reporting Configuration' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' '404': description: 'Not Found' delete: operationId: destroyConsumptionReportingConfiguration summary: 'Destroy the current Consumption Reporting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Consumption Reporting Configuration' '404': description: 'Not Found' components: schemas: ConsumptionReportingConfiguration: type: object properties: reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' locationReporting: type: boolean

19th Change

## C.3.7	M1_MetricsReportingProvisioning API

openapi: 3.0.0 info: title: M1_MetricsReportingProvisioning version: 1.0.0 description: ¦ 5GMS AF M1 Metrics Reporting Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_MetricsReportingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Metrics Reporting Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/metrics-reporting-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: activateMetricsReporting summary: 'Activate the Metrics reporting procedure for the specified Provisioning Session by providing the Metrics Reporting Configuration' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '201': description: 'Metrics Reporting Configuration Created' headers: Location: description: 'URL of the newly created Metrics Reporting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/metrics-reporting-configurations/{metricsReportingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: metricsReportingConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of a Metrics Reporting Configuration.' get: operationId: retrieveMetricsReportingConfiguration summary: 'Retrieve the specified Metrics Reporting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' put: operationId: updateMetricsReportingConfiguration summary: 'Update the specified Metrics Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '204': description: 'Updated Metrics Reporting Configuration' '404': description: 'Not Found' patch: operationId: patchMetricsReportingConfiguration summary: 'Patch the specified Metrics Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '200': description: 'Patched Metrics Reporting Configuration' content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' '404': description: 'Not Found' delete: operationId: destroyMetricsReportingConfiguration summary: 'Destroy the specified Metrics Reporting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Metrics Reporting Configuration' '404': description: 'Not Found' components: schemas: MetricsReportingConfiguration: type: object required: - metricsReportingConfigurationId - scheme properties: metricsReportingConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' scheme: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string minItems: 1 metrics: type: array items: type: string minItems: 1

20th Change

## C.3.8	M1_PolicyTemplatesProvisioning API

openapi: 3.0.0 info: title: M1_PolicyTemplatesProvisioning version: 1.0.0 description: ¦ 5GMS AF M1 Policy Templates Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_PolicyTemplatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Policy Templates Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/policy-templates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createPolicyTemplate summary: 'Create (and optionally upload) a new Policy Template' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '201': description: 'Policy Template Created' headers: Location: description: 'URL of the newly created Policy Template resource.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/policy-templates/{policyTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'A unique identifier of the Provisioning Session.' - name: policyTemplateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'A resource identifier of a Policy Template.' get: operationId: retrievePolicyTemplate summary: 'Retrieve a representation of an existing Policy Template in the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' '404': description: 'Not Found' put: operationId: updatePolicyTemplate summary: 'Update a Policy Template for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '204': description: 'Updated Policy Template' '404': description: 'Not Found' patch: operationId: patchPolicyTemplate summary: 'Patch the Policy Template for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/PolicyTemplate' application/json-patch+json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' '404': description: 'Not Found' delete: operationId: destroyPolicyTemplate responses: '204': description: 'Destroyed Policy Template' '404': description: 'Not Found' components: schemas: PolicyTemplate: type: object required: - policyTemplateId - state - apiEndPoint - apiType - externalReference - ApplicationSessionContext properties: policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' state: anyOf: - type: string enum: [PENDING, INVALID, READY, SUSPENDED] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. apiEndPoint: type: string apiType: anyOf: - type: string enum: [N5, N33] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. externalReference: type: string qoSSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/M1QoSSpecification' ApplicationSessionContext: type: object properties: afAppId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId' sliceInfo: $ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai' dnn: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' chargingSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/ChargingSpecification'

21st Change

## C.4.2	M5_ConsumptionReporting API

openapi: 3.0.0 info: title: M5_ConsumptionReporting version: 1.0.0 description: ¦ 5GMS AF M5 Consumption Reporting API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ConsumptionReporting description: '5G Media Streaming: Media Session Handling (M5) APIs: Consumption Reporting' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /consumption-reporting/{aspId}: parameters: - name: aspId in: path required: true schema: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' description: 'See 3GPP TS 26.512 clause 11.3.2.' post: operationId: submitConsumptionReport summary: 'Submit a Consumption Report' requestBody: description: 'A Consumption Report' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReport' responses: '204': description: 'Consumption Report Accepted' '400': description: 'Bad Request' '415': description: 'Unsupported Media Type' components: schemas: ConsumptionReport: type: object required: - mediaPlayerEntry - reportingClientId - consumptionReportingUnits properties: mediaPlayerEntry: type: string reportingClientId: type: string consumptionReportingUnits: type: array items: $ref: '#/components/schemas/ConsumptionReportingUnit' ConsumptionReportingUnit: type: object required: - mediaConsumed - startTime - duration properties: mediaConsumed: type: string startTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' duration: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' locations: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/TypedLocation' minItems: 1

22nd Change

## C.4.3	M5_MetricsReporting API

openapi: 3.0.0 info: title: M5_MetricsReporting version: 1.0.0 description: ¦ 5GMS AF M5 Metrics Reporting API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ConsumptionReporting description: '5G Media Streaming: Media Session Handling (M5) APIs: Metrics Reporting' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /metrics-reporting/{provisioningSessionId}/{metricsReportingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: metricsReportingConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of a Metrics Configuration in the specified Provisioning Session.' post: operationId: submitMetricsReport summary: 'Submit a Metrics Report' requestBody: description: 'A Metrics Report' required: true content: application/3gpdash-qoe-report+xml: schema: type: string format: xml application/*: schema: type: string responses: '204': description: 'Metrics Report Accepted' '400': description: 'Bad Request' '415': description: 'Unsupported Media Type'

23rd Change

## C.4.4	M5_DynamicPolicies API

openapi: 3.0.0 info: title: M5_DynamicPolicies version: 1.0.0 description: ¦ 5GMS AF M5 Dynamic Policy API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_DynamicPolicies description: '5G Media Streaming: Media Session Handling (M5) APIs: Dynamic Policies' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /dynamic-policies: post: operationId: createDynamicPolicy summary: 'Create (and optionally upload) a new Dynamic Policy resource' requestBody: description: 'An optional JSON representation of a Dynamic Policy resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '201': description: 'Created Dynamic Policy Resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' headers: Location: description: 'The URL of the newly created Dynamic Policy resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' '400': description: 'Bad Request' '401': description: 'Unauthorized' /dynamic-policies/{dynamicPolicyId}: parameters: - name: dynamicPolicyId description: 'The resource identifier of a Dynamic Policy resource' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveDynamicPolicy summary: 'Retrieve an existing Dynamic Policy resource' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' put: operationId: updateDynamicPolicy summary: 'Update an existing Dynamic Policy resource' requestBody: description: 'A replacement JSON representation of a Dynamic Policy resource' required: true content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' patch: operationId: patchDynamicPolicy summary: 'Patch an existing Dynamic Policy resource' requestBody: description: 'A JSON patch to a Dynamic Policy resource' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' application/json-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '200': description: 'Patched Dynamic Policy' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '204': description: 'Patched Dynamic Policy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' delete: operationId: destroyDynamicPolicy summary: 'Destroy an existing Dynamic Policy resource' responses: '204': description: 'Destroyed Dynamic Policy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' components: schemas: DynamicPolicy: type: object required: - dynamicPolicyId - policyTemplateId - serviceDataFlowDescription - provisioningSessionId properties: dynamicPolicyId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' serviceDataFlowDescriptions: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ServiceDataFlowDescription' provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' qosSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' enforcementMethod: type: string enforcementBitRate: type: integer

24th Change

## C.4.5	M5_NetworkAssistance API

openapi: 3.0.0 info: title: M5_NetworkAssistance version: 1.0.0 description: ¦ 5GMS AF M5 Network Assistance API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_NetworkAssistance description: '5G Media Streaming: Media Session Handling (M5) APIs: Network Assistance' externalDocs: description: 3GPP TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols. url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. paths: /network-assistance/: post: operationId: createNetworkAssistanceSession summary: 'Create a new Network Assistance Session.' responses: '201': description: 'Created Network Assistance Session' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' headers: Location: description: 'The URL of the nely created Network Assistance Session resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' '400': description: 'Bad Request' '401': description: 'Unauthorized' /network-assistance/{naSessionId}: parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveNetworkAssistanceSession summary: 'Retrieve an existing Network Assistance Session resource' responses: '200': description: 'A JSON representation of a Network Assistance Session resource' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' put: operationId: updateNetworkAssistanceSession summary: 'Update an existing Network Assistance Session resource' requestBody: description: 'A replacement JSON representation of a Network Assistance Session resource' required: true content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' patch: operationId: patchNetworkAssistanceSession summary: 'Patch an existing Network Assistance Session resource' requestBody: description: 'A JSON patch to a Network Assistance Session resource' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' application/json-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '200': description: 'Patched Network Assistance Session' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '204': description: 'Patched Network Assistance Session' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' delete: operationId: destroyNetworkAssistanceSession summary: 'Destroy an existing Network Assistance Session resource' responses: '204': description: 'Destroyed Network Assistance Session' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' /network-assistance/{naSessionId}/recommendation: get: operationId: requestBitRateRecommendation summary: 'Obtain a bit rate recommendation for the next recommendation window' parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Success' content: application/json: schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' /network-assistance/{naSessionId}/boostRequest: post: operationId: requestDeliveryBoost summary: 'Request a delivery boost' parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Delivery Boost Request Processed' content: application/json: schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/OperationSuccessResponse' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' components: schemas: NetworkAssistanceSession: type: object required: - naSessionId properties: naSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' serviceDataFlowDescription: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ServiceDataFlowDescription' minItems: 1 policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' requestedQoS: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' recommendedQoS: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' notficationURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url'

End of Changes